3GPP TSG-RAN WG4 Meeting # 108												R4-2314201

Toulouse, France, August 21 – August 25, 2023

Agenda item:			7.33

Source:	Moderator (Qualcomm Incorporated)

Title:	Topic summary for [108][119] US_900MHz

Document for:	Information

# Introduction

This document is a summary of contributions submitted to agenda item 7.33.  Contributions on system parameters, UE requirements, and BS requirements have been submitted.  The LTE band work item for the US 900 MHz has already been completed, but the NR band work item (RP-230723) is scheduled to complete in December 2023 and is the subject of this summary.  The NR band work item objective includes the following dependency

  - To support 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

# Topic #1: System parameters

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2312023 | ZTE Corporation | Discussion on band definition for 900 MHz NR Band Proposal 1: To define the new NR FDD band as in Table 2.1-1. Proposal 2: For band n106, it supports 3MHz channel bandwidth with 15 kHz SCS. Proposal 3: For band n106, to define the transmission bandwidth configuration NRB = 15 for 3 MHz CBW. Proposal 4: For band n106, it is proposed to define the minimum guardband as 142.5 kHz for 3 MHz CBW. Proposal 5: For NR-ARFCN, it is proposed to define as Table 2.4-1. Proposal 6: For band n106, it is proposed to follow the sync raster design of N * 600kHz + M * 50 kHz + 300 kHz, N ϵ {1:2499}, M ϵ {1,3,5} for 3 MHz CBW and wait for the conclusion of sync raster index in NR_FR1_lessthan_5MHz_BW. Proposal 7: For TX–RX frequency separation, it is proposed to define as Table 2.5-1. |
| R4-2313257 | Ericsson | 900 MHz NR band in US - system parameters Proposal: Consider the above band definition, channel bandwidth and channel arrangement. Further discuss if needed the applicability of LTE band 106 channel restriction (EARFCN) for NR. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Band definition, 3 MHz channel bandwidth, 15 kHz SCS, Tx-Rx separation

- These parameters are all agreeable in accordance with the WID

### Sub-topic 1-2 Channel numbering

- ARFCN ranges as shown below

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n106 | 100 | 179200 – <20> – 180200 | 187000 – <20> – 188000 |

Issue 1-2: ARFCN restriction

- Proposals

  - Option 1: Add the note “In the present version of the specification, only NR-ARFCN 179800 and 187600 is applicable for 3 MHz channel bandwidth.”

  - Option 2:  Add the note “In the present version of the specification, only NREF 179800 and 187600 is applicable.”

  - Option 3: No restriction

- Recommended WF

  - Option 1 or Option 2 is needed to be consistent with the FCC rules.

### Sub-topic 1-3 Sync raster

Issue 1-3: Sync raster

- Proposals

  - Option 1: (Ericsson)

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n106 | 15 kHz | Case A | 54679 – <1> – 54693 |
| NOTE xx: In the present version of the specification, only GSCN [54688-54690] are applicable. |  |  |  |

  - Option 2:  (ZTE)

For band n106, it is proposed to follow the sync raster design of N * 600kHz + M * 50 kHz + 300 kHz, N ϵ {1:4998}, M ϵ {1,3,5} for 3 MHz CBW and wait for the conclusion of sync raster index in NR_FR1_lessthan_5MHz_BW.

  - Option 3: Other

- Recommended WF

  - Wait for the conclusion of less than 5 MHz work item.  Generic aspects to be captured in that work item’s CR and only the n106 band specific aspects need to be considered in this work item.

# Topic #2: UE requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2311245 | Apple | A-MPR results for NR US 900 and 3MHz CBW Observation: For the NR waveforms DFT-s-OFDM and CP-OFDM there is no need for additional power back-off in case the emission limit for the frequency range 869 to 894 MHz is set to -30dBm/MHz. Proposal: Set emission limit for the frequency range 869 to 894 MHz to -30dBm/MHz. |
| R4-2312024 | ZTE Corporation | Discussion on UE RF requirements for 900 MHz NR Band Proposal 1: For band n106, to define UE power class as Table 2.1-1. Proposal 2: For band n106, it is proposed to reuse the reference sensitivity requirement for B8 defined in 36.101 and define the reference sensitivity as -99.2 dBm for 3MHz CBW. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: Emissions and A-MPR

- Proposals

  - Option 1: Set emission limit for the frequency range 869 to 894 MHz to -30dBm/MHz.  No A-MPR is allowed.  (Moderator note:  Should the lower limit of the frequency range be extended to 859 MHz to cover Band n26?)

  - Option 2: Other

- Recommended WF

  - Option 1.  Also suggest to note that WF in R4-2310436 is relevant (its title is “WF on LTE and NR …”)

### Sub-topic 2-2

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-2: Power class and Refsens

- Proposals

  - Option 1: PC3, 23 dBm +/- 2 dB, -99.2 dBm refsens

  - Option 2: TBA

- Recommended WF

  - Option 1

# Topic #3: BS requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2312025 | ZTE Corporation | Discussion on BS RF requirements for 900 MHz NR Band Table 2.1. Summary of related RF requirement 6.6.4 Operating band unwanted emissions Once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW, band n106 needs to added. 6.6.5 Transmitter spurious emissions Band n106 needs to added in Table 6.6.5.2.3-1, Table 6.6.5.2.4-1. |

## Open issues summary

Moderator suggests to directly treat the CR’s and draft CR’s.

# Topic #4: CR’s and draft CR’s

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2311448 | Huawei, HiSilicon | draft CR for 38101-1 to inoduce n106 |
| R4-2311449 | Huawei, HiSilicon | draft CR for 38104 to inoduce n106 |
| R4-2311450 | Huawei, HiSilicon | draft CR for 38133 to inoduce n106 |
| R4-2311451 | Huawei, HiSilicon | draft CR for 36101 to inoduce n106 |
| R4-2311452 | Huawei, HiSilicon | draft CR for 36104 to inoduce n106 |
| R4-2311453 | Huawei, HiSilicon | draft CR for 36141 to inoduce n106 |
| R4-2311454 | Huawei, HiSilicon | draft CR for 37104 to inoduce n106 |
| R4-2311455 | Huawei, HiSilicon | draft CR for 37141 to inoduce n106 |
| R4-2311456 | Huawei, HiSilicon | draft CR for 37145-1 to inoduce n106 |
| R4-2311457 | Huawei, HiSilicon | draft CR for 37145-2 to inoduce n106 |
| R4-2311480 | Nokia, Anterix | draftCR introduction of n106 |
| R4-2312026 | ZTE Corporation | CR to TS 36.104: the introduction of band n106 |
| R4-2312027 | ZTE Corporation | CR to TS 36.141: the introduction of band n106 |
| R4-2312028 | ZTE Corporation | CR to TS 37.104: the introduction of band n106 |
| R4-2312029 | ZTE Corporation | CR to TS37.141: the introduction of band n106 |
| R4-2312030 | ZTE Corporation | CR to TS37.145-1: the introduction of band n106 |
| R4-2312031 | ZTE Corporation | CR to TS37.145-2: the introduction of band n106 |
| R4-2312236 | ZTE Corporation | CR to TS 38.133: Introduction of 900 MHz NR Band in the US |
| R4-2313281 | Nokia, Nokia Shanghai Bell | draftCR to 36.104 on introduction of Band n106 |
| R4-2313282 | Nokia, Nokia Shanghai Bell | draftCR to 36.141 on introduction of Band n106 |
| R4-2313283 | Nokia, Nokia Shanghai Bell | draftCR to 37.104 on introduction of Band n106 |
| R4-2313284 | Nokia, Nokia Shanghai Bell | draftCR to 37.141 on introduction of Band n106 |
| R4-2313285 | Nokia, Nokia Shanghai Bell | draftCR to 38.104 on introduction of Band n106 |
| R4-2313286 | Nokia, Nokia Shanghai Bell | draftCR to 38.141-1 on introduction of Band n106 |
| R4-2313287 | Nokia, Nokia Shanghai Bell | draftCR to 38.141-2 on introduction of Band n106 |

## Open issues summary

The following table lists the affected specifications from the WID as well as the CR’s and draft CR’s presented in this meeting.  Also, a suggested company to organize revision and merging of CR’s and to incorporate agreements made this meeting is provided below.

| Specification | Huawei | Nokia | ZTE | Merge |
| --- | --- | --- | --- | --- |
| 38.101-1 | R4-2311448 | R4-2311480 |  | Nokia |
| 38.133 | R4-2311450 |  | R4-2312236 | ZTE |
| 38.104 | R4-2311449 | R4-2313285 |  | Huawei |
| 38.141-1 |  | R4-2313286 |  | Nokia |
| 38.141-2 |  | R4-2313287 |  | Nokia |
| 36.104 | R4-2311452 | R4-2313281 | R4-2312026 | ZTE |
| 36.141 | R4-2311453 | R4-2313282 | R4-2312027 | ZTE |
| 37.104 | R4-2311454 | R4-2313283 | R4-2312028 | Nokia |
| 37.141 | R4-2311455 | R4-2313284 | R4-2312029 | ZTE |
| 37.105 |  |  |  | Ericsson |
| 37.145-1 | R4-2311456 |  | R4-2312030 | Huawei |
| 37.145-2 | R4-2311457 |  | R4-2312031 | Huawei |
| 36.101 (not in WID) | R4-2311451 |  |  | Huawei |