# 3GPP TSG-RAN WG4 Meeting # 108	      R4-2314218

# Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.13.6

Source:	Moderator (CMCC)

Title:	Topic summary for [108][136] NR_ATG_UERF_part1

Document for:	Information

Introduction

RAN#96 meeting approved RP-221369 Revised WID on Air-to-ground network for NR in Rel-18.

This thread focuses on adjacent channel co-existence evaluation for Rel-18 ATG and corresponds to agenda 8.13.1 and 8.13. According to the agreed timeline during last meeting, the target of this meeting is to finish the collection of simulation results for non-synchronization case.

Topic #1: Simulation assumption

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2311263 | Ericsson | Updated co-existence simulation results for synchronized scenarios. In this document, we present our updated co-existence simulation results for synchronized scenarios on the agreed template [2] which will further be used to formulate the TP consisting of results from all the companies [4] |
| R4-2311264 | Ericsson | Co-existence simulation results for non-synchronized scenarios Observation 1 The overlapping coverage case with a boresight angle of zero degrees is the worst case. Observation 2 The non-overlapping coverage cases, with boresight angles of, e.g., thirty and sixty degrees are unrealistic to design for since in reality (both TN and ATG) operators do not have control on which direction the traffic is coming from. Observation 3 Assuming ATG traffic always coming from the same direction (i.e., ATG UEs flying in a straight line) might be too simplistic and real deployments will likely experience larger degradation than observed in the simulations for thirty and sixty degrees. Observation 4 In the simulation of unsynchronized scenarios, where we aim to determine the isolation distance required between an actual TN BS and a ATG BS, wrap around in the TN does not make sense because what we are actually modeling is the absence of a network. Observation 5 For the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, the isolation distance that needs to be maintained between TN and ATG BSs is unreasonable large, and the possibilities of future deployments are extremely slim. Observation 6 For the case where the TN BS is the victim, i.e., scenario 5, the isolation distance needs to be up to 32 km. Observation 7 TN and ATG synchronization implies that the UL and DL slots need to be the same between TN BSs and ATG BSs. Based on the discussion in the previous sections we propose the following: Proposal 1 Wrap around shall not be used in unsynchronized co-existence simulations. Proposal 2 TN and ATG synchronization is necessary to avoid unfeasible isolation distances. |
| R4-2311265 | Ericsson | TP to TR 38.876: Extra results of co-existence synchronized scenarios in Annexure |
| R4-2311266 | Ericsson | TP to TR 38.876: Addition of Co-existence simulation results for Synchronized Scenarios |
| R4-2311267 | Ericsson | Excel template to collect Non-synchronized scenario simulation results |
| R4-2311641 | CATT | From the above simulation results, 20 km isolation distance is needed. We will provide more results in next RAN4 meeting. |
| R4-2311816 | CMCC | Observation 1: for 4GHz ATG DL interfering TN UL case, when 0 degree is assumed, isolation distance is 7km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 2: for 4GHz ATG DL interfering TN UL case, when 30 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 3: for 4GHz ATG DL interfering TN UL case, when 60 degree is assumed, isolation distance is 3km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 4: for 4GHz TN DL interfering ATG UL case, when 0 degree is assumed, isolation distance is 9km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 5: for 4GHz TN DL interfering ATG UL case, when 30 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 6: for 4GHz TN DL interfering ATG UL case, when 60 degree is assumed, isolation distance is 3km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 7: for 2GHz TN DL interfering ATG UL case, when 0 degree is assumed, isolation distance is 12km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. Observation 8: for 2GHz TN DL interfering ATG UL case, when 30 degree is assumed, isolation distance is 0km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. Observation 9: for 2GHz TN DL interfering ATG UL case, when 60 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. |
| R4-2312291 | CMCC | Draft TR for post meeting approval to collect all approved TP in this meeting. |
| R4-2312960 | Huawei, HiSilicon | Observation 1: A, B and C sites are the nearest TN BSs for case 1, case 2 and case 3 respectively. Proposal 1: If observation 1 is not RAN4’s common understanding, RAN4 need to further discuss which BS site is the nearest TN BS for case1, 2 and 3 respectively. Proposal 2: RAN4 should clarify what the worst throughput loss mean and whether it’s the maximum value of LossACI, which is calculated by the following formula. LossACI=1−ThputACIThputSINGLE Observation 2: current test metric for non-synchronized scenario 13 result that it’s very impossible to achieve the reasonable isolation distance. The test metric for scenario 13 and 7 need to be further discussed. Proposal 3: 900m ISD is proposed for 4GHz non-synchronized scenario 5 and 7. |
| R4-2312961 | Huawei, HiSilicon | Simulation result for ATG Synchronized scenarios based on latest template Proposal 1: the ATG ACLR and ACS requirements are proposed in the following table. Table 2 Agreed ATG ACLR and ACS values based on synchronized scenarios ATG; Values. BS; ACLR; 45. ACS; 46. UE; ACLR; 30. ACS; 33. |
| R4-2313168 | ZTE Corporation | Proposal 1: to reuse the FR1 TN ACLR and ACS requirement for ATG BS. Proposal 2: to reuse the FR1 TN PC3 UE ACLR and ACS requirement for ATG CPE. - Observation 1: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 20km. - Observation 2a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 3km with 30 degree angle shift. - Observation 2b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. - Observation 3: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 4a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 9km with 30 degree angle shift. - Observation 4b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift. - Observation 5: for Case 11 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 6a: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 10km with 30 degree angle shift. - Observation 6b: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. |
| R4-2313217 | Qualcomm CDMA Technologies | Proposal 1: For ATG UE ACS, it is sufficient to reuse the legacy FR1 UE ACS of 33dB for ATG UE. Observation 1: For case 5, where TN UL is a victim and ATG DL is aggressor, large isolation distance or higher ATG BS ACLR is required to ensure throughput loss below the 5% loss threshold. Observation 2: For cases 7 and 14, where ATG UL is a victim and TN DL is aggressor, large isolation distance or higher legacy TN BS ACLR is required to ensure throughput loss below the 5% loss threshold. Proposal 2: RAN4 to consider only synchronized operation of TN and ATG deployments to avoid either large isolation distance or high ACIR values. |

Open issues summary

Sub-topic 1-1 the necessity of synchronized ATG network and TN

Issue 1-1: the necessity of synchronized ATG network and TN

- Proposals

  - Option 1: TN and ATG synchronization is necessary to avoid unfeasible isolation distances. TN and ATG synchronization implies that the UL and DL slots need to be the same between TN BSs and ATG BSs.

  - Option 2: TBD

- Recommended WF

  - Wait for conclusion of non-synchronized scenario.

Sub-topic 1-2 wrap around assumption of TN for non-synchronized

Issue 1-2: wrap around in TN for unsynchronized scenarios

- Proposals

  - Option 1: Wrap around shall not be used in unsynchronized co-existence simulations. (Ericsson)

    - In the simulation of unsynchronized scenarios, where we aim to determine the isolation distance required between an actual TN BS and a ATG BS, wrap around in the TN does not make sense because what we are actually modeling is the absence of a network.

  - Option 2: TBD

- Recommended WF

  - Option 1.

Sub-topic 1-3 metric

Agreement: - When TN gNB as victim, only focus on the TN sector with worst throughput loss, 5% and mean among all drops - When ATG gNB as victim, 5% and mean among all drops.

Issue 1-3: clarification of metric for TN gNB as victim

- Proposals

  - Option 1: RAN4 should clarify what the worst throughput loss mean and whether it’s the maximum value of LossACI, which is calculated by the following formula. (Huawei)

LossACI=1−ThputACIThputSINGLE

  - Option 2: TBA

- Recommended WF

  - The TN cell with worst throughput loss is the cell with maximum value of LossACI

    - LossACI=1−ThputACIThputSINGLE

Sub-topic 1-4 nearest TN BS assumption for different deployment case

Issue 1-4: nearest TN BS assumption for different deployment case

- Proposals

  - Option 1: A, B and C sites are the nearest TN BSs for case 1, case 2 and case 3 respectively (Huawei).

For Case 1: (Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero)

Figure 1 network layout for Case 1

For Case 2: (Angle between ATG BS boresight and nearest TN BS boresight in azimuth are 30 degrees.)

Figure 2 network layout for Case 2

For Case 3: (Angle between ATG BS boresight and nearest TN BS boresight in azimuth are 60 degrees.)

Figure 3 network layout for Case 3

- Recommended WF

  - Option 1 is the common understanding.

Sub-topic 1-5 ISD assumption for 4GHz non-synchronized scenario 5 and 7

Issue 1-5: ISD assumption

- Proposals

  - Option 1: 900m ISD is proposed for 4GHz non-synchronized scenario 5 and 7. (Huawei)

    - For scenario 5, if 3500m ISD is assumed for 4GHz, it seems that the throughput loss is smaller than 5% without any isolation distance. However, if the ISD 3500m for 4GHz is decreased, the throughput loss can’t be ignored.

- Recommended WF

  - Further check whether the issue proposed from Huawei is the common issue.

- Topic #2: Simulation results of synchronized scenario

Main technical topic overview. The structure can be done based on sub-agenda basis.

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2311263 | Ericsson | Updated co-existence simulation results for synchronized scenarios. In this document, we present our updated co-existence simulation results for synchronized scenarios on the agreed template [2] which will further be used to formulate the TP consisting of results from all the companies [4] |
| R4-2311264 | Ericsson | Co-existence simulation results for non-synchronized scenarios Observation 1 The overlapping coverage case with a boresight angle of zero degrees is the worst case. Observation 2 The non-overlapping coverage cases, with boresight angles of, e.g., thirty and sixty degrees are unrealistic to design for since in reality (both TN and ATG) operators do not have control on which direction the traffic is coming from. Observation 3 Assuming ATG traffic always coming from the same direction (i.e., ATG UEs flying in a straight line) might be too simplistic and real deployments will likely experience larger degradation than observed in the simulations for thirty and sixty degrees. Observation 4 In the simulation of unsynchronized scenarios, where we aim to determine the isolation distance required between an actual TN BS and a ATG BS, wrap around in the TN does not make sense because what we are actually modeling is the absence of a network. Observation 5 For the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, the isolation distance that needs to be maintained between TN and ATG BSs is unreasonable large, and the possibilities of future deployments are extremely slim. Observation 6 For the case where the TN BS is the victim, i.e., scenario 5, the isolation distance needs to be up to 32 km. Observation 7 TN and ATG synchronization implies that the UL and DL slots need to be the same between TN BSs and ATG BSs. Based on the discussion in the previous sections we propose the following: Proposal 1 Wrap around shall not be used in unsynchronized co-existence simulations. Proposal 2 TN and ATG synchronization is necessary to avoid unfeasible isolation distances. |
| R4-2311265 | Ericsson | TP to TR 38.876: Extra results of co-existence synchronized scenarios in Annexure |
| R4-2311266 | Ericsson | TP to TR 38.876: Addition of Co-existence simulation results for Synchronized Scenarios |
| R4-2311267 | Ericsson | Excel template to collect Non-synchronized scenario simulation results |
| R4-2311641 | CATT | From the above simulation results, 20 km isolation distance is needed. We will provide more results in next RAN4 meeting. |
| R4-2311816 | CMCC | Observation 1: for 4GHz ATG DL interfering TN UL case, when 0 degree is assumed, isolation distance is 7km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 2: for 4GHz ATG DL interfering TN UL case, when 30 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 3: for 4GHz ATG DL interfering TN UL case, when 60 degree is assumed, isolation distance is 3km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 4: for 4GHz TN DL interfering ATG UL case, when 0 degree is assumed, isolation distance is 9km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 5: for 4GHz TN DL interfering ATG UL case, when 30 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 6: for 4GHz TN DL interfering ATG UL case, when 60 degree is assumed, isolation distance is 3km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 7: for 2GHz TN DL interfering ATG UL case, when 0 degree is assumed, isolation distance is 12km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. Observation 8: for 2GHz TN DL interfering ATG UL case, when 30 degree is assumed, isolation distance is 0km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. Observation 9: for 2GHz TN DL interfering ATG UL case, when 60 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. |
| R4-2312291 | CMCC | Draft TR for post meeting approval to collect all approved TP in this meeting. |
| R4-2312960 | Huawei, HiSilicon | Observation 1: A, B and C sites are the nearest TN BSs for case 1, case 2 and case 3 respectively. Proposal 1: If observation 1 is not RAN4’s common understanding, RAN4 need to further discuss which BS site is the nearest TN BS for case1, 2 and 3 respectively. Proposal 2: RAN4 should clarify what the worst throughput loss mean and whether it’s the maximum value of LossACI, which is calculated by the following formula. LossACI=1−ThputACIThputSINGLE Observation 2: current test metric for non-synchronized scenario 13 result that it’s very impossible to achieve the reasonable isolation distance. The test metric for scenario 13 and 7 need to be further discussed. Proposal 3: 900m ISD is proposed for 4GHz non-synchronized scenario 5 and 7. |
| R4-2312961 | Huawei, HiSilicon | Simulation result for ATG Synchronized scenarios based on latest template Proposal 1: the ATG ACLR and ACS requirements are proposed in the following table. Table 2 Agreed ATG ACLR and ACS values based on synchronized scenarios ATG; Values. BS; ACLR; 45. ACS; 46. UE; ACLR; 30. ACS; 33. |
| R4-2313168 | ZTE Corporation | Proposal 1: to reuse the FR1 TN ACLR and ACS requirement for ATG BS. Proposal 2: to reuse the FR1 TN PC3 UE ACLR and ACS requirement for ATG CPE. - Observation 1: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 20km. - Observation 2a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 3km with 30 degree angle shift. - Observation 2b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. - Observation 3: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 4a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 9km with 30 degree angle shift. - Observation 4b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift. - Observation 5: for Case 11 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 6a: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 10km with 30 degree angle shift. - Observation 6b: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. |
| R4-2313217 | Qualcomm CDMA Technologies | Proposal 1: For ATG UE ACS, it is sufficient to reuse the legacy FR1 UE ACS of 33dB for ATG UE. Observation 1: For case 5, where TN UL is a victim and ATG DL is aggressor, large isolation distance or higher ATG BS ACLR is required to ensure throughput loss below the 5% loss threshold. Observation 2: For cases 7 and 14, where ATG UL is a victim and TN DL is aggressor, large isolation distance or higher legacy TN BS ACLR is required to ensure throughput loss below the 5% loss threshold. Proposal 2: RAN4 to consider only synchronized operation of TN and ATG deployments to avoid either large isolation distance or high ACIR values. |

  - Open issues summary

Following is approved simulation scenarios.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 4GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 4GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 4GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 4GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 4GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 4GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 4GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 4GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Sub-topic 2-1 Detailed simulation results which will be captured into TR

Ericsson, Qualcomm, ZTE, Huawei, CATT, CMCC companies (6) have proposed simulation results during offline discussion and in this meeting.

Issue 1-1-1: ACLR/ACS value based on simulation results

- Proposals

  - Option 1:

Table 2 Agreed ATG ACLR and ACS values based on synchronized scenarios

| ATG |  | Values |
| --- | --- | --- |
| ATG BS | ACLR | 45 |
|  | ACS | 46 |
| ATG CPE | ACLR | 30 |
|  | ACS | 33 |

- Recommended WF

  - Option 1

- Topic #3: Simulation results of non-synchronized scenario

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2311263 | Ericsson | Updated co-existence simulation results for synchronized scenarios. In this document, we present our updated co-existence simulation results for synchronized scenarios on the agreed template [2] which will further be used to formulate the TP consisting of results from all the companies [4] |
| R4-2311264 | Ericsson | Co-existence simulation results for non-synchronized scenarios Observation 1 The overlapping coverage case with a boresight angle of zero degrees is the worst case. Observation 2 The non-overlapping coverage cases, with boresight angles of, e.g., thirty and sixty degrees are unrealistic to design for since in reality (both TN and ATG) operators do not have control on which direction the traffic is coming from. Observation 3 Assuming ATG traffic always coming from the same direction (i.e., ATG UEs flying in a straight line) might be too simplistic and real deployments will likely experience larger degradation than observed in the simulations for thirty and sixty degrees. Observation 4 In the simulation of unsynchronized scenarios, where we aim to determine the isolation distance required between an actual TN BS and a ATG BS, wrap around in the TN does not make sense because what we are actually modeling is the absence of a network. Observation 5 For the cases where the ATG BS is the victim, i.e., scenarios 7 and 14, the isolation distance that needs to be maintained between TN and ATG BSs is unreasonable large, and the possibilities of future deployments are extremely slim. Observation 6 For the case where the TN BS is the victim, i.e., scenario 5, the isolation distance needs to be up to 32 km. Observation 7 TN and ATG synchronization implies that the UL and DL slots need to be the same between TN BSs and ATG BSs. Based on the discussion in the previous sections we propose the following: Proposal 1 Wrap around shall not be used in unsynchronized co-existence simulations. Proposal 2 TN and ATG synchronization is necessary to avoid unfeasible isolation distances. |
| R4-2311265 | Ericsson | TP to TR 38.876: Extra results of co-existence synchronized scenarios in Annexure |
| R4-2311266 | Ericsson | TP to TR 38.876: Addition of Co-existence simulation results for Synchronized Scenarios |
| R4-2311267 | Ericsson | Excel template to collect Non-synchronized scenario simulation results |
| R4-2311641 | CATT | From the above simulation results, 20 km isolation distance is needed. We will provide more results in next RAN4 meeting. |
| R4-2311816 | CMCC | Observation 1: for 4GHz ATG DL interfering TN UL case, when 0 degree is assumed, isolation distance is 7km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 2: for 4GHz ATG DL interfering TN UL case, when 30 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 3: for 4GHz ATG DL interfering TN UL case, when 60 degree is assumed, isolation distance is 3km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 1. Observation 4: for 4GHz TN DL interfering ATG UL case, when 0 degree is assumed, isolation distance is 9km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 5: for 4GHz TN DL interfering ATG UL case, when 30 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 6: for 4GHz TN DL interfering ATG UL case, when 60 degree is assumed, isolation distance is 3km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 2. Observation 7: for 2GHz TN DL interfering ATG UL case, when 0 degree is assumed, isolation distance is 12km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. Observation 8: for 2GHz TN DL interfering ATG UL case, when 30 degree is assumed, isolation distance is 0km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. Observation 9: for 2GHz TN DL interfering ATG UL case, when 60 degree is assumed, isolation distance is 5km among 5% of users within the cell with largest throughput loss. Detailed simulation results are in table 3. |
| R4-2312291 | CMCC | Draft TR for post meeting approval to collect all approved TP in this meeting. |
| R4-2312960 | Huawei, HiSilicon | Observation 1: A, B and C sites are the nearest TN BSs for case 1, case 2 and case 3 respectively. Proposal 1: If observation 1 is not RAN4’s common understanding, RAN4 need to further discuss which BS site is the nearest TN BS for case1, 2 and 3 respectively. Proposal 2: RAN4 should clarify what the worst throughput loss mean and whether it’s the maximum value of LossACI, which is calculated by the following formula. LossACI=1−ThputACIThputSINGLE Observation 2: current test metric for non-synchronized scenario 13 result that it’s very impossible to achieve the reasonable isolation distance. The test metric for scenario 13 and 7 need to be further discussed. Proposal 3: 900m ISD is proposed for 4GHz non-synchronized scenario 5 and 7. |
| R4-2312961 | Huawei, HiSilicon | Simulation result for ATG Synchronized scenarios based on latest template Proposal 1: the ATG ACLR and ACS requirements are proposed in the following table. Table 2 Agreed ATG ACLR and ACS values based on synchronized scenarios ATG; Values. BS; ACLR; 45. ACS; 46. UE; ACLR; 30. ACS; 33. |
| R4-2313168 | ZTE Corporation | Proposal 1: to reuse the FR1 TN ACLR and ACS requirement for ATG BS. Proposal 2: to reuse the FR1 TN PC3 UE ACLR and ACS requirement for ATG CPE. - Observation 1: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 20km. - Observation 2a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 3km with 30 degree angle shift. - Observation 2b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. - Observation 3: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 4a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 9km with 30 degree angle shift. - Observation 4b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift. - Observation 5: for Case 11 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km. - Observation 6a: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 10km with 30 degree angle shift. - Observation 6b: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km with 60 degree angle shift. |
| R4-2313217 | Qualcomm CDMA Technologies | Proposal 1: For ATG UE ACS, it is sufficient to reuse the legacy FR1 UE ACS of 33dB for ATG UE. Observation 1: For case 5, where TN UL is a victim and ATG DL is aggressor, large isolation distance or higher ATG BS ACLR is required to ensure throughput loss below the 5% loss threshold. Observation 2: For cases 7 and 14, where ATG UL is a victim and TN DL is aggressor, large isolation distance or higher legacy TN BS ACLR is required to ensure throughput loss below the 5% loss threshold. Proposal 2: RAN4 to consider only synchronized operation of TN and ATG deployments to avoid either large isolation distance or high ACIR values. |

  - Open issues summary

Following is approved simulation scenarios.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 4GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 4GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 4GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 4GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 4GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 4GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 4GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 4GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Sub-topic 3-1 Detailed simulation results which will be captured into TR

For non-synchronized scenario, gNB-to-gNB interference is the dominant interference type. Non-synchronization scenario will only focus on scenario 5, 7, 14.

Moderator note: it seems all the simulation results for non-synchronized scenario is note aligned. Maybe we can conclude final isolation distance in next meeting.

Simulation scenario 5: ATG DL -> TN UL (4GHz)

- Summary of simulation results:

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CMCC 7km, ZTE 17km, Ericsson 32km)

  - whereas other companies show larger than 100km is required(Qualcomm 120km) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (Ericsson 3-11km, CMCC 2-5km, ZTE 3-17km)

- Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2311264.

In scenario 5 (below), the isolation distance needed to maintain the throughput degradation of the TN (most impacted cell) at 5% is more than 32 km for the zero-degree case, more than 11 km for the thirty-degree case, and more than 3 km for the sixty-degree case.

- Observation: Simulation result from CMCC R4-2311816.

Table 1: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 7 | 5 | 3 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 4 | 0 | 2 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null steering and lead to less interference in our simulation.

- Observation: Simulation result from ZTE R4-2313168

- Observation 1: for Case 5 with overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 20km.

- Observation 2a: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 3km with 30 degree angle shift.

- Observation 2b: for Case 5 with non-overlapping coverage, the performance degradation of TN cell of largest throughtput loss of victim network is less than 5% with isolation distance as 17km with 60 degree angle shift.

- Observation: Simulation result from Qualcomm R4-2313217

Observation 1: For case 5, where TN UL is a victim and ATG DL is aggressor, large isolation distance (around 120km) or higher ATG BS ACLR is required to ensure throughput loss below the 5% loss threshold.

Simulation scenario 7: TN rural DL -> ATG UL (4GHz)

- Summary of simulation results:

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CATT 11km, CMCC 9km, ZTE 20km)

  - whereas other companies show 100-300km is required(Qualcomm larger than 200km, Ericsson 150km) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (CATT 11km, CMCC 5km, ZTE 9km, Ericsson 16km)

- Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2311264.

In scenario 7 (below), the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is around 150 km for the zero-degree case, around 16 km for the thirty-degree case, and around 6 km for the sixty-degree case.

- Observation: Simulation result from CATT R4-2311641.

11 km isolation distance is needed

- Observation: Simulation result from CMCC R4-2311816.

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | 9 | 5 | 3 |
|  |  | Average of all users in the whole network | 5 | 0 | 2 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null steering and lead to less interference in our simulation.

- Observation: Simulation result from ZTE R4-2313168

- Observation 3: for Case 7 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km.

- Observation 4a: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 9km with 30 degree angle shift.

- Observation 4b: for Case 7 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 6km with 60 degree angle shift.

- Observation: Simulation result from Qualcomm R4-2313217

Observation 2: For cases 7 and 14, where ATG UL is a victim and TN DL is aggressor, large isolation distance (larger than 200km) or higher legacy TN BS ACLR is required to ensure throughput loss below the 5% loss threshold.

Simulation scenario 14: TN rural DL -> ATG UL (2GHz)

- Summary of simulation results:

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CATT 20km, CMCC 12km, ZTE 20km)

  - whereas other companies show 100-300km or even larger is required(Qualcomm larger than 200km, Ericsson 220km, Huawei larger distance) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (CMCC 3-5km, ZTE 10-17km, Ericsson 6-26km)

- Following list companies’ simulation results.

- Observation: Simulation result from Ericsson R4-2311264.

In scenario 14 (below), the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is around 220 km for the zero-degree case, around 26 km for the thirty-degree case, and around 6 km for the sixty-degree case.

- Observation: Simulation result from CATT R4-2311641.

20 km isolation distance is needed.

- Observation: Simulation result from CMCC R4-2311816.

Table 3: Simulation results for 2GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | 12 | 0 | 5 |
|  |  | Average of all users in the whole network | 7 | 0 | 3 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null steering and lead to less interference in our simulation.

- Observation: Simulation result from Huawei R4-2312960.

Table 1 the performance loss for different isolation distance assuming boresight angle is zero

| Isolation distance | 10km | 100km | 1000km |
| --- | --- | --- | --- |
| SINR at 50% CDF point | -8dB | 7dB | 14.5dB |
| SINR at 5% CDF point | -25dB | -2.5dB | 11dB |
| Throughput loss at 5% CDF point | 100% (40.22Mbps loss) | 87.7% (32.295Mbps loss) | 24.8% (9.99Mbps loss) |

- Observation : Simulation result from ZTE R4-2313168

- Observation 5: for Case 11 with overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 20km.

- Observation 6a: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 10km with 30 degree angle shift.

- Observation 6b: for Case 11 with non-overlapping coverage, the performance degradation of ATG BS as victim network is less than 5% with isolation distance as 17km with 60 degree angle shift.

- Observation: Simulation result from Qualcomm R4-2313217

Observation 2: For cases 7 and 14, where ATG UL is a victim and TN DL is aggressor, large isolation distance or higher legacy TN BS ACLR is required to ensure throughput loss below the 5% loss threshold.

- Topic #4: TR or TP

In this meeting, following TR and TPs are proposed.

| R4-2311265 | TP to TR 38.876: Extra results of co-existence synchronized scenarios in Annexure | Ericsson |
| --- | --- | --- |
| R4-2311266 | TP to TR 38.876: Addition of Co-existence simulation results for Synchronized Scenarios | Ericsson |
| R4-2312291 | Draft TR for ATG For post meeting approval to capture all approved TP in this meeting. | CMCC |