3GPP TSG-RAN WG4 Meeting # 108														R4-2314252

Toulouse, France, August 21 – August 25, 2023

Agenda item:			6.8.1

Source:	Moderator (Huawei)

Title:	Topic summary for [108][316] IoT_NTN_SANRF

Document for:	Information

# Introduction

This summary covers contributions submitted to agenda item 6.8.1.

# Topic #1: Title

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2312641 | China Telecomunication Corp | Proposal 1: It’s suggested to use BWNecessary instead of BWChannel in TS36.108 for Unwanted emission requirement of IoT NTN . FFS on whether corresponding update should be aligned between NR NTN and IoT NTN. Proposal 2: The OBUE requirement should be extended to BWNecessary beyond DL operating band edge. Proposal 3: The PSD terminology in OBUE requirement should be updated to align with ITU recommendation. |

## Open issues summary

### Sub-topic 1-1: OOB requirement

- Proposals (not mutually exclusive)

  - Proposal 1: It’s suggested to use BWNecessary instead of BWChannel in TS36.108 for Unwanted emission requirement of IoT NTN. FFS on whether corresponding update should be aligned between NR NTN and IoT NTN (R4-2312641, China Telecomunication Corp)

  - Proposal 2: The OBUE requirement should be extended to BWNecessary beyond DL operating band edge (R4-2312641, China Telecomunication Corp)

  - Proposal 3: The PSD terminology in OBUE requirement should be updated to align with ITU recommendation (R4-2312641, China Telecomunication Corp)

  - Proposal 4: RAN4 agreed to adopt out-of-band emissions instead of Operating band unwanted emissions for NR NTN solutions. Same approach shall be taken for NB-IoT/eMTC core & performance requirements for NTN, i.e. Introduce OOB emissions and remove OBUE; Introduce BWSAN and remove ΔfOBUE (R4-2311704/05; NEC)

- Recommended WF: collect feedback during the 1st round.

# Topic #2: CRs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2312639 | China Telecomunication Corp | CR on Unwanted emission requirement for IoT NTN 36.108, Rel-18 Moderator: related to sub-topic 1-1. |
| R4-2312640 | China Telecomunication Corp | CR on Unwanted emission requirement for IoT NTN, 36.181, Rel-18 Moderator: related to sub-topic 1-1. Moderator: Wrong WI code (LTE_NBIoT_eMTC_NTN_req-Core), shall be “Perf” |
| R4-2311704 | NEC | CR to 36.108: Out-of-band emissions requirements Moderator: related to sub-topic 1-1. |
| R4-2311705 | NEC | CR to 36.181: Out-of-band emissions requirements Moderator: related to sub-topic 1-1. |
| R4-2311706 | NEC | CR to 36.108: Characteristics of the interfering signals |
| R4-2311707 | NEC | CR to 36.181: Characteristics of the interfering signals |
| R4-2311708 | NEC | CR to 36.181: Test model correction for total power dynamic range requirements |

## Open issues summary