3GPP TSG-RAN WG4 Meeting #108	R4-2314273

Toulouse, France, 21st ‒ 25th August, 2023

Title: 	WF on RRM requirements for NR support for dedicated spectrum less than 5MHz for FR1

Agenda Item:	8.14.5

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# Topic #1: Remaining Core Requirements

## Sub-topic 1-1 Measurements

Agreement:

Issue 1-1: Define CSI-RS-based L1 measurement requirements?

-	Do not define CSI-RS-based L1 measurements, RLM, BFD, CBD requirements in Rel-18

Agreement::

Issue 1-4: SSB based L1-RSRP measurement period?

-	Apply the existing SSB based L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB for CBW less then 5MHz.

## Sub-topic 1-2 Radio Link Monitoring

Agreement:

Issue 1-1: Define CSI-RS-based L1 measurement requirements?

-	Do not define CSI-RS-based L1 measurements, RLM, BFD, CBD requirements in Rel-18.

Based on this agreement, Issues 1-6, 1-7, 1-11 and 1-12 are closed. Additionally, Issues 1-20, 1-21, 1-25 and 1-26 are closed.

Agreement:

The agreements made for RLM OOS can be applied also for BFD procedure.

Agreement:

PDCCH transmission parameters for SSB-based RLM and BFD:

- Issue 1-5: The BW in hypothetical PDCCH transmission parameters for SSB-based RLM

  - Define SSB-based RLM requirements for 12 PRBs [and 15 PRBs] hypothetical PDCCH transmission bandwidth.

    - FFS on applicability rules of RLM requirements for different bands.

Agreement:

PDCCH transmission parameters for RLM and BFD requirement:

  - Aggregation level (CCE) for RLM OOS and BFD

    - AL = 4 for 12 PRBs hypothetical PDCCH transmission BW

    - AL = 8 for 15 PRBs hypothetical PDCCH transmission BW

      - Note: AL 8 will be considered if it is supported by RAN1 design

  - Aggregation level (CCE) for RLM IS

    - AL = [2] for 12 PRBs hypothetical PDCCH transmission BW

    - AL = [4] for 15 PRBs hypothetical PDCCH transmission BW

  - Number of control OFDM symbols

    - 2 OFDM symbols for 12 PRBs hypothetical PDCCH transmission BW

    - 3 OFDM symbols for 15 PRBs hypothetical PDCCH transmission BW

Agreement:

SSB based RLM evaluation period in 3MHz – OOS and IS:

  - SSB-based OOS

    - Reuse existing SSB based RLM evaluation period TEvaluate_out_SSB

  - SSB-based IS

    - Reuse existing SSB based RLM evaluation period TEvaluate_in_SSB

Agreement:

Based on the agreement: The agreements made for RLM OOS can be applied also for BFD procedure.

SSB based BFD evaluation period in 3MHZ:

  - Reuse existing SSB based BFD evaluation period TEvaluate_BFD_SSB

SSB based CBD evaluation period in 3MHZ:

  - Reuse existing SSB based CBD evaluation period TEvaluate_CBD_SSB

Agreement:

Issue 1-13: Use one table for 3MHz out-of-sync transmission parameters?

| Attribute | Value for BLER Configuration #0 |  |
| --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) |
| DCI format | 1-0 |  |
| Number of control OFDM symbols | [2] | [3] |
| Aggregation level (CCE) | [4] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |
| Bandwidth (PRBs) | 12 | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Normal |  |
| Mapping from REG to CCE | Distributed |  |

Issue 1-17: Use one table for 3MHz in-sync transmission parameters?

| Attribute | Value for BLER Configuration #0 |  |
| --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) |
| DCI format | 1-0 |  |
| Number of control OFDM symbols | [2] | [3] |
| Aggregation level (CCE) | [2] | [4] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | [0]dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | [0]dB |  |
| Bandwidth (PRBs) | 12 | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Normal |  |
| Mapping from REG to CCE | Distributed |  |

Note: it is FFS if Aggregation level (CCE) 2 can be used.

Issue 1-27: Use one table for 3MHz CBD transmission parameters?

| Attribute | Value for BLER Configuration #0 |  |
| --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) |
| DCI format | 1-0 |  |
| Number of control OFDM symbols | [2] | [3] |
| Aggregation level (CCE) | [4] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |
| Bandwidth (PRBs) | 12 | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Normal |  |
| Mapping from REG to CCE | Distributed |  |

## Sub-topic 1-3 MIB/PBCH Reading

Issue 1-31: MIB/PBCH decoding requirements are derived based on the assumption of single shot measurement?

Agreement:

MIB/PBCH decoding requirements are derived based on the assumption of single shot measurement.

## Sub-topic 1-4 Indication of PBCH BW

Issue 1-32: Add information on whether the PBCH is 12 or 20 PRBs in the measurement object?

No consensus in RAN4#108 meeting. Recommend continuing discussion.

Issue 1-33: BW for PBCH (e.g., 12 PRBs) of target cell shall be provided to UE in HO command?

No consensus in RAN4#108 meeting. Recommend continuing discussion.

## Sub-topic 1-5 Side Conditions

Issue 1-34: Use following side condition for NR target cell detection for RRC connection re-establishment and RRC connection release with re-direction?

- FFS

  - Intra-frequency NR target cell

    - Option 1: target cell Es/Iot≥-6 dB

    - Option 2: target cell Es/Iot≥-4 dB

  - Inter-frequency NR target cell

    - Option 1: target cell Es/Iot≥-4 dB

    - Option 2: Other

Issue 1-35: Tsearch in HO requirements for 12PRB SSB shall be revised to?

- FFS

  - Unknown intra-frequency target cell:

    - Option 1: target cell Es/Iot≥-2 dB

    - Option 2: Other

  - Unknown inter-frequency target cell:

    - Option 1: target cell Es/Iot≥-2 dB

    - Option 2: Other