3GPP TSG-RAN WG4 Meeting # 108      	R4-2314307

Toulouse, France, August 21 – 25, 2023

Title: 	WF on NR ATG RRM requirements

Agenda Item:	8.13.6

Source: 	CMCC

Document for:	Approval

- Mobility

  - Mobility in RRC_IDLE/INACTIVE

Issue 2-1-1: Cell re-selection mechanism

Agreement:

- Do not introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN

Issue 2-1-3: BS indication and UE capability for Cell re-selection requirement

- Proposal 1: The network indication of set1 or set2 requirements should be a new one that could be broadcasted in a different SIB (e.g., ATG specific SIB). The IE highSpeedMeasInterFreq-r17 in SIB4 should not be used for this purpose.

- Proposal 2: UE supporting set2 requirements for ATG is an optional feature without capability signalling.

Agreement:

The network indication of set1 or set2 requirements is up to RAN2.

UE supporting set2 requirements for ATG is an optional feature without capability signalling.

  - Mobility in RRC_CONNECTED

Issue 2-2-1: location-based CHO mechanism

Agreement:

- Introduce location-based CHO for ATG, reusing the procedure in R17 NTN for R18 ATG

Issue 2-2-2: Definition for distance threshold

- Option 1:

  - Revise the definition of distanceThreshFromReference1 to the distance from the serving cell reference location in two-dimensional direction. Besides, introduce new concept of distanceThreshFromReference1_ATG which present the distance threshold between UE and serving cell reference location . The value of distanceThreshFromReference1_ATG could be calculated by UE through it’s real-time height and distanceThreshFromReference1, distanceThreshFromReference1_ATG = distanceThreshFromReference1(signalled by network) 2+ H2,  H is the altitude of UE.

  - Revise the definition of distanceThreshFromReference2 to the distance from the neighbour cell reference location in two-dimensional direction. Besides, introduce new concepts of distanceThreshFromReference2_ATG which present the distance threshold between UE and neighbour cell reference location . The value of distanceThreshFromReference2_ATG could be calculated by UE through it’s real-time height and distanceThreshFromReference2. distanceThreshFromReference2_ATG = distanceThreshFromReference2(signalled by network) 2+ H2, H is the altitude of UE.

Agreement:

Reuse the existing definition of distanceThreshFromReference1 of NTN

- Timing

Issue 3-1: Special TDD patterns/scheduling restrictions method for GP issue

Online Agreement:

- Recommend RAN2 to introduce an optional TA reporting for ATG UE based on NTN TA reporting mechanism

  - The signalling granularity is for ATG TA reporting is FFS and shall provide approximately symbol-level granularity

  - Note: No new network behavior will be defined

Agreement on signaling granularity:

- Threshold for new TA reporting:

  - Range: 1 symbol ~28 symbol

  - Granularity: 1 symbol

- TA reporting: Symbol level granularity

- Measurement

  - Phase antenna array impact

Issue 4-1-1: Phase antenna array impact for L3 measurements

Agreement:

- Recommend RAN2 to introduce network assistance on neighbor ATG cells reference locations similar to NTN assistance framework

- Do not introduce scaling factor N and the existing measurement requirements can be reused for ATG UEs [with omnidirectional antennas]

- Introduce scaling factor N for L3 measurements for ATG UEs with [antenna arrays]

  - N = 3 for the case when network assistance on ATG cells reference locations is provided

  - N = 4 otherwise

- FFS if any additional UE capability is needed for ATG UEs with [antenna arrays]

- Note: the terms for UE types need to be further aligned with RF session

Issue 4-1-2: Phase antenna array impact for L1 measurements

- Option 1: Do not introduce scaling factor N. Existing measurement requirements can be reused, provided that beam steering based on the positioning is capable for the ATG UE.

- Option 2: Introduce scaling factor.

  - Option 2-1:

    - For FR1 ATG UE with phase array antenna, scaling factor ’P’ for L1 serving cell measurement, L3 neighbouring cell intra-frequency measurement without MG and inter-frequency measurement without MG should be introduced. The impacted requirements including:

      - Radio link monitoring

      - Link recovery procedures

      - L1-RSRP/SINR measurements for Reporting

      - intra-frequency measurements without measurement gaps for ATG

      - Inter frequency measurements without measurement gaps for ATG

      - CSI-RS based intra-frequency measurements

Agreement: Option 1

Issue 4-1-3: Sharing factor between L1 and L3 measurements if introduced

- Option 1: P value for L1 serving cell measurement is defined as

  - Psharing factor * Ntotal / Noutside_MG with Navailable = 0

  - Ntotal / Navailable with Navailable > 0

  - For a window W of duration max(TL1,  MGRPmax), where MGRPmax is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any  resource occasion:

  - Ntotal is the total number of L1-measurement-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

  - Noutside_MG is the number of L1-measurement-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

  - Navailable is

    - the number of L1-measurement-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, and

    - same as Noutside_MG, otherwise

  - TL1 is periodicity of the target L1-measurement-RS

  - Psharing factor = [3].

- Option 2: P value for L3 intra-frequency measurement without gap and inter-frequency measurement without gap is defined as:

  - P=1,

    - if only serving cell are measured (for intra-frequency measurement)

    - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by SMTC occasions, or

    - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by  SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, and RSSI symbols are indicated by SS-RSSI-Measurement;

  - P = 1.5 (Psharing factor/Psharing factor-1), otherwise.

Agreement: Option 1+Option 2

Issue 4-1-4: Phase antenna array impact for scheduling restriction

- Option 1:

  - when the ATG UE with phase array antenna performs intra-frequency neighbouring cell measurements in TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement:

    - If deriveSSB-IndexFromCell is enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

    - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

  - When the ATG UE with phase array antenna performs intra-frequency neighbouring cell measurements in TDD band, the following restrictions apply due to SS-RSRQ measurement:

    - If deriveSSB-IndexFromCell is enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signaling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

    - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

  - When ATG UE with phase array antenna performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

    - UE is not expected to receive PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

  - When ATG UE with phase array antenna performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRQ measurement

    - UE is not expected to receive PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

- Agreement: Option 1.

  - Measurement requirement

Issue 4-2-1: Applicability rule

- Option 1: The measurement requirements for ATG neighbor cells should apply provided that SSB of neighbor cells are within SMTC window in Rel-18.

- Agreement: Option 1.