3GPP TSG RAN WG4 Meeting #108		R4-2314308

Toulouse, France, August 21 – 25, 2023

Title:	LS on NR ATG network assistance and TA reporting

Release:	Rel-18

Study Item:	NR_ATG_Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Shiyuan Wang

E-mail Address:	wangshiyuan@chinamobile.com

1. Overall Description:

In RAN4#108 meeting, RRM requirements for ATG are discussed. In order to reduce the L3 measurement delay, RAN4 agreed to introduce network assistance on neighbor ATG cells reference locations similar to NTN assistance framework.

In addition, RAN4 agreed to introduce an optional TA reporting for ATG UE based on NTN TA reporting mechanism in order to assist network reducing GP overhead for TDD system. The coverage range of an ATG cell is up to 300 kilometers [1]. Different from NTN TA reporting, ATG TA reporting requires finer granularity, which are:

- Threshold for new TA reporting:

  - Range: 1 symbol ~[28] symbol

  - Granularity: 1 symbol

- TA reporting: Symbol level granularity, for a coverage range up to 300 km.

2. Actions:

To RAN2

ACTION: 	RAN4 would like to respectfully ask RAN2 to introduce the optional network assistance information on BS reference locations of ATG neighbour cells, and optional TA reporting for ATG UE.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #108-bis 	9 Oct - 13 Oct 2023	               Xiamen, CN

TSG-RAN WG4 Meeting #109 	13 Nov - 17 Nov 2023	       Chicago, US

[1]   RP-230279, Revised WID on Air-to-ground network for NR, CMCC