3GPP TSG-RAN WG4 Meeting #108	R4-2314314

# Toulouse, France, August 21 – 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | XXXX | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 L1 measurement impact for concurrent MG enhancements (Case 1 and Case 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2 |  |  |  |  |  | Date: |  |  | 25/08/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | L1 measurement impact is missing for MG enhancements related to Case 1 requirements. In this case NW has configured - the combination of Pre-MGs or - the combination of Pre-MG(s) and concurrent MG(s). L1 measurement impact is missing for MG enhancements related to Case 2 requirements. In this case NW has configured - the combination of NCSGs or - the combination of NCSG(s) and concurrent MG(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | L1 measurement impact is added for MG enhancements related to Case 1 and Case 2 requirements for RLM, BFD, CBD and L1 RSRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing L1 measurement impact for Case 1 and Case 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.2, 8.1.3.2, 8.1A.2.2, 8.5.2.2, 8.5.3.2, 8.5.5.2, 8.5.6.2, 8.5A.2.2, 8.5A.5.2, 9.5.4.1, 9.5.4.2, 9.5A.4, 9.8.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2313770. |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

### 8.1.2	Requirements for SSB based radio link monitoring

<unchanged sections omitted>

#### 8.1.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-1 for FR1.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with scaling factor N=8 for FR2-1 and N=12 for FR2-2, for FR2 power classes other than power class 6 or for FR2 power class 6 when highSpeedMeasFlagFR2-r17 is not configured

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-3 for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-4 for FR1 (deactivated PSCell).

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-5 for FR2 (deactivated PSCell) with scaling factor N=8 for FR2-1 and N=12 for FR2-2.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target RLM-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where an RLM-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target RLM-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target RLM-RS.

Otherwise, for a UE  neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs are overlapping with some but not all occasions of the SSB; and

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2

-	P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when the RLM-RS resource is not overlapped with GAP and RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBxRP - TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with GAP  and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBxRP, when the RLM-RS is partially overlapped with GAP  and the RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=Psharing factor1-TSSBxRPP=Psharing factor1-TSSBxGP, when the RLM-RS resource is partially overlapped with GAP  and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP  (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the RLM-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, an RLM-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, an RLM-RS resource or an SMTC occasion is not considered as overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P N) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P N) TDRX | Ceil(5 P N) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.2-3: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N Note2) TSSB) | Max(100, Ceil(5 P N Note2) TSSB) |
| DRX cycle≤80ms | Max(200, Ceil(15 P N Note2) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N Note2) Max(TDRX,TSSB)) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2. |  |  |

Table 8.1.2.2-4: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1(deactivated PSCell)

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Ceil(10 P) measCyclePSCell | Ceil(5 P) measCyclePSCell |
| DRX cycle≤ 320ms | Ceil(15 P) Max(TDRX, measCyclePSCell) | Ceil(7.5 P) Max(TDRX, measCyclePSCell) |
| DRX cycle> 320ms | Ceil(10 P) Max(TDRX, measCyclePSCell) | Ceil(5 P) Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

Table 8.1.2.2-5: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2(deactivated PSCell)

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Ceil(10 P N) measCyclePSCell | Ceil(5 P N) measCyclePSCell |
| DRX cycle≤ 320 ms | Ceil(15 P N) Max(TDRX, measCyclePSCell) | Ceil(7.5 P N) Max(TDRX, measCyclePSCell) |
| DRX cycle> 320 ms | Ceil(10 P N) Max(TDRX, measCyclePSCell) | Ceil(5 P N) Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

--- End of change 1 ---

--- Start of change 2 ---

### 8.1.3	Requirements for CSI-RS based radio link monitoring

<unchanged sections omitted>

#### 8.1.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS ms period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS ms evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS ms period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS ms evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-1 for FR1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-2 for FR2 with scaling factor N=1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-3 for FR1 (deactivated PSCell).

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-4 for FR2 (deactivated PSCell) with scaling factor N=1.

The requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target RLM-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where an RLM-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target RLM-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target RLM-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs] are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is not overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the RLM-RS resource is not overlapped with GAP and RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the RLM-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

- 	If the UE is configured with Pre-MG, an RLM-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, an RLM-RS resource or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of Mout and Min used in Table 8.1.3.2-1, Table 8.1.3.2-2, Table 8.1.3.2-3 and Table 8.1.3.2-4 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P) × TDRX | Ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P×N)×TCSI-RS) | Max(100, Ceil(Min×P×N) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P×N)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P×N)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P×N) × TDRX | Ceil(Min×P×N) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-3: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P) × measCyclePSCell | Ceil(Min×P) × measCyclePSCell |
| DRX ≤ 320ms | Ceil(1.5 ×Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(1.5 ×Min×P) × Max(TDRX, measCyclePSCell) |
| DRX > 320ms | Ceil(Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(Min×P) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

Table 8.1.3.2-4: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P×N) × measCyclePSCell | Ceil(Min×P×N) × measCyclePSCell |
| DRX ≤ 320ms | Ceil(1.5 × Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(1.5 × Min×P×N) × Max(TDRX, measCyclePSCell) |
| DRX > 320ms | Ceil(Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(Min×P×N) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

--- End of change 2 ---

--- Start of change 3 ---

## 8.1A	Radio Link Monitoring with CCA on Target Frequency

<unchanged sections omitted>

### 8.1A.2	Requirements for SSB Based Radio Link Monitoring

<unchanged sections omitted>

#### 8.1A.2.2	Minimum Requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB,CCA [ms] period becomes worse than the threshold Qout_SSB,CCA within TEvaluate_out_SSB,CCA [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA [ms] period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA [ms] evaluation period. During the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when Lin exceeds Lin,max, where Lin and Lin,max are defined in Table 8.1A.2.2-1.

TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA are defined in Table 8.1A.2.2-1 for FR1.

TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA are defined in Table 8.1A.2.2-2 for FR2-2 with scaling factor N = 12.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target RLM-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where an RLM-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target RLM-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for an RLM-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TSSBxRPP=11-TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs are overlapping with some but not all occasions of the SSB RLM-RS resources; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB RLM-RS resources.

When a measurement gap is configured and the measurement gap is not NCSG,

-	an RLM-RS resource is considered to be overlapped with the GAP f it overlaps a measurement gap occasion, and

-	xRP = MGRP

Otherwise, when NCSG measurement gap is configured,

-	an RLM-RS resource is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

If the UE is configured with Pre-MG, an RLM-RS resource is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

When concurrent gaps are configured, an RLM-RS resource is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

For FR2-2,

-	P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when the RLM-RS resource is not overlapped with measurement gap and RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP - TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBMGRP, when the RLM-RS is partially overlapped with measurement gap and the RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P=Psharing factor1-TSSBMRGP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 1, if the RLM-RS resource outside measurement gap is

- not overlapped with the SSB symbols indicated by SSB-ToMeasure and K data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and K data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier,  and K is defined in clause 9.2.5.3.3, and,

- not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and K data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and K data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured, and K is defined in clause 9.2.5.3.3.

-	Psharing factor = 3, otherwise.

where,

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2-2 band, provided the SMTC offset of all CCs in FR2-2 have the same offset.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR1

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(10ms, P * TSSB) if no DRX is used, Once per Max(10ms, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

Table 8.1A.2.2-2: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR2-2

| Configuration | TEvaluate_out_SSB,CCA (ms) | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil([12]*P* N)*TSSB) | Max(100, Ceil((5 +Lin)*P* N)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*[10]*P* N)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5 +Lin)*P* N)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil([10]*P* N)*TDRX | Ceil((5+Lin)*P* N)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions groups which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. A RLM-RS SSB occasions group consists of N consecutive RLM-RS SSB occasions, and the RLM-RS SSB occasions group is not available at the UE when at least one RLM-SSB occasion in the group is not transmitted by the gNB. When DRX is configured, Lin is the number of DRX cycles groups which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. A DRX group consists of N DRX cycles, and the DRX group is not available when there is at least one DRX in which at least one RLM-RS SSB occasion is not available. The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. |  |  |

--- End of change 3 ---

--- Start of change 4 ---

## 8.5	Link Recovery Procedures

<unchanged sections omitted>

### 8.5.2	Requirements for SSB based beam failure detection

<unchanged sections omitted>

#### 8.5.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-1 or Table 8.5.2.2-4 (deactivated PSCell) for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-2 or Table 8.5.2.2-5 (deactivated PSCell) for FR2 with scaling factor N=8 for FR2-1 and N=12 for FR2-2, for FR2 power classes other than power class 6 or for FR2 power class 6 when highSpeedMeasFlagFR2-r17 is not configured.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-3 for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target BFD-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a BFD-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target BFD-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target BFD-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB.

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2

-	P=11-TSSBTSMTCperiod, when BFD-RS resource is not overlapped with GAPs and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBxRP - TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with GAP (TSSB <xRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TSSBxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the BFD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, given the SMTC offset of all CCs in FR2 provided the same offset.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, a BFD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a BFD-RS resource or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

Longer evaluation period would be expected if the combination of BFD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P N) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-3: Evaluation period TEvaluate_BFD_SSB for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N Note2) TSSB) |
| DRX cycle ≤ 80ms | Max(50, Ceil(7.5 P N Note2) Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2. |  |

Table 8.5.2.2-4: Evaluation period TEvaluate_BFD_SSB for deactivated PSCell in FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Ceil(5 P) measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(7.5 P) Max(measCyclePscell, TDRX) |
| DRX cycle > 320ms | Ceil(5 P) Max(measCyclePscell, TDRX) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

Table 8.5.2.2-5: Evaluation period TEvaluate_BFD_SSB for deactivated PSCell in FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Ceil(5 P N) measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(7.5 P N) Max(measCyclePscell, TDRX) |
| DRX cycle > 320ms | Ceil(5 P N) Max(measCyclePscell, TDRX) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

--- End of change 4 ---

--- Start of change 5 ---

### 8.5.3	Requirements for CSI-RS based beam failure detection

<unchanged sections omitted>

#### 8.5.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS ms period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.3.2-1 or Table 8.5.3.2-3 (deactivated PSCell) for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.3.2-2 or Table 8.5.3.2-4 (deactivated PSCell) for FR2 with N=1. The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target BFD-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a BFD-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target BFD-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

TL1 is periodicity of the target BFD-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TCSI-RSxRPP=11-TCSI-RSMGRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS.

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when the BFD-RS resource is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with GAP (TCSI-RS < xRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the BFD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, a BFD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a BFD-RS resource or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

Note:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MBFD used in Table 8.5.3.2-1 and Table 8.5.3.2-2 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PBFD used in Table 8.5.3.2-1 and Table 8.5.3.2-2 are defined as

For each CSI-RS resource in the set  configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PBFD = 1.

For each CSI-RS resource in the set  configured for PSCell in NR-DC

-	PBFD = 2 if UE is configured for beam failure detection on SCell, 1 otherwise.

For each CSI-RS resource in the set  configured for a SCell

-	PBFD = Z in EN-DC or NE-DC or SA.

-	PBFD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell.

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD P PBFD) TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | Ceil(MBFD P PBFD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD P N PBFD) TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 × MBFD P N PBFD) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | Ceil(MBFD P N PBFD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-3: Evaluation period TEvaluate_BFD_CSI-RS for deactivated PSCell in FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Ceil(MBFD P PBFD) measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(1.5 × MBFD P PBFD) Max(TDRX, measCyclePscell) |
| DRX cycle > 320ms | Ceil(MBFD P PBFD) Max(TDRX, measCyclePscell) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

Table 8.5.3.2-4: Evaluation period TEvaluate_BFD_CSI-RS for deactivated PSCell in FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Ceil(MBFD P N PBFD) measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(1.5 × MBFD P N PBFD) Max(TDRX, measCyclePscell) |
| DRX cycle > 320ms | Ceil(MBFD P N PBFD) Max(TDRX, measCyclePscell) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

--- End of change 5 ---

--- Start of change 6 ---

### 8.5.5	Requirements for SSB based candidate beam detection

<unchanged sections omitted>

#### 8.5.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB ms period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.5.5.2-1 and 8.5.5.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_SSB is defined in Table 8.5.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in Table 8.5.5.2-2 for FR2 with scaling factor N=8 for FR2-1 and N=12 for FR2-2.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a CBD-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target CBD-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB,

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBxRP - TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5 × TSMTCperiod

-	P=Psharing factor1-TSSBxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=Psharing factor1-TSSBxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the CBD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	If the UE is configured with Pre-MG, a CBD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG is measurement gap configured,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, a CBD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a CBD-RS resource or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of PCBD used in Table 8.5.5.2-1 and Table 8.5.5.2-2 are defined as

For each SSB resource in the set  configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PCBD = 1.

For each SSB resource in the set  configured for PSCell in NR-DC

-	PCBD = 2 if UE is configured for candidate beam detection on SCell, 1 otherwise.

For each SSB resource in the set  configured for a SCell

-	PCBD = Z in EN-DC or NE-DC or SA.

-	PCBD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell

-	PCBD is the number of band(s) on which UE is performing candidate beam detection only for SCell.

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 P PCBD) TSSB) |
| DRX cycle > 320ms | Ceil(3 P PCBD) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.5.2-2: Evaluation period TEvaluate_CBD_SSB for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 P N PCBD) TSSB) |
| DRX cycle > 320ms | Ceil(3 P N PCBD) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

--- End of change 6 ---

--- Start of change 7 ---

### 8.5.6	Requirements for CSI-RS based candidate beam detection

<unchanged sections omitted>

#### 8.5.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in table 8.5.6.2-1 and 8.5.6.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.5.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.5.6.2-2 for FR2 with scaling factor N=8 for FR2-1 and N=12 for FR2-2.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a CBD-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target CBD-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when candidate beam detection RS is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P =Psharing factor, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod,, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=31-TCSI-RSxRP,, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the CBD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

- 	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps the measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, a CBD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a CBD-RS resource or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

Note:	The overlap between CSI-RS for CBD and SMTC means that CSI-RS for CBD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and GAP  configurations does not meet previous conditions.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 8.5.6.3.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MCBD used in Table 8.5.6.2-1 and Table 8.5.6.2-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PCBD used in Table 8.5.6.2-1 and Table 8.5.6.2-2 are defined as

-	For each CSI-RS resource in the set  configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PCBD = 1.

-	For each CSI-RS resource in the set  configured for PSCell in NR-DC

-	PCBD = 2 if UE configured for candidate beam detection on SCell, 1 otherwise.

-	For each CSI-RS resource in the set  configured for a SCell

-	PCBD = Z in EN-DC or NE-DC or SA.

-	PCBD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell

-	PCBD is the number of band(s) on which UE is performing candidate beam detection only for SCell.

Table 8.5.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P PCBD) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P PCBD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P N PCBD) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P N PCBD) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

--- End of change 7 ---

--- Start of change 8 ---

## 8.5A	Link Recovery Procedures when CCA is used on target frequency

<unchanged sections omitted>

### 8.5A.2	Requirements for SSB based beam failure detection

<unchanged sections omitted>

#### 8.5A.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured BFD-RS SSB resource in set q0 estimated over the last TEvaluate_BFD_SSB_CCA ms period becomes worse than the threshold Qout_LR_SSB,CCA within TEvaluate_BFD_SSB_CCA ms period.

The value of TEvaluate_BFD_SSB_CCA is defined in Table 8.5A.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB_CCA is defined in Table 8.5A.2.2-2 for FR2-2 with scaling factor N=12.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target BFD-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a BFD-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target BFD-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for a BFD-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

-	P=11-TSSBxRPP=11-TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the BFD-RS SSB.

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the BFD-RS SSB.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a BFD-RS resource is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a BFD-RS resource is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

- 	If the UE is configured with Pre-MG, a BFD-RS resource is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a BFD-RS resource is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

For FR2-2,

-	P=11-TSSBTSMTCperiod, when BFD-RS resource is not overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with measurement gap and the BFD-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP - TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with measurement gap (TSSB <MGRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=Psharing factor1-TSSBMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

where,

-	Psharing factor = 1, if the BFD-RS resource outside measurement gap is

-	not overlapped with  the SSB symbols indicated by SSB-ToMeasure and K data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and K data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and K is defined in clause 9.2.5.3.3, and;

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and K data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and K data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured, and K is defined in clause 9.2.5.3.3.

-	Psharing factor = 3, otherwise.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2-2 band, given the SMTC offset of all CCs in FR2-2 provided the same offset.

Longer evaluation period would be expected if the combination of BFD-RS SSB resource, SMTC occasion and GAP configurations does not meet previous conditions.

Table 8.5A.2.2-1: Evaluation period TEvaluate_BFD_SSB_CCA for FR1

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |  |
| --- | --- | --- |
|  | BFD-RS SSB Es/Iot Note2 ≥ -7 dB | BFD-RS SSB Es/Iot Note2 < -7 dB |
| no DRX | Max(50, Ceil((10 P) TSSB)) | Max(50, Ceil((12 P) TSSB)) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 8 P) Max(TDRX,TSSB)) | Max(50, Ceil(1.5 10 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(7 P) TDRX | Ceil(8 P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. Note 2: BFD-RS SSB Es/Iot is the averaged BFD-RS SSB Es/Iot over the most recent previous evaluation period. |  |  |

Table 8.5A.2.2-2: Evaluation period TEvaluate_BFD_SSB_CCA for FR2-2

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |
| --- | --- |
| no DRX | Max(200, Ceil([12]*P* N)*TSSB) |
| DRX cycle ≤ 320ms | Max(200, Ceil(1.5*[10]*P* N)*Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil([10]*P* N)*TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. |  |

--- End of change 8 ---

--- Start of change 9 ---

### 8.5A.5	Requirements for SSB based candidate beam detection

<unchanged sections omitted>

#### 8.5A.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CBD-RS SSB resource in set  estimated over the last TEvaluate_CBD_SSB_CCA ms period becomes better than the threshold Qin_LR,CCA provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.5A.5.2-1 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_SSB_CCA is defined in Table 8.5A.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB_CCA is defined in Table 8.5A.5.2-2 for FR2-2 with scaling factor N=TBD.

For FR1,

- 	For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

- 	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS.

-	For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a CBD-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target CBD-RS.

-	For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for a CBD-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with measurement gap occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

-	P=11-TSSBxRPP=11-TSSBMRGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CBD-RS SSB,

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CBD-RS SSB.

For FR2-2,

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP - TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5 × TSMTCperiod

-	P=Psharing factor1-TSSBMGRP, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P=Psharing factor1-TSSBMGRP, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

where,

-	Psharing factor = 1, if the candidate beam detection RS outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and TBD data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and TBD data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and;

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and TBD data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and TBD data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured

-	Psharing factor = 3, otherwise.

-	If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2-2 band, provided the SMTC offset of all CCs in FR2-2 have the same offset.

-

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CBD-RS resource is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a CBD-RS resource is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

- If the UE is configured with Pre-MG, a CBD-RS resource is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

- When concurrent gaps are configured, a CBD-RS resource is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

Table 8.5A.5.2-1: Evaluation period TEvaluate_CBD_SSB_CCA for FR1

| Configuration | TEvaluate_CBD_SSB_CCA (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil((3 + LCBD) P) TSSB) |
| DRX cycle > 320ms | Ceil((3 + LCBD) P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. Note 2: When DRX is not configured, LCBD is the number of CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. When DRX is configured, LCBD is the number of DRX cycles in which at least one of the CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. The UE, which is configured with DRX, is not required to determine the availability of SSB occasions more frequent than Once per Max(25ms, P * TSSB) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. Note 3: LCBD,max=7 for Max(TDRX, TSSB) ≤ 40 assuming TDRX=0 for non-DRX, LCBD,max=5 for 40 < Max(TDRX, TSSB) ≤ 320, LCBD,max=3 for TDRX > 320. Note 4 If LCBD>LCBD,max, the UE shall assume no new candidate beams are found for this evaluation period. |  |

Table 8.5A.5.2-2: Evaluation period TEvaluate_CBD_SSB_CCA for FR2-2

| Configuration | TEvaluate_CBD_SSB_CCA (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil((3 + LCBD) P N) TSSB) |
| DRX cycle > 320ms | Ceil((3 + LCBD) P N) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. Note 2: When DRX is not configured, Lin is the number of CBD-RS SSB occasions group which are not available at the UE during TEvaluate_CBD_SSB,CCA, where LCBD ≤ LCBD,max. A CBD-RS SSB occasions group consists of N consecutive CBD-RS SSB occasions, and the CBD-RS SSB occasions group is not available at the UE when at least one CBD-SSB occasion in the group is not transmitted by the gNB. When DRX is configured, Lin is the number of DRX cycles groups which are not available at the UE during TEvaluate_CBD_SSB,CCA, where Lin ≤ LCBD,max. A DRX group consists of N DRX cycles, and the DRX group is not available when there is at least one DRX in which at least one CBD-RS SSB occasion is not available. The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX. Note 3: LCBD,max=7 for Max(TDRX, TSSB) ≤ 40 assuming TDRX=0 for non-DRX, LCBD,max=5 for 40 < Max(TDRX, TSSB) ≤ 320, LCBD,max=3 for TDRX > 320. Note 4: If LCBD>LCBD,max, the UE shall assume no new candidate beams are found for this evaluation period. |  |

--- End of change 9 ---

--- Start of change 10 ---

### 9.5.4	L1-RSRP measurement requirements

#### 9.5.4.1	SSB based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1. The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8 in Table 9.5.4.1-2.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a SSB resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target SSB.

For a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

Otherwise,  neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TSSBxRP11-TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

P1= 11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is PL1_sharing*Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P1=11-TSSBxRP-TSSBTSMTCperiod, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBxRP, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P1= 11-TSSBmin(TSMTCperiod,xRP), when SSB is partially overlapped with GAP (TSSB < xRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

-	If SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and P2 is valid accoding to 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P11-P1*TSSBP2*TSSB_CDP ,   if P1*TSSB < P2*TSSB_CDP.

-	P = P1, if P1*TSSB > P2*TSSB_CDP.

-	P = 2*P1, if P1*TSSB = P2*TSSB_CDP.

-	Otherwise, P = P1

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and Psharing_factor,CDP is used in 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, an SSB or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with highSpeedMeasFlagFR2-r17 for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80ms< DRX ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N1 = 2 when highSpeedMeasFlagFR2-r17 = set1; N1 = 6 when highSpeedMeasFlagFR2-r17 = [set2]. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

#### 9.5.4.2	CSI-RS based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CSI-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a CSI-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target CSI-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrent gaps according to clause 9.1.8 are configured,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when CSI-RS is not overlapped with a GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=Psharing factor, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=1, when aperiodic CSI-RS resource is not overlapped with GAP

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

Where:

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

TSMTCperiod = the configured SMTC period.

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

If the UE is configured with Pre-MG, a CSI-RS or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if it overlapps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-    If the UE is configured with Pre-MG, a CSI-RS or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-    When concurrent gaps are configured, a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. Note 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17. |  |

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

--- End of change 10 ---

--- Start of change 11 ---

### 9.5A.4	L1-RSRP measurement requirements

#### 9.5A.4.1	SSB based L1-RSRP Reporting

The value of TL1-RSRP_Measurement_Period_SSB_CCA is defined in Table 9.5A.4.1-1 for FR1, and in Table 9.5A.4.1-2 for FR2-2, where,

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N = TBD

For FR1, for a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for SSB resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

For FR1, for a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a SSB resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a SSB resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target SSB.

For FR1, for a UE supporting concurrentMeasGap-r17 and when concurrent measurement gaps according to clause 9.1.8 are configured,

-	P value for an SSB resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	TL1 is periodicity of the target SSB.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For FR1,

-	P=11-TSSBxRP11-TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2-2,

-	P=11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP-TSSBTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBMGRP, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P= 11-TSSBTSMTCperiod, when S SSB is partially overlapped with measurement gap (TSSB <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is  Psharing factor1-TSSBMRGP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

Where:

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	Where:

TSSB = ssb-periodicityServingCell

- TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured

If the UE is configured with Pre-MG, an SSB is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When a measurement gap is configured,

-	an SSB is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	When NCSG is configured,

-	an SSB is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, an SSB is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

UE shall report RSRP_0 (Not valid) if L1>L1max, where L1 and L1max are defined in Table 9.5A.4.1-1.

Table 9.5A.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB_CCA for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_CCA (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil((M+L1)*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: L1=0 if higher layer parameter timeRestrictionForChannelMeasurement is configured. Otherwise, when DRX is not configured L1 is the number of SSBs not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, and when DRX is configured L1 is the number of DRX cycles in which at least one SSB is not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, where L1 ≤ L1max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(TReport, P * TSSB) if no DRX is used, Once per Max(TReport, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. Note 3: L1max =7 for Max(TDRX,TSSB) ≤ 40ms assuming TDRX=0 for non-DRX, L1max =5 for 40ms < Max(TDRX, TSSB) ≤ 320ms, L1max =3 for TDRX > 320ms. |  |

Table 9.5A.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB_CCA for FR2-2

| Configuration | TL1-RSRP_Measurement_Period_SSB_CCA (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: L1=0 if higher layer parameter timeRestrictionForChannelMeasurement is configured. Otherwise, when DRX is not configured L1 is the number of SSB occasion groups not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, where L1 ≤ L1max. An SSB occasions group consists of N consecutive SSB occasions, and the SSB occasions group is not available at the UE when at least one SSB occasion in the group is not transmitted by the gNB. When DRX is configured, L1 is the number of DRX cycle groups in which at least one SSB occasion is not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, where L1 ≤ L1max. A DRX group consists of N DRX cycles, and the DRX cycle group is not available when there is at least one DRX in which at least one SSB occasion is not available. The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX. Note 3: L1max =7 for Max(TDRX,TSSB) ≤ 40ms assuming TDRX=0 for non-DRX, L1max =5 for 40ms < Max(TDRX, TSSB) ≤ 320ms, L1max =3 for TDRX > 320ms. |  |

--- End of change 11 ---

--- Start of change 12 ---

### 9.8.4	L1-SINR measurement requirements

#### 9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

edicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only is defined in Table 9.8.4.1-1 for FR1 and in Table 9.8.4.1-2 for FR2, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] are configured,

-	P value for CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all GAP configurations, i.e. configured per-UE measurement gaps and per-FR measurement gaps, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CSI-RS.

For a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y] are configured,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum MGRP or VIRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	where a CSI-RS resource or an SMTC occasion is considered to be overlapped with the NCSG measurement gap if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	TL1 is periodicity of the target CSI-RS.

For a UE supporting concurrentMeasGap-r17 and when concurrentgaps are configured,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies (i.e. concurrent measurement gaps according to clause 9.1.8, concurrent measurement gap(s) and Pre-MG(s) according to clause [Case 1 requirements, 9.1.x] and concurrent measurement gap(s) and NCSG measurement gap(s) according to clause [Case 2 requirements, 9.1.y],

For the value of P in FR1,

-	P=11-TCSI-RSxRP11-TCSI-RSxGP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For the value of P in FR2,

-	P=1, when CSI-RS is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP11-TCSI-RSxGP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod 11-TCSI-RSxGP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP  and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRPPsharing factor1-TCSI-RSxGP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP  and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,xRP), when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP .

-	P=Psharing factor1-TCSI-RSxRP, Psharing factor1-TCSI-RSxGPwhen CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP  (TSMTCperiod < xRP)

Where:

Psharing factor = 1, if the CSI-RS configured for L1-SINR measurement outside gap is

not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured.

-	TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

-

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a CSI-RS is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, a CSI-RS resourse or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and GAP configurations does not meet previous conditions.

Table 9.8.4.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

Table 9.8.4.1-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

--- End of change 12 ---