3GPP TSG-RAN WG4 Meeting # 108	R4-2314337

Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.8.4

Source:	OPPO

Title:	WF on NR FeRRM (part2)

Document for:	Approval

Topic #1: RRM Core requirements for FR1-FR1 NR-DC

Agreement: RAN4 can leverage the conclusion from R17 maintenance of SCG activation/deactivation if any.

Issue 1-1: Side condition for RACH-less SCG activation/deactivation

No consensus. Keep using current definition of side conditions for CR drafting with bracket.

- Option 1(Apple, OPPO): keep current side conditions for Tsearch and

    - known target cell

    - known TCI state

    - UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD

In FR1 and FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise PSCell is unknown. If the UE is configured to perform BFD while the SCG is deactivated - The TCI state is known if the following conditions are met: - During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated, - UE has not detected beam failure - Otherwise, the TCI state is unknown.

- Option 2(Ericsson): New known/unknown PScell definition #1

- During the last 5 seconds before the reception of the SCG activation command: - The configured TAT timer alignment timer is running - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3.

- Option 3(Nokia): New known/unknown PScell definition #2

In FR1 and FR2, the deactivated PSCell is known if it has been meeting the following conditions: - If bfd_and_RLM with value true is configured for the deactivated PSCell and the TCI state is known, and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. or - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

Issue 1-2: Tsearch requirement for RACH-less PSCell activation

- Option 1(Apple, OPPO): current requirements + current known/unknown PScell definition.

| Current Tsearch | Current known/unknown PScell definition. |
| --- | --- |
| For RACH-less based PSCell activation, if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known PSCell. There are no requirements if PSCell is unknown. | In FR1 and FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise PSCell is unknown. |

- Option 2(Ericsson): current requirements + new known/unknown PScell definition.

| Current Tsearch | New known/unknown PScell definition #1 |
| --- | --- |
| For RACH-less based PSCell activation, if RLM and BFD are configured: if TCI state is known or if the target cell is a known PSCell, Tsearch = 0 ms. Otherwise, there are no requirements. | During the last 5 seconds before the reception of the SCG activation command: - The configured TAT timer alignment timer is running - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. |

- Option 3(Nokia): current requirements + new known/unknown PScell definition.

| Current Tsearch | New known/unknown PScell definition #2 |
| --- | --- |
| For RACH-less based PSCell activation, if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known PSCell. There are no requirements if PSCell is unknown. | In FR1 and FR2, the deactivated PSCell is known if it has been meeting the following conditions: - If bfd_and_RLM with value true is configured for the deactivated PSCell and the TCI state is known, and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. or - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise PSCell is unknown. |

- Option 4(Nokia): updated requirements + current side condition unchanged

| New Tsearch | Current known/unknown PScell definition. |
| --- | --- |
| For RACH-less based PSCell activation, if RLM and BFD are configured: if TCI state is known or if the target cell is a known PSCell, Tsearch = 0 ms. Otherwise, there are no requirements. | In FR1 and FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise PSCell is unknown. |

Issue 1-3:  LS on clarification of UE behaviour during RACH-less SCG activation

No conclusion is reached on LS out in this meeting.

- Option 1(vivo): LS is not needed to check with RAN2 on the indication for the case that beam failure has been detected during SCG activation procedure

  - RAN4 assumes UE should not indicate beam failure to the network if beam failure has been detected during SCG activation procedure.

- Option 2(Ericsson): Send LS to RAN2 on clarification of UE behavior during RACH-less SCG activation

Issue 2-1: Tsearch for RACH-based PSCell activation

- Option 1(Apple, OPPO): Not necessary to change the RACH based PSCell activation requirement in agreed CR R4-2310080, i.e.,

  - If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

- Option 2 (Nokia):

| New Tsearch | Current known/unknown PScell definition. |
| --- | --- |
| if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell configured with bfd-and-RLM with value true and no RLM has occurred, then Tsearch = [1]* Trs ms, otherwise if Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. | In FR1 and FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise PSCell is unknown. |

- Option 3 (Nokia):

| Current Tsearch | New known/unknown PScell definition. |
| --- | --- |
| For RACH based PSCell activation, if the target cell is a known NR PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms. | In FR1 and FR2, the deactivated PSCell is known if it has been meeting the following conditions: - If bfd_and_RLM with value true is configured for the deactivated PSCell and the TCI state is known, and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. or - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise PSCell is unknown. |