3GPP TSG-RAN WG4 Meeting #108                                                               R4-2314342

Toulouse, France, August 21st – 25th, 2023

Agenda item:			8.12.6

Source:	Samsung

Title:	Ad-hoc minutes for FR2 HST enhancement RRM requirements

Document for:	Approval

# Introduction

For Rel-18 work item on enhanced NR support for high speed train scenario in frequency range 2, there are two threads  [215] and [216] organized for RRM discussion on RAN4#108, and one ad-hoc meeting is planned for further discussion after 1st round online treatment.

# Topic #1: Simultaneous multi-panel operation for train roof-mounted FR2 high power devices

## Background and Progress on Monday RRM session

[Moderator] Existing agreements from RAN4#107 in approved WF [R4-2310041], in which the scope of RRM requirement impact is provided as below:

- Agreement: - Below green highlighted items from Thursday Ad-Hoc Session: - Below yellow highlighted items agreed in this WF - Table 1. RRM requirement impact summary for FR2 HST multi-panel RX (informative) RRM Req. Category; Sub-Category; Proposed WF on RRM requirement impact; Work Split. RRC_IDLE state mobility; Cell Re-selection; Precluded. RRC_CONNECTED state mobility; Handover; Precluded. RRC Connection Mobility Control; Precluded. Timing; MRTD; New requirement required; Intel. Signalling characteristics; Requirements for SSB based RLM; New requirement required: #### 8.1.2.2 Minimum requirement 8.1.2.3 Measurement restrictions for SSB based RLM; Qualcomm. Requirements for CSI-RS based RLM; Precluded. Scheduling availability of UE during RLM; FFS 8.1.7.3; Ericsson, Nokia. Requirements for SSB based BFD; New requirement is required 8.5.2.2 Minimum requirement 8.5.2.3 Measurement restriction for SSB based beam failure detection; Xiaomi. Requirements for SSB based CBD; Precluded. Requirements for CSI-RS based BFD/CBD; Precluded. Scheduling availability of UE during BFD/CBD; Precluded. Active TCI state switching delay; To be treated UL TX timing. Uplink spatial relation switch delay; Precluded. Measurement Procedure; NR intra-frequency measurements w/o gaps; Precluded. Measurement Requirements for SSB based L1-RSRP; New requirement is required #### 9.5.4.1 SSB based L1-RSRP Reporting; #### 9.5.5.1Measurement restriction for SSB based L1-RSRP; Huawei. Measurement Requirements for CSI-RS based L1-RSRP; Precluded. Scheduling availability of UE during L1-RSRP measurement; New requirement required; Samsung. L1-SINR measurements for Reporting; Precluded.

[Moderator] The following agreement achieved on Monday online session:

Sub-topic 1-1: Possible scenarios analysis in Rel-18 FR2 HST - Agreement - Define the RRM requirements for Rel-18 FR2 HST Multi-Rx based on the following scenarios described in R4-2314160 clause 1.2.1: - Scenario A & D: The FR2 PC6 UE uses two active panels pointed forward and backward to receive DL signal transmitted from two RRHs with different QCL TypeD RSs at the same time. Issue 1-3-1: Whether to define the new scheduling restriction requirements for data+L1 measurement - Tentative agreement - Define enhanced scheduling restriction requirements for data+L1 measurement for Rel-18 FR2 HST multi-panel simultaneous reception - No scheduling restrictions apply for the case of simultaneous data reception on one panel and SSB reception on the other panel in case of data and SSB transmission from different RRHs - Conclusion: further discuss based on tentative agreement and aim to conclude in this meeting

## Sub-topic 1-2: Whether need to define a new or reuse the existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception

[Moderator] In last meeting, following WF was approved with FFS point

Issue 1-1-5: The applicability and necessity of GBBR in FR2 HST - Agreement (Updated agreement from Thursday Ad-Hoc Session): - FR2 HST requirement shall be defined without GroupBasedBeamReporting-r17 supported and GBBR configured - FFS the performance degradation if GBBR is not configured. New Issue related to Issue 1-1-5: Applicable condition for FR2 HST multi-RX in the spec, if without GBBR as prerequisite: - Agreement from Thursday Ad-Hoc Session: - Applicable condition for FR2 HST multi-panel requirement, if without GBBR as prerequisite - FR2 PC6 UE shall support the multi-panel simultaneous reception capability - FFS new or existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception - NW shall configure Rel-17 HST flag (highSpeedMeasFlagFR2) and bi-direction flag (i.e., highSpeedDeploymentTypeFR2)

Issue 1-2-1: Whether need to define a new or reuse the existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception

[Background] The agreement regarding simultaneousReceptionDiffTypeD-r16 capability achieved in FR2 Multi-Rx in RAN4 #106-bis-e [R4-2306318] is duplicated as below. The agreement indicates that the IE is only applicable for supporting of simultaneous reception PDSCH with different QCL-type D RS.

WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1) Issue 1-4-1: Clarification/understanding on R16 UE capabilitiy simultaneousReceptionDiffTypeD <Agreement >: - It is RAN4 understanding that the R16 UE capability simultaneousReceptionDiffTypeD is only applicable for simultaneous PDSCH reception.

[Moderator] The contribution [R4-2312188] discuss the prerequisite of reusing the existing IE, summarized as option 2.

- Proposals

  - Option 1 (Ericsson, Samsung, Qualcomm Incorporated): Define a new UE capability to indicate support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE

    - Option 1-1 (Samsung): The design of such UE capability is postponed until there is sufficient progress on requirements

  - Option 2 (Nokia):

    - RAN4 can reuse simultaneousReceptionDiffTypeD-r16 UE capability when highSpeedMeasFlagFR2 and highSpeedDeploymentTypeFR2-r17 = bidirectional NW signalling are configured as a pre-requisite of Rel-18 FR2 HST enhanced operation (i.e., simultaneous reception)

    - RAN4 should consider multi-RX agreement for defining new or reusing existing UE capability for FR2 enhanced simultaneous reception in DL.

- Recommended WF

  - Define a new UE capability to indicate support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE

    - The design of such UE capability can be further discussed in UE feature list discussion.

Ad-Hoc meeting:

- Discussion:

- Agreement:

## Sub-topic 1-3: Whether to define the new scheduling restriction requirements for data+L1 measurement

Issue 1-3-1: Whether to define the new scheduling restriction requirements for data+L1 measurement

- Tentative agreement

  - Define enhanced scheduling restriction requirements for data+L1 measurement for Rel-18 FR2 HST multi-panel simultaneous reception

    - No scheduling restrictions apply for the case of simultaneous data reception on one panel and SSB reception on the other panel in case of data and SSB transmission from different RRHs

- Conclusion: further discuss based on tentative agreement and aim to conclude in this meeting

Recommended WF:

  - [Ericsson/Nokia] Draft CR on scheduling availability of UE performing RLM for HST FR2

In HST FR2 scenario, the power class 6 UE supporting [simultaneousReceptionDiffTypeD-r16] is able to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI with one Rx chain on SSB symbols to be measured for RLM when performing SSB based RLM measurement with the other Rx chain, provided that the highSpeedMeasFlagFR2-r17 is configured and highSpeedDeploymentTypeFR2-r17 is bidirectional.

  - [Samsung] Draft CR on scheduling availability of UE performing L1-RSRP measurement for HST FR2

- In FR2 HST scenario, when highSpeedMeasFlagFR2-r17 and highSpeedDeploymentTypeFR2 are configured for a UE indicating [simultaneousReceptionFR2HST-r18].

For the case where PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI are transmitted from more than one RRHs in one slot, scheduling restriction relaxation can be made for one symbol before and one symbol after each consecutive SSB symbols, periodic CSI-RS resource, aperiodic CSI-RS resource to be measured for L1-RSRP measurement with multi-panel operation.

If following conditions are met,

-	FR2 PC6 UE is equipped with two panels, and activates the two panels all the time or most of the time, and

-	The RSs and one of the PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI are transmitted through different RRHs corresponding to two serving beam orientation at the same time, and

-	If CSI-RS resource is configured for L1-RSRP measurement, the CSI-RS resource configured for L1-RSRP measurement has the same QCL source as the active TCI state of any one of the PDCCH/PDSCHs, but has different QCL Type D with the active TCI state of other PDCCH/PDSCHs, or

-	If SSB resource is configured for L1-RSRP measurement, the SSB resource configured for L1-RSRP measurement is QCL Type D with the active TCI state of any one of the PDCCH/PDSCHs, but has different QCL Type D with the active TCI state of other PDCCH/PDSCHs

Note: The transmission beams from two RRHs are pointing opposite direction of train movement trend

Ad-Hoc meeting:

- Discussion:

- Agreement:

## Sub-topic 1-4: Whether to/How to define the new measurement restriction requirements for simultaneous L1+L1 measurement

Issue 1-4-1: Restriction on SSB configuration for neighboring RRHs

[Background] A recap from TS 38.133, the specified restriction on SSB transmission for FR2 HST is copied as follows for reference.

3.6.14 Applicability of requirements for FR2 Power Class 6 For Rel-17 FR2 power class 6 for the UE type of high speed train roof-mounted UE, UE shall only be in NR SA operation. Measurement and evaluation period requirements with highSpeedMeasFlagFR2-r17 configured in clause 4.2.2.3, 8.1.2.2, 8.5.2.2, 9.2.5, 9.2.6, 9.5.4, [9.8.4], delay requirements in clause 6.2.1.2.1, 8.10.3A, and UL timing adjustment in clause 7.1.2.3, are only applicable when any two SSBs from adjacent [RRHs] are not on the same or consecutive symbols. Editor note: The definition of RRH shall be added in clause 3.1.

- Proposals

  - Option 1 (Nokia, Huawei, HiSilicon): There is no need to consider measurement restriction enhancement on SSB configuration for neighboring RRHs for Rel-18 FR2 HST multi-panel simultaneous reception

- Recommended WF

  - There is no need to consider measurement restriction enhancement on SSB configuration for neighboring RRHs for Rel-18 FR2 HST multi-panel simultaneous reception

Ad-Hoc meeting:

- Discussion:

- Agreement:

## Sub-topic 1-5: Whether to/How to reduce the sweeping factor N1

[Moderator] In last meeting, following WF was approved with FFS point

Whether to consider/enhance simultaneous L3+L3, L3+L1 and L1+L1 measurement for multi-panel RX in Rel-18 FR2 HST WI? - Agreement from Thursday Ad-Hoc Session: - RAN4 shall consider the simultaneous L1+L1 measurement for multi-panel RX in Rel-18 FR2 HST WI, and new requirements shall be defined accordingly - FFS applicable condition of the enhancement for new requirement: - Restriction on SSB configuration for neighboring RRHs - Enhancement on only set1, or both set1 and set2. - The sweeping factor N1 can be reduced in FR2 HST multi-panel simultaneous reception - FFS how to reduce N1

[Moderator] [R4-2312189] elaborate their views on whether to reduce the Rx beam sweeping factor in FR2 HST Multi-Rx. More specifically, they provide some insights to the potential gains that could be achieved with simultaneous two-panel measurements and reception, and verify the necessity of considering reduction of Rx beam sweeping factor under different uni-directional scenarios with DPS through SLS. Their valuable observations are captured as follows.

- Observations from [R4-2312189]

  - For uni-directional deployment, reducing scaling factor increases the beam switching and ping-pong rates, but decreases the time of outage and beam failure indication rate while improving the SINR level in low SINR percentiles.

  - Reduced scaling factors improve mobility performances especially in cases when the train is moving opposite to the beam orientation and with long DRX cycles used.

  - Observation 1 and Observation 2 could be generalized to the bi-directional deployment with the UE capable of doing simultaneously data reception and measurement.

Issue 1-5-1: Whether to reduce the sweeping factor N1

- Proposals

  - Option 1 (Nokia, Ericsson, Samsung): Yes.

    - Option 1-1 (Nokia): Introduce enhanced scaling factors in L1 measurement requirements for PC6 UE supporting [simultaneousReceptionDiffTypeD-r16] and when highSpeedDeploymentTypeFR2-r17 = bidirectional.

    - Option 1-2 (Ericsson): RX sweep beam number in two active panel reception operation shall be abbreviated from the number used in single active panel reception in Rel-17 since each panel directs forward or backward direction only.

  - Option 2 (Qualcomm Incorporated): No.

    - No need to reduce the Rx beam sweeping scaling factor for a UE operating in a simultaneous multi-panel reception mode

    - Number of Rx beams the UE sweeps is not equally distributed on both the panels and can dynamically change along the train track

  - Option 3 (Huawei, HiSilicon):

    - For FR2 HST multi-panel simultaneous reception, beam sweeping factor reduction is not considered for SSB based L1 measurements configured with set1(N1=2)

    - For FR2 HST multi-panel simultaneous reception, whether to reduce beam sweeping factor for SSB based L1 measurements configured with set2(N1=6) is up to UE capability.

- Recommended WF

  - TBA

Ad-Hoc meeting:

- Discussion:

- Agreement:

Issue 1-5-2: How to reduce the sweeping factor N1

- Proposals

  - Option 1 (Nokia): The N1 for PC6 UE with simultaneous L1 measurement capability can be enhanced to N1 = 1 when highSpeedMeasFlagFR2-r17 = set1; N1 = 3 when highSpeedMeasFlagFR2-r17 = set2

  - Option 2 (Ericsson, Xiaomi, Nokia): To define N1= [1] for set1 configuration, N2=[3] for set2 configuration

  - Option 3 (Samsung):

    - For set1 configuration: N1 can be further reduced to [1]

    - For set2 configuration: The candidate values for beam sweeping factor reduction can be {3,4} for FR2 HST Multi-RX

    - RAN4 to define new optional UE capability for beam sweeping factor N1 reduction for L1-RSRP measurement if the FR2 PC6 UE is capable of multi-Rx operation

- Recommended WF

  - TBA

Ad-Hoc meeting:

- Discussion:

- Agreement:

## Sub-topic 1-6: Necessity of introducing other requirements enhancement

Issue 1-6-1: L3 scheduling restriction requirement enhancement

[Moderator][R4-2312359] provide their views on L3 scheduling restriction requirement enhancement for Rel-18 FR2 HST multi-Rx.

- Proposals

  - Option 1 (Ericsson): Regarding L3 scheduling restriction, additional requirements shall be added as follows:

    - In HST scenario, the power class 6 UE is able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on one RX chain when performing SS-RSRP, SS-SINR or SS-RSRQ measurement on an FR2 intra/inter-frequency cell on the other RX chain, provided that the highSpeedMeasFlagFR2-r17 is configured and highSpeedDeploymentTypeFR2-r17 is bidirectional.

[Moderator]It is moderator understanding that the previous agreement in the WF already preclude the further discussion on the related issue, which is copied as below. Therefore, the discussion is ruled out in this meeting.

Whether to consider/enhance simultaneous data+L1 measurement and data+L3 measurement for multi-panel RX in Rel-18 FR2 HST WI? - Agreement from Thursday Ad-Hoc Session: - RAN4 shall preclude any enhancement on the scheduling restriction requirement for simultaneous L3 measurement and data reception in Rel-18 FR2 HST.

Ad-Hoc meeting:

- Discussion:

- Agreement:

Issue 1-6-2: Other requirements need to be enhanced

[Moderator][R4-2312188] indicate that some additional measurement restrictions / scheduling availability requirements need to be enhanced in Rel-18 FR2 HST.

- Proposals

  - Option 1 (Nokia, Nokia Shanghai Bell): RAN4 to enhance the scheduling restriction/availability requirements for mult-Rx capable UE in HST FR2 scenarios:

    - TS 38.133 clause 8.5.5.3; Measurement restriction for SSB based candidate beam detection.

    - TS 38.133 clause 8.5.7.3; Scheduling availability of UE performing beam failure detection on FR2

    - TS 38-133 clause 8.5.8.3; Scheduling availability of UE performing L1-RSRP measurement on FR2 for candidate beam detection

    - TS 38-133 clause 8.18.2.3; Measurement restriction for SSB based beam failure detection	(Multi-TRP)

    - TS 38-133 clause 8.18.5.3; Measurement restriction for SSB based candidate beam detection (Multi-TRP)

    - TS 38-133 clause 8.18.9.3; Scheduling availability of UE performing L1-RSRP measurement on FR2 for candidate beam detection

It is moderator understanding that the previous agreement in the WF already preclude the further discussion on the following issues, which is copied as below.

  - RAN4 shall preclude any enhancement on the SSB based CBD requirements for FR2 HST multi-panel RX.

  - Preclude enhancement on scheduling availability of UE during BFD/CBD

- Recommended WF

  - There is no need to consider the requirements enhancement for 8.5.5.3, 8.5.7.3 and 8.5.8.3 in Rel-18 FR2 HST.

  - Preclude all the MTRP related discussions in the issue 1-6-2 until the requirements marked for enhancement in WF [R4-2310041] are specified in Rel-18 FR2 HST multi-Rx.

Ad-Hoc meeting:

- Discussion:

- Agreement:

## Draft CRs for simultaneous multi-panel operation

| R4-2312341 revised to R4-2314286 | Intel | CR on requirements for MRTD | Comment: Against RAN#106 agreement: Agreement The configuration of supporting CA with multi-RX chains - Rel-18 FR2 PC6 UE can be configured with multiple carriers even with multi-RX chains enabled for intra-band contiguous CA: - FFS intra-band non-contiguous CA. |
| --- | --- | --- | --- |
| R4-2313798 (late contribution） | Qualcomm | DraftCR on RLM for FR2 HST Enh |  |
| R4-2311850 revised to R4-2314285 | Xiaomi | CR on requirements for SSB based BFD |  |
| R4-2312357 revised to R4-2314289 | Ericsson, Nokia, Nokia Shanghai Bell | CR on scheduling availability requirement of UE during RLM for FR2 HST |  |
| R4-2313543¦ revised to R4-2314288 | Samsung | CR on scheduling availability requirement of UE during L1-RSRP measurement for FR2 HST |  |
| R4-2312872 revised to R4-2314287 | Huawei, HiSilicon | CR on measurement requirements for SSB based L1-RSRP |  |

Ad-Hoc meeting:

- Discussion:

- Agreement:

# Topic #2: Intra-band carrier aggregation (CA) scenario

## Background and Progress on Monday RRM session

[Moderator] Existing agreements from RAN4#107 in approved WF [R4-2310041], in which the scope of RRM requirement impact is provided as below:

Issue 2-1-6: Expected RRM requirement impact - Agreement: Table 2. RRM requirement impact summary for FR2 HST CA requirement (informative) RRM Req. Category; Proposed WF on RRM requirement impact; Work Split. PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements; For PSS/SSS detection, Time index detection for inter-frequency measurements: New requirement required For PSS/SSS detection, Time index detection for intra-frequency measurements: No impact identified; Nokia. L1-RSRP measurement on SCell; No impact identified. SCell activation delay; FFS; Ericsson. Inter-frequency measurement requirements in Idle mode; New requirement required; Samsung. SSB-based BFD and RLM on SCell; No impact identified. TCI state switching on SCell; No impact identified. *Requirements were classified in the following five categories: - New requirement required: Need to define new requirements in Rel-18 FR2 HST - No impact identified: No impact on the existing requirement identified

[Background] A recap from TS 38.331, the HST related parameters are copied as follows for reference.

HighSpeedConfig field descriptions

HighSpeedDemodCA-Scell If the field is present and UE supports demodulationEnhancementCA-r17, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SCell.

highSpeedDemodFlag If the field is present and UE supports demodulationEnhancement-r16, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SpCell.

highSpeedDeploymentTypeFR2 If the field is present, and field value is unidirectional, the UE shall assume uni-directional deployment or if field value is birectional the UE shall assume bidirectional deployment for FR2 up to 350km/h as specified in TS 38.133 [14].

highSpeedLargeOneSteptUL-TimingFR2 If the field is present, large one step UE autonomous uplink transmit timing adjustment for FR2 up to 350km/h as specified in TS 38.133 [14] is enabled.

highSpeedMeasCA-Scell If the field is present and UE supports measurementEnhancementCA-r17, the UE shall apply the enhanced RRM requirements to the serving frequency of SCell for carrier aggregation to support high speed up to 500 km/h as specified in TS 38.133 [14].

highSpeedMeasFlag If the field is present and UE supports measurementEnhancement-r16, the UE shall apply the enhanced intra-NR and inter-RAT EUTRAN RRM requirements to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports intraNR-MeasurementEnhancement-r16, the UE shall apply enhanced intra-NR RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports interRAT-MeasurementEnhancement-r16, the UE shall apply enhanced inter-RAT EUTRAN RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. This parameter only applies to the serving frequency of SpCell.

highSpeedMeasFlagFR2 If the field is present, the UE shall apply enhanced intra-NR RRM requirement set one to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14], if the field value is set1 or RRM requirement set two if the field value is set2.

highSpeedMeasInterFreq If the field is present and UE supports measurementEnhancementInterFreq-r17, the UE shall apply the enhanced RRM requirements for inter-frequency measurement in RRC_CONNECTED to support high speed up to 500 km/h as specified in TS 38.133 [14].

[Moderator] The following agreement achieved on Monday online session:

Topic #2: Intra-band carrier aggregation (CA) scenario Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST - Agreements - Introduce per-UE capability to indicate whether the UE is capable of supporting the enhanced RRM requirements for Connected and Idle mode inter-frequency measurements for FR2 HST - FFS whether to introduce a cell specific network signalling to inform UE whether to apply the enhanced RRM requirements for Connected and Idle inter-frequency measurement for FR2 HST. - FFS: Do not introduce new network indication or UE capabilities to support CA enhancements for R18 FR2 HST

## Sub-topic 2-1: Inter-frequency measurement requirements

Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST

Recommended WF:

  - Introduce a cell specific network signalling [highSpeedMeasInterFreqFR2-r18] to inform UE whether to apply the enhanced RRM requirements for Connected and Idle inter-frequency measurement for FR2 HST.

Ad-Hoc meeting:

- Discussion:

  - Huawei: name of IE, need “r-18”?

  - QC: really need a new signaling for that? Configure the inter-freq MO and the rel-17 network flag signaling is enough.

  - Nokia:

- Agreement:

  - Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in Connected mode.

  - Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for intra-frequency measurement on SCC for FR2 HST in Connected mode.

- Recommended WF:

  - Discuss latter issue first for SCell activation delay.

  - Do not introduce new network indication or UE capabilities to support CA enhancements for R18 FR2 HST

Ad-Hoc meeting:

- Discussion:

  - Huawei: The CA enhancement capability/flag impact

  - Chair: double check whether or not we need new UE capability.

- Agreement:

Issue 2-1-2: Whether need to introduce signalling for idle mode inter-frequency measurement requirements

Issue 2-1-5: Inter-frequency measurement requirements in Idle mode The necessity of specifying the requirements for inter-frequency measurement requirements in idle mode - Agreement - Specify the requirements for inter-frequency measurement requirements in idle mode. How to define the requirements for inter-frequency measurement requirements in idle mode - Agreement - RAN4 to use Rel-17 FR2-HST principles to define inter-frequency measurement requirements in idle mode, i.e., use beam sweeping scaling factor N1 of 2 and 6 for set1 and set2, respectively - For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, RX sweeping factor is assumed as follows to derive inter-frequency measurement in IDLE mode: RX beam sweeping factor is 2 if [highSpeedMeasFlagFR2-r17] = set1; RX beam sweeping factor is 6 if [highSpeedMeasFlagFR2-r17] = set2

- Proposals

  - Option 1 (Nokia, Nokia Shanghai Bell, Huawei, HiSilicon): Yes

    - Option 1-1 (Nokia): Introduce a new parameter (e.g., highSpeedMeasFlagFR2-CA) broadcast by the system information to indicate to the UE that the new inter-frequency measurement requirements should be applied in idle mode. The new parameter can be optionally included in the idle measurement configuration (MeasIdleConfig to limit it to idle mode measurement configuration)

  - Option 2 (Samsung): No

    - Use the existing IE [highSpeedMeasFlagFR2-r17] to define inter-frequency measurement requirements in idle mode.

- Recommended WF

  - [Moderator] If difficulty to define SIB for Idle mode network flagging, can we further think about the necessity of inter-frequency measurement requirement in idle mode?

Ad-Hoc meeting:

- Discussion:

- Tentative Agreement:

  - Use the existing IE [highSpeedMeasFlagFR2-r17] in SIB to define inter-frequency measurement requirements in idle mode.

## Sub-topic 2-2: Intra-frequency measurement requirements on SCC

- Proposals

  - Option 1 (Nokia, Nokia Shanghai Bell): No

    - No new parameter and signaling need to be introduced to indicate intra-frequency measurement enhancements as the enhancements from Rel-17 naturally apply when highSpeedMeasFlagFR2 is configured.

  - Option 2 (Huawei, HiSilicon): Yes

[Moderator] Below revision is provided in Huawei CR [R4-2312637], but without this change, the requirement shall still be applied for SCC.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated SCell), 9.2.5.2-4(deactivated SCell), 9.2.5.2-5 or 9.2.5.2-6(deactivated SCell), 9.2.5.2-8(deactivated PSCell) or 9.2.5.2-9(deactivated PSCell). - For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.5.2-7; otherwise, T SSB_measurement_period_intra is given in Table 9.2.5.2-2. - For UE supporting power class 6 with [highSpeedCAMeasFlagFR2-r18] configured, the T SSB_measurement_period_intra specified for UE supporting highSpeedMeasFlagFR2-r17 shall apply for SCC.

Ad-Hoc meeting:

- Discussion:

- Agreement:

## Sub-topic 2-3: The RRM impact for SCell activation delay requirement for Rel-18 FR2 HST UE

[Background] A recap from TS 38.133, the specified SCell Activation Delay Requirement for Deactivated SCell is copied as follows for reference.

<TS 38.133 8.3.2 > Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. <unrelated contents omit> If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided: - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, - The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell. - SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided - the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band. <unrelated contents omit>

[Background] In last meeting, the following agreement was achieved.

Issue 2-1-4: SCell activation delay - Agreement - FFS the RRM impact for SCell activation delay requirement for Rel-18 FR2 HST UE

- Proposals

  - Option 1 (Ericsson): SCell activation delay with 3ms Tactivation_time can be applied without having to fulfil legacy conditions in HST FR2 scenario, the below dCR [R4-2312358] proposed:

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE is capable of FR2 power class 6 and provided that the highSpeedMeasFlagFR2-r17 is configured, Tactivation_time is 3 ms.

- Recommended WF

  - [Moderator] Even without this CR, 3ms activation time still applies for the assumed FR2 deployment. If so, do we really need explicit change?

Ad-Hoc meeting:

- Discussion:

  - Chair: really need the change?

  - Ericsson: still have concern on the condition currently defined for FR2 UE.

  - Apple: the intention of this enhancement is not clear.

  - Ericsson: add editorial notes in CR?

  - Chair: continue to discuss the current condition for 3ms requirement can be applicable to FR2 HST UE or not by this meeting. If yes, the CR may not be needed.

- Agreement:

## Draft CRs for Intra-band CA

| R4-2312637 revised to R4-2314290 | Huawei, HiSilicon | CR on intra-frequency requirements enhancement in FR2 HST |  |
| --- | --- | --- | --- |
| R4-2312191 revised to R4-2314289 | Nokia, Nokia Shanghai Bell | CR on inter-frequency requirements enhancement in FR2 HST |  |
| R4-2313544 revised to R4-2314291 | Samsung | CR on inter-frequency measurement requirements in Idle mode for FR2 HST |  |
| R4-2312358 revised to R4-2314292 | Ericsson | CR on SCell activation delay for FR2 HST |  |

Ad-Hoc meeting:

- Discussion:

- Agreement:

# Topic #3: UL TX timing adjustment

## Background and Progress on Monday RRM session

[Moderator] The following agreement achieved on Monday online session:

- Agreement - If MAC-CE bit indicates “1” then for the TCI state switch UE may expect that there is a large timing difference between DL signals from two RRHs and UE [may or shall] apply one shot UL large timing adjustment. - If MAC-CE bit indicates “0” then for the TCI state switch UE may expect that there is no large timing difference between DL signals from two RRHs. - FFS if any clarifications on UE behavior are needed

- Tentative Agreement from Ad-Hoc:

  - Rel-17 UE capability of supporting one-shot timing adjustment shall be used as the prerequisite of the Rel-18 UE capability of supporting the newly introduced MAC-CE indication and new UE requirements.

- Tentative Agreement from Ad-Hoc:

  - If MAC-CE bit indicates “1” then for the TCI state switch UE may expect that there is a large timing difference between DL signals from two RRHs and UE [may or shall] apply one shot UL large timing adjustment.

    - There is no need to check the DL timing difference threshold, and UE shall apply the one-shot timing adjustment.

- Tentative Agreement from Ad-Hoc:

  - If MAC-CE bit indicates “0” then for the TCI state switch UE may expect that there is no large timing difference between DL signals from two RRHs or from the same RRH.

    - FFS if any clarifications on UE behavior are needed

## Draft Reply LS

[Moderator] Nokia has prepared the draft LS:  (Draft)R4-2314299_Reply_LS_HST_FR2_enh_v02_Samsung_Nokia.docx, here the content is copied here for discussion:

- Question 1 (RAN2): Is it correct RAN2 understanding that the RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH in 6.1.3.15 in TS 38.321? - Answer 1 (RAN4): Yes, RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH in 6.1.3.15 in TS 38.321. - Question 2 (RAN2): Whether the enhanced TCI state indication in 6.1.3.44 of TS 38.321 (i.e., the MAC CE indicates two target TCI states) or the unified TCI state indication in 6.1.3.47 (i.e., the MAC CE indicating a unified states for UL and DL) is intended to be supported for cross-RRH TCI state switch? - Answer 2 (RAN4): Enhanced TCI state indication in 6.1.3.44 (i.e., the MAC CE indicates two target TCI states) and the unified TCI state indication in 6.1.3.47 (i.e., the MAC CE indicating a unified states for UL and DL) are not intended to be supported for [cross-RRH TCI state switch] MAC-CE indication. Huawei: provide to interpret the MAC-CE indication and double check the term of [cross-RRH TCI state switch] before answering three detailed questions. Qualcomm: Echo Huawei’s comment on check the term of [cross-RRH TCI state switch] based on the expected UE behavior. - Question 3 (RAN2): What is the intended UE behavior (e.g. regarding timing advance handling) upon reception of the MAC CE with indication on the TCI state switch across RRHs? For example, Does UE behavior also depend on the existing RRC parameter highSpeedDeploymentTypeFR2? Is it possible to update timing advance upon reception of the MAC CE with indication on the TCI state switch across RRHs and if not should UE stop uplink transmissions? - Answer 3 (RAN4): In FR2 HST scenario, when highSpeedMeasFlagFR2-r17 is configured and PC 6 UE supports new [interRRH-TCISwitch-EnhMACCE-HSTFR2-r18] capability, the following HST PC 6 UE behavior should be expected upon the reception of MAC CE with indication on the TCI state switch across RRHs - MAC-CE indicates “1”: For the TCI state switch UE may expect that there is a large timing difference between DL signals from two RRHs. - Rel-17 TCI state switching delay requirements in HST FR2 scenarios from TS 38.133, Clause 8.10.3A are followed without changes. - If NW configures (highSpeedLargeOneStepUL-TimingFR2-r17 is enabled) and UE supports ([largeOneStepUL-timingFR2-r17] UE capability) large one-shot UL timing adjustment - Large one-shot UL timing adjustment shall be performed at TCI state switch - [There is no need to check the DL timing difference threshold and UE shall adjust UL Tx timing by default based on DL propagation delay difference.] - [Option 1: UE does not stop uplink transmission after receiving the cross-RRH TCI state switch indication MAC CE, since UE would apply one shot large timing adjustment after receiving the 1 bit cross-RRH MAC-CE indication, and perform normal UL transmission] - [If one-shot UL timing adjustment is not supported by UE or not enabled by the NW: - RACH-based procedure shall be initiated to acquire UL TX timing (TA) for the new target TCI state.] - [Option 2: UE shall not transmit any signals in UL with old TCI/wrong UL TX timing before the value of UL timing is adjusted with one-shot adjustment or the new TA value is signalled from the NW when RACH is initiated - When UE MAC timeAlignmentTimer may be stopped or suspended after indicated inter-RRH TCI state switch, no UL transitions are expected.] - MAC-CE indicates “0”: for the TCI state switch UE may expect that there is no large timing difference between DL signals from two RRHs. - [There is no need in additional time for DL synchronization at TCI state switch, i.e., Rel-15 requirements from TS 38.133, Clause 8.10.3 are followed.] - Gradual timing adjustment requirements in Clause 7.1.2.1 apply to the first UL transmission after TCI state switch, i.e., large one-shot UL timing adjustment is not needed. UE behavior does not depend on the existing RRC parameter highSpeedDeploymentTypeFR2. No new PC6 UE behaviour is expected in comparison to Rel-17 when Rel-18 MAC CE with indication on the TCI state switch across RRHs is not signalled or supported by the UE. [Additionally, RAN4 sees a need to defined new optional PC6 UE capability to support enhanced MAC CEs with inter-RRH indications in HST FR2 deployments, i.e., [interRRH-TCISwitch-EnhMACCE-HSTFR2-r18] capability].

## Draft CRs for UL TX timing adjustment

| R4-2312874 revised to R4-2314293 | Huawei, HiSilicon | DraftCR on UE timing requirements for R18 FR2 HST |  |
| --- | --- | --- | --- |
| R4-2312193 revised to R4-2314292 | Nokia, Nokia Shanghai Bell | Draft CR to 38.133 on HST FR2 Enhanced TCI State Switch |  |

Ad-Hoc meeting:

- Discussion:

- Agreement:

# Topic #4: Tunnel deployment

## Draft CRs for UL TX timing adjustment

| R4-2312197 revised to R4-2314294 | Nokia, Nokia Shanghai Bell | Draft CR to TR 38.854 on HST FR2 Enhanced Tunnel Deployment |  |
| --- | --- | --- | --- |

Ad-Hoc meeting:

- Discussion:

- Agreement: