3GPP TSG-RAN WG4 Meeting # 108												          R4-2314344

# Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.35.4

Source:	Moderator (Huawei)

Title:	Ad-hoc minutes for NES

Document for:	Information

Introduction

Following issues are scheduled to be discussed in Ad-hoc session for [234] NES:

Topic #1: SSB-less SCell operation (~ 35 min)

- Issue 1-1-1:

- Issue 1-2-1:

- Issue 1-2-2:

- Issue 1-2-5:

Topic #2: RRM impacts of other objectives (~ 10 min)

- Issue 2-1-1/2-1-2/2-1-3/2-1-4

- Issue 2-2

- Issue 2-3

Topic #1: SSB-less SCell operation

Main technical topic overview. The structure can be done based on sub-agenda basis.

Sub-topic 1-1 Scenarios

Background:

Following agreements were reached in RAN4#107 R4-2310086

## Sub-topic 1-1: Scenarios Issue 1-1-1/2/3: Scenario 1 / 2 / 2a - Agreements - Continue RAN4 work on the following SSB-less SCell scenarios - Scenario 1: SCell without SSB transmission and with TRS transmission - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective. - Deprioritize RAN4 work on the following SSB-less SCell scenario - Scenario 2: SCell without SSB transmission and without TRS transmission - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

Issue 1-1-1: Scenarios

- Proposals

  - Option 1: Only consider scenario 1 (Apple, Huawei, Ericsson?)

    - Option 1a: RAN4 to only consider scenario 1 for SSB-less SCell operation for FR1 inter-band collocated CA in R18. (Apple, CATT)

    - Option 1b: Scenario 2a is not considered in R18 NES (Huawei)

    - Option 1c: RAN4 to study and specify SSB less SCell feasibility conditions and requirements for DL only SSB SCell and DL+UL SSB less SCell (Ericsson)

    - Option 1d: RAN4 to prioritize Scenario 1 before scenarios 2a gets clarified. (Nokia)

  - Option 2: (ZTE)

By shutting down the NW DL transmitter at the SSB-less SCell, Scenario 2a is beneficial to achieve the significant NW power saving effect.

From the perspective of NW power saving effect, Scenario 2a is better than Scenario 2, and Scenario 2 is better than Scenario 1

Scenario 2a has the following impact:

•	For RRM requirements, the SCell activation latency is 3 ms for this scenario;

•	No RAN1 impact;

•	For RRC signalling, the NW indicates UE which active serving cell is the reference cell to borrow the coarse time synchronization for the SSB-less SCell when the side condition met.

  - Option 3: (Intel)

    - No activation delay requirement is specified for scenario 2a

    - Existing interruption requirements still apply to UE configured with scenario 2a SCell activations.

- Recommended WF

  - Scenario 2a is not considered in Rel-18 NES.

Discussion:

ZTE: If no LS from other WG, no decision in RAN4. Can up to RP.

LGE: Support option 1.

Apple: New BC for 2a. BC needs clarification before RAN4 work.

QC: NO RRM impacts of 2a.

ZTE: 2a is easier than others.

Huawei: 2a beyond scope.

Intel: RRM requirements is for DL scell activation.

CMCC:  Discuss details.

Ericsson: Discuss details.

Vivo: No consensus in RP to support 2a. No need to discuss it in this WI. Can discuss in future release.

Nokia: Discuss scenario 1 in this meeting.

Sub-topic 1-2 Feasibility conditions

Background:

For RTD conditions, following agreements were reached in RAN4#107 R4-2310086

Issue 1-2-1: RTD conditions for scenario 1 - Agreements - Further consider the following cases for requirements definition - Set 1: RTD ≤ 3us + X (X is FFS) - Set 2: 260ns < RTD < min(CP, 3us) - note: the CP corresponding to the largest SCS across CCs - Set 3: RTD ≤ 260ns - FFS whether all subsets are feasible from UE implementation perspective

Open issues and candidate options before meeting:

Issue 1-2-1: RTD conditions for scenario 1

- Proposals for Set1/2/3

  - Set 1: RTD ≤ 3us + X (X is FFS) (support: QC, Intel, Ericsson, Nokia, Samsung, LGE)

    - Support:

      - Option 1-1: RTD condition shall not exceed 3us (X=0) for inter-band CA SSBless Scell co-located deployment for scenario 1. (QC)

      - Option 1-2: In order to apply the 3ms SCell activation delay on the SSB-less target cell, it is required as one of the prerequisite conditions that the RTD between the target inter-band SCell (without SSB) and the active serving cell with SSB is within ±3us. (Intel)

      - Option 1-3:  (Ericsson)

        - SSB less SCell activation would work when RTD is less than 4.7µs for Scenario 1 when SCS of reference cell is 15kHz. For existing TAE of 3µs, SSB less SCell activation would work.

        - SSB less SCell activation would work when RTD is less than 2.35µs for Scenario 1 when SCS of reference cell is 30kHz.

        - RAN4 to study potential solutions for UE to acquire timing when the TAE is 3µs and reference cells SSB is 30kHz.

          - One potential solution is tightening TAE to 2.3µs

          - Other solutions are FFS

      - Option 1-4: the feasibility of SSB-less operation when RTD exceeds CP needs to be checked by RAN1. (Nokia)

    - Not support:

      - Option 1-5: If RTD between SSB-less SCell and inter-band active serving cell is larger than CP length, certain interference cannot be avoided, thus timing information from inter-band active serving cell may not be reused for SSB-less SCell under this condition. Therefore, set 1 may not be feasible for RTD conditions for SSB-less SCell operation for FR1 inter-band CA. (CTC)

    - Others:

      - Option 1-6: If the RTD requirement decided to set 1, the TA value for SSBless SCell only separate from the reference cell could be necessary. (LGE)

  - Set 2: 260ns < RTD < min (CP, 3us) (support: Apple, Nokia, CTC, CMCC, Huawei, Samsung, Ericsson, LGE, CATT)

    - Option 2-1: 260ns < RTD < min (CP, 3us), and one additional TRS/ATRS for fine time tracking is needed. (Huawei)

    - Option 2-2: The UE is able to track the timing on SSB-less SCell with assistance of TRS and without performance degradation, when RTD is between CP/2 and CP

  - Set 3: RTD ≤ 260ns (support: Apple, CTC, MTK, CMCC, Huawei, ZTE,vivo, Samsung)

    - Option 3-1: RTD ≤ 260ns, no additional time for fine time tracking. (Huawei)

    - Option 3-2: Further RAN4 to agree that SET3 (RTD ≤ 260ns) is not feasible from NW point of view. (Ericsson)

    - Option 3-3: The UE is able to reuse PCell timing for SSB-less SCell without performance degradation when RTD is within CP/2. (Nokia)

- Proposals for other issues for RTD:

  - Option 1: (Nokia)

    - RAN4 to define one or more sets of RTD conditions in RRM to ensure SSB-less operation.

    - RAN4 to discuss the SSB-less operation (if feasible) for different sets of RTD conditions respectively

    - The UE shall indicate the RTD or the set of RTD condition where applicable, so that network is aligned with UE on the expected behavior as well as the requirements it is supposed to meet.

  - Option 2: (Samsung)

    - the definition of different RTD sets could depend partly on the frequency separation range between the bands of SSB-less SCell and the reference cell.

- Recommended WF

  - Moderator:

    - Simulation results provided by Nokia can be found in R4-2311575.

    - Based on the preproposal from companies, the status of supporting of Set 1/2/3 are summarized in following Table (Y: support; N: show concerns in proposals):

| Companies | Set 1 | Set 2 | Set 3 |
| --- | --- | --- | --- |
| QC | Y (X = 0) |  |  |
| Apple |  | Y | Y |
| Nokia | N | Y | Y |
| CTC | N | Y | Y |
| MTK |  |  | Y |
| CMCC |  | Y | Y |
| Intel | Y (X = 0) |  |  |
| Huawei |  | Y | Y |
| ZTE |  |  | Y |
| Vivo |  |  | Y |
| Ericsson | Y | Y | N |
| Samsung | Y | Y | Y |
| LGE | Y | Y |  |
| CATT |  | Y |  |

Set 1: 5 companies support Set 1; 2 companies show concerns on Set 1;

Set 2: 9 companies support Set 2:

Set 3: 8 companies support Set 3; 1 company show concerns on Set 3.

Moderator recommendation:

- Inter-band SSB less operation is feasible for RTD conditions Set 2 and Set 3, and the requirements can be defined separately.

- UE shall indicate the supported set(s) of RTD condition in UE capability.

Discussion:

ZTE: Set 3. 260 ns can be support by NW.

Intel: Have concerns on set 3. Support Set 1.

Nokia: Do not change TAE requirements. Open to discuss different RTD in RRM requirements. Can not agree only set 3.

Ericsson: UE can support separate window. RTD less than or equal to CP.

QC: can compromise to RTD less than or equal to CP

CMCC: Purely from UE and not need to discuss TAE. Ok with set 2 + set 3. UE by default can support smaller RTD.

Vivo: smaller TAE can help simplify UE implementation. Multiple window may not help. Set 3.

Samsung: Support different requirements for different set. Support set 2 and set 3.

Huawei: Set2 and Set 3 as package.

CATT: Prefer to have one set combining set 2 and set 3.

Apple: support one of Set 2 and set 3.

ZTE: EVM should be considered even it is RF requirements. It is not a minor feature.

CTC: Support Set 3. Fine with different.

Apple: Depends on frequency separation.

Nokia: We only consider the feasible conditions.

Apple: Legacy we have SSB even with large RTD.

Agreements:

FFS set 2 and set 3.

- FFS set 2 for 15 kHz

Issue 1-2-2: Power difference conditions for scenario 1

For reception power difference conditions, following agreements were reached in RAN4#107 R4-2310086

Issue 1-2-4: Power difference conditions for scenario 1 FFS: - Proposal 1: The difference of the reception power with the FR1 inter-band active serving cell is within 6dB (Apple, CATT, Intel, MTK, CTC, CMCC, Vivo, Huawei, ZTE) - Proposal 1a: For a UE using single RF chain, the difference of the reception power with the FR1 inter-band active serving cell is within 6dB (Ericsson, CMCC) - Proposal 1b: Reception power difference between the reference cell with applying transmit power compensation and SSBless SCell is within 6dB. (QC) - Proposal 2: For a UE using dual RF chains, the maximum power difference UE can handle is FFS, and RAN4 to study whether UE can use TRS transmission in scenario 1 for computing AGC (Ericsson) - Proposal 2a: For a UE using dual RF chains, the maximum power difference UE can larger than 6dB, the SCell activation delay should be further studied. (CMCC) - Proposal 3: The difference of reception power is expected to vary between 6dB and 25dB. (Nokia)

- Proposals

  - Option 1: The reception power difference shall be within 6dB. (Apple, MTK, CMCC, Intel, SS, Huawei, Vivo, Ericsson, ZTE, LGE, CATT)

    - Option 1a: For a UE using single RF chain, the difference of reception power with the reference cell selected from FR1 inter-band active serving cells is within 6dB (SS, Ericsson)

    - Option 1b: (Vivo)

      - The difference of Tx power with the FR1 inter-band active serving cell is within 6dB, and the same UE RF chain is used for the SSB-less SCell

  - Option 2: The reception power difference can be larger than 6dB. (QC, CMCC, SS, Huawei, Ericsson, Nokia, vivo)

    - Option 2a: for a UE using dual RF chains, the difference of reception power with the reference cell selected from FR1 inter-band active serving cells is within 6dB+Y, the value of Y is TBD. (SS)

    - Option 2b: If UE can support TRS-based AGC adjustment, there is no power difference limitation. (Huawei)

    - Option 2c: (QC, LGE)

      - NW provides explicit information about transmit power offset between reference cell and SSBless SCell, where transmit power offset = transmit power of RS on reference Cell – Transmit power of RS on SSBless SCell.

      - If NW provide transmit power offset information, RAN4 does not need to define side condition of AGC for SSBless Scell operation.

    - Option 2d: (Ericsson)

      - For a UE using dual RF chains, the maximum power difference UE can handle is 25dB.

      - NW can indicate the power difference between the reference cell and target cell to UE to compensate the AGC gain

      - For a UE using dual RF chains, RAN4 to study whether UE can use TRS transmission in scenario 1 for computing AGC.

    - Option 2e: (Nokia)

      - RAN4 to discuss the feasibility and corresponding UE behavior when reception power difference is within 6dB and when reception power difference is larger than 6dB respectively.

    - Option 2f: (vivo)

      - The UE is allowed at least one CSI-RS based measurement for AGC adjustment on the SCell before or during the activation of the SCell. Such CSI-RS based measurement can be A-TRS, TRS, CSI-RS for mobility or CSI-RS for pathloss measurements.

- Recommended WF

  - Moderator: Based on the preproposal from companies, the status of supporting of Set 1/2/3 are summarized in following Table (Y: support; N: show concerns in proposals):

| Companies | Set 1 within 6 dB | Set 2 larger than 6dB |
| --- | --- | --- |
| QC |  | Y |
| Apple | Y |  |
| Nokia |  | Y |
| CTC | Y |  |
| MTK | Y |  |
| CMCC | Y | Y |
| Samsung | Y | Y |
| Intel | Y |  |
| Huawei | Y | Y |
| ZTE | Y |  |
| Vivo | Y | Y |
| Ericsson | Y | Y |
| LGE | Y |  |
| CATT | Y |  |

  - Option 1: The reception power difference shall be within 6dB. (Apple, MTK, CMCC, Intel, SS, Huawei, Vivo, Ericsson, ZTE, LGE, CATT)

Set 1 within 6 dB: 12 companies support Set 1;

Set 2 larger than 6dB: 7 companies support Set 2:

Moderator recommendation:

- Inter-band SSB less operation is feasible for reception power difference conditions Set 1 and Set 2, and the requirements can be defined separately.

- UE shall indicate the supported set(s) of reception power difference condition in UE capability.

- FFS: NW provide transmit power offset information

Issue 1-2-3: Conditions for Scenarios 2a

- Proposals

  - Option 1: The side conditions for Scenario 2a includes: (ZTE)

    - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell -- Set 3;

  - Option 2:  RAN4 need not consider the power difference conditions for scenario 2a. (LGE, ZTE)

  - Option 3: TCI state indication can be skipped in Scenario 2a.(ZTE)

- Recommended WF

  - Discuss above proposals, which is subject to conclusion of issue 1-1-1.

Issue 1-2-4: QCL/TCI indication for Scenario 1.

For TCI/QCL assumptions, following agreements were reached in RAN4#107 R4-2310086

Issue 1-2-8: QCL/TCI indication FFS: - Proposal 1: RS of SCell without SSB is QCL-A with TRS of the SCell without SSB, and the TRS(s) of the SCell is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell. (Vivo, Apple, CATT, MTK, Huawei) - Proposal 1a: NW shall be able to provide TCI state information for the SSBless SCell based on measurement and report conducted from the reference Cell. (QC) - Proposal 2: TCI state indication may not be needed (CTC, Ericsson) - Proposal 3: Consider both with and without QCL configuration between the RSs from inter-band carriers. (Nokia)

- Proposals

  - Option 1: RS of SCell without SSB is QCL-A with TRS of the SCell without SSB, and the TRS(s) of the SCell is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell.  (QC, CATT, Apple, Nokia, Huawei, MTK, CTC, Samsung, Intel)

  - Option 2: Without QCL configuration between the RSs from inter-band carriers (Nokia, CMCC)

  - Option 2a: TCI state indication may not be needed to complete the SSB less SCell activation. (Ericsson)

- Recommended WF

  - Moderator: 9 companies support that option 1 with inter-band QCL. 2 companies support option 2 to consider without inter-band QCL, and 1 company propose that inter-band QCL may not needed for SCell activation.

  - Check whether option 1 can be taken as the baseline.

Issue 1-2-5: TRS related

- Periodic TRS/A-TRS

  - Option 1a: Periodic TRS is available for fine time/freq tracking purpose. (QC, Huawei, Apple)

  - Option 1b: A-TRS can be consider for SSB-less SCell activation (Apple, Huawei, Vivo)

  - Option 1c: RAN4 to clarify what type of TRS is intended for SSB-less SCell operation in Scenario 1. (Nokia)

- Whether TRS is needed for SSB-less SCell activation:

  - Option 1: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved). (Nokia)

    - Option 1a: RAN4 to study whether transmitting no TRS would work or not (Ericsson)

  - Option 2: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected). (Nokia)

- Recommended WF

  - For information:

TS 38.214 - 5.1.5 Antenna ports quasi co-location …… For an aperiodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates qcl-Type set to 'typeA' with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, qcl-Type set to 'typeD' with the same periodic CSI-RS resource. - …… - 5.1.6.1.1 CSI-RS for tracking A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

  - Companies please check following clarification:

    - TRS refers to periodic TRS, and A-TRS can be considered during SCell activation;

    - Whether TRS/A-TRS monitoring is needed in SCell activation delay can be discussed in detailed requirements based on the conclusion on RTD/power difference conditions.

Sub-topic 1-3 RRM requirements for SSB-less Scell activation

For SCell activation requirements, following agreements were reached in RAN4#107 R4-2310086

## Sub-topic 1-3: RRM requirements for SSB-less SCell activation Issue 1-3-1: Whether TRS is used in SCell activation FFS: - Proposal 1: RAN4 to study the SSB-less SCell operation for scenario 1 considering the following cases: (Nokia) - Case 1: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved). - Case 2: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected). - Proposal 2: For band combinations with small frequency domain separation, there is no need to consider requirements based on TRS transmission during SCell activation. (CTC) Issue 1-3-2: SCell activation requirements FFS: - Proposal 1: For FR1 co-located SSB-less inter-band CA, when the conditions defined above are met, the SCell activation delay can be further reduced to 3ms (CATT, CTC, Huawei) - Proposal 2: Use fast SCell Activation Delay Requirement for FR1(section 8.3.16 of TS38.133) as the baseline to specify the requirement for FR1 SSB-less SCell activation, e.g., A-TRS is used for fine timing tracking and fine AGC settling (Apple) - Proposal 2a: (Vivo) For an SSB-less SCell that configured with CSI-RS based L3 measurement, the known/unknown condition of the SCell is defined based on the L3 MR of the CSI-RS based L3 measurement If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is unknown - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms. Otherwise, the SSB-less SCell is unknown - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms - Proposal 3: There is at least one active serving cell as reference cell which can provide SSB for T/F sync, AGC for the SSB-less Scell. The reference cell shall keep being activated. (QC) - SSBless SCell for interband CA is considered as known SCell from Scell activation latency perspective. (e.g, UE can skip coarse AGC, measurement and report for TCI activation can be skipped) - Proposal 4: RAN4 to discuss the SCell activation delay for activating an inter-band co-located SSB-less SCell after the feasibility study is concluded. (Nokia) - Proposal 5: (CMCC) - In scenario 1, for the case of RTD> CP/2-Te, and/or reception power difference larger than 6dB, the SCell activation delay should be further studied, to at least capture fine sync delay and/or AGC adjustment delay, and considering the impaction due to lack of QCL-ed relationship. Issue 1-3-3: Other RRM requirements FFS: - Proposal 1: If SSB less SCell supports UL, RAN4 to study UL timing requirements of SSB less SCell. (Ericsson)

Issue 1-3-1: SCell activation requirements

- Proposals

  - Option 1: SSB less SCell activation delay can be 3 ms: (SS, Nokia, Huawei, CMCC, CATT, ZTE for scenario 1 and 2a, Intel, CTC)

    - Option 1a: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved). (Nokia)

    - Option 1b: For FR1 co-located SSB-less inter-band CA, when the conditions defined above are met, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms). (CATT, Intel, SS, CTC)

    - Option 1c: Under side condition of 260ns RTD (set 3) and 6dB power difference, Tactivation_time is 3 ms. (Huawei)

    - Option 1d: (CMCC):

      - RTD: ≤ 260ns

      - RPD: Within 6dB

      - TCI/QCL indication: QCL-Type-C could be assumed between TRS of SSB-less SCell and SSB of inter-band active serving cell

      - Frequency domain separation between CCs:  Small(negligible) frequency domain separation

      - SCell activation delay requirement: Tactivation_time=3ms

  - Option 2: SSB less SCell activation delay can be longer than 3 ms: (Nokia, CMCC, Huawei, Vivo, Apple)

    - Option 2a: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected). (Nokia)

    - Option 2b: (CMCC)

      - Case 2:

        - RTD: ≤ 260ns < RTD < min(CP, 3us)

        - RPD: Within 6dB

        - TCI/QCL indication: No QCL-ed relationship could be assumed between TRS of SSB-less SCell and SSB of inter-band active serving cell

        - Frequency domain separation between CCs:  large(general) frequency domain separation

        - SCell activation delay requirement: Tactivation_time larger than 3ms, at least cover fine time/frequency sync delay, the impact due to lack of QCL-ed relationship need further study

      - Case 3:

        - RTD: ≤ 260ns < RTD < min(CP, 3us)

        - RPD: Larger than 6dB

        - TCI/QCL indication: No QCL-ed relationship could be assumed between TRS of SSB-less SCell and SSB of inter-band active serving cell

        - Frequency domain separation between CCs:  General frequency domain separation

        - SCell activation delay requirement: Tactivation_time larger than 3ms, to at least cover AGC adjust delay and fine time/frequency sync delay, the impact due to lack of QCL-ed relationship need further study

    - Option 2c: Under side condition of less than CP RTD (set 2) and 6dB power difference, Tactivation_time is Tfirst_TRS +3 ms or Tfirst_ATRS+3 ms (Huawei)

    - Option 2d: (Vivo)

The SSB-less SCell activation requirements are only applicable if the Ês/Iot ≥ -2dB and the SSB of the timing reference serving cell meeting detectable condition in 9.2 of TS 38.133

-	If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is unknown

	The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms.

-	Otherwise, the SSB-less SCell is unknown

	The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

    - Option 2e: (Apple):

      - use fast SCell Activation Delay Requirement for FR1(section 8.3.16 of TS38.133) as the baseline to specify the requirement for FR1 SSB-less SCell activation, e.g., A-TRS is used for fine timing tracking and fine AGC settling.

- Recommended WF

  - Moderator: The detailed requirements highly depend on the conclusion on RTD/power difference conditions. Companies are invited to discussed detailed requirements based on conclusion of conditions.

  - Proposals with detailed SCell activation delay are summarized in following table.

| Companies | RTD conditions | Power difference conditions | TCI/QCL | Requirements |
| --- | --- | --- | --- | --- |
| CATT | Set 2 | Set 1 | Y | 3ms |
| CMCC | Set 3 | Set 1 | Y | 3ms |
|  | Set 2 | Set 1 | Y | larger than 3ms |
|  | Set 2 | Set 2 | N | larger than 3ms |
| Intel | Set 1 | Set 1 | Y | 3ms |
| Huawei | Set 3 | Set 1 | Y | 3ms |
|  | Set 2 | Set 1 | Y | Tfirst_TRS +3 ms or Tfirst_ATRS+3 ms |
| CTC | Set 3 | Set 1 | Y | 3ms |
|  | Set 2 | Set 1 | Y | 3ms |
| ZTE | Set 3 | Set 1 |  | 3ms |
| Vivo | Set 3 | Set 2 | Y | Known: Tfirst_TRS +5 ms or Tfirst_ATRS+5 ms Unknown: TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms |

Sub-topic 1-4 L1 measurement for SSB-less Scell

For L1 measurement, following agreements were reached in RAN4#107 R4-2310086

- Sub-topic 1-4: L1 measurement for SSB-less Scell Issue 1-4-1: Whether to have L1 measurement on SSB-less SCell FFS: - Proposal 1: L1 measurement on less SCell is not needed. (MTK, ZTE, CATT, Huawei) - Proposal 1a: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L1/L3 measurement can be skipped (CATT) - Proposal 1b: When CSI-RS resources for L1 measurement are not configured, as long as the RTD, power difference conditions are ensured between two carriers, the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1 measurements can be skipped (Huawei) - Proposal 2: CSI-RS based L1 measurement is needed on SSB-less SCell - Proposal 2a: RAN4 to assume no SSB but with CSI-RS resource for L1 measurement on the FR1 inter-band SSB-less SCell (Apple) - Proposal 2b: If the conditions are not met but CSI-RS based measurement is supported and configured, L1 measurement needs to be specified for SSB-less SCell operation. (CATT) - Proposal 2c: When CSI-RS resources for L1 measurement are configured, the legacy requirements for CSI-RS based L1 measurement can be reused for SSB-less SCell operation. (Huawei) - Proposal 3a: RAN4 needs to discuss the impact on the CSI-RS based L1 measurement requirements due to SSB-less SCell operation. (Nokia) - Proposal 3b: RAN4 can study whether and how to perform RLM/BFD/CBD on the SSBless SCell based on reference Cell measurement. (Vivo, QC) - Proposal 3c: For L1/L3 measurement, following two options can be discussed in parallel in RRM session. （LGE） - Option 1: No SSB but with CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell - Proposal 4 (from moderator): - When RTD, power difference conditions are ensured and CSI-RS based L1 measurement is not configured, L1 measurement on SSB-less SCell is not needed - When CSI-RS based L1 measurement is configured, legacy requirements can apply.

Issue 1-4-1: Whether to have L1 measurement on SSB-less SCell

- Proposals

  - Option 1: L1 measurement on less SCell is not needed. (MTK, CMCC, CATT, Huawei, CTC, ZTE)

    - Option 1a: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L1/L3 measurement can be skipped (CATT)

    - Option 1b: No L1/L3 measurement on the inter-band SSB-less SCell (MTK)

    - Option 1c: When RTD, power difference conditions are ensured and CSI-RS based L1 measurement is not configured, L1 measurement on SSB-less SCell is not needed. (CMCC)

    - Option 1d: No new measurement requirement is specified for L1 or L3 measurements for SSB-less SCell operations. (Intel)

    - Option 1e: When CSI-RS based L1 measurement isn’t configured and the conditions of RTD/power difference/QCL indication defined above are fulfilled, L1 measurement on SSB-less SCell can be skipped. (CTC)

    - Option 1f: Sharing the L1/L3 measurement results between the SSB-less SCell and reference cell (ZTE)

  - Option 2: CSI-RS based L1 measurement is needed on SSB-less SCell (Apple, CMCC, SS, Huawei, CTC)

    - Option 2a: RAN4 to assume no SSB but with CSI-RS resource for L1 measurement on the FR1 inter-band SSB-less SCell (Apple)

    - Option 2b: If the conditions are not met but CSI-RS based measurement is supported and configured, L1 measurement needs to be specified for SSB-less SCell operation. (CATT)

    - Option 2c: When CSI-RS resources for L1 measurement are configured, the legacy requirements for CSI-RS based L1 measurement can be reused for SSB-less SCell operation. (Huawei, CMCC, SS, CTC)

  - Option 3a: RAN4 needs to discuss the impact on the CSI-RS based L1/L3 measurement requirements due to SSB-less SCell operation.  (Nokia)

  - Option 3b: RAN4 can study whether and how to perform RLM/BFD/CBD on the SSBless SCell based on reference Cell measurement. (QC)

- Recommended WF

    - When RTD, power difference conditions are ensured and CSI-RS based L1 measurement is not configured, L1 measurement on SSB-less SCell is not needed

    - When CSI-RS based L1 measurement is configured, legacy requirements can apply.

Sub-topic 1-5 L3 measurement for SSB-less SCell

For L3 measurement, following agreements were reached in RAN4#107 R4-2310086

Issue 1-5-1: Whether to have L3 measurement on SSB-less SCell FFS: - Proposal 1: L3 measurement is no needed on SSB-less SCell (Appe, CATT, MTK, Huawei, ZTE, QC) - Proposal 1a: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L3 measurement can be skipped (CATT) - Proposal 1b: As long as the RTD, power difference conditions are ensured between two carriers, the SSB based L3 measurements on PCC can be leveraged for SSB-less SCC. (Huawei) - Proposal 2: If the conditions are not met but CSI-RS based measurement is supported and configured, L3 measurement needs to be specified for SSB-less SCell operation. (CATT) - Proposal 3a: RAN4 needs to discuss the impact on the CSI-RS based L3 measurement requirements due to SSB-less SCell operation. (Nokia) - Proposal 3b: For L3 measurement, following two options can be discussed in parallel in RRM session. （LGE） - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

Issue 1-5-1: L3 RRM requirements

- Proposals

  - Option 1: L3 measurement is no needed on SSB-less SCell (Appe, CATT, MTK, CMCC, Intel, SS, Huawei, ZTE, CTC)

    - Option 1a: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L3 measurement can be skipped (CATT).

    - Option 1b: The UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells (Nokia).

  - Option 2: If the conditions are not met but CSI-RS based measurement is supported and configured, L3 measurement needs to be specified for SSB-less SCell operation. (CATT)

    - Option 2a: CSI-RS based L3 measurement should be supported for the SSB-less SCell. The known/unknown condition of the SCell can be defined based on the L3 MR of the CSI-RS based L3 measurement. (Vivo)

  - Option 3: (Nokia)

    - RAN4 needs to discuss the impact on the CSI-RS based L1/L3 measurement requirements due to SSB-less SCell operation.

- Recommended WF

  - No RRM impacts on L3 measurement for inter-band SSB-less operation

Sub-topic 1-6 Others

Issue 1-6-1: Reference Cell

- Proposals

  - Option 1: First Consider PCell as reference cell. (CMCC)

  - Option 2:  For the SSB-less SCell to be activated, a reference cell should be selected from PCell and activating SCell(s) with SSB, and the reference cell selection criteria should be defined. (SS)

    - Option 2a: RAN4 not to restrict the reference cell for SSB-less SCell as PCell. Reference cell switching time delay should be considered if the reference cell is reconfigured. (LGE)

  - Option 3:  RAN4 should define applicability rule that other Scell should keep being activated when the Scell is used as the reference cell. (QC)

  - Option 4: Reference cell is not configured as a single cell but as a list of cells. How to select the reference cell at any given time instance from the list of reference cells can be FFS (Ericsson)

  - Option 5: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SSB-less SCell. (Apple)

- Recommended WF

  - Discuss above proposals.

Issue 1-6-2: Reference Cell indication

- Proposals

  - Option 1: (Apple)

    - regarding T/F information acquisition for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives:

      - Alt 1: expand the definition of QCLed-typeC to indicate the RTD between the SSB-less SCell and the inter-band active serving cell is within a small range, e.g., ±260ns. RAN4 needs to check with RAN1 for this solution.

      - Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SSB-less SCell. RAN4 needs to check with RAN2 for this solution.

    - regarding AGC settling for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives:

      - Alt 1: if T/F information is reused from an inter-band FR1 serving cell, the AGC info of same inter-band FR1 serving cell can also be used for the target SSB-less SCell.

      - Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as AGC source for the SSB-less SCell.

  - Option 2: (Ericsson)

    - RAN4 to agree to introduce NW flag to further indicate in which bands SSB less operation will be configured.

- Recommended WF

  - Discuss above proposals.

Issue 1-6-3: UE capability

- Proposals

  - Option 1: UE capability for SSBless SCell should be defined per band combo where the band combo indicate the bands of reference cell and the SSBless SCell: (QC, Huawei, Ericsson)

    - Option 1a: (Ericsson)

      - RAN4 to agree to introduce UE capability to indicate which bands UE can support SSB less operation

      - RAN4 to agree to introduce NW flag to further indicate in which bands SSB less operation will be configured

      - RAN4 can send LS to RAN2 to introduce the UE capability signalling.

  - Option 2: If FR1 inter-band SSB-less SCell operation is introduced in R18 NES, the existing capability scellWithoutSSB can be expanded to cover the R18 FR1 inter-band CA case (Apple)

    - Option 2a: New capabilities should be specified for intra-band non-contiguous CA and inter-band CA similar as scellWithoutSSB, or current definition of scellWithoutSSB should be extended to support SSB-less SCell for intra-band non-contiguous CA and inter-band CA scenarios. (SS)

- Recommended WF

  - Discuss above proposals.

Issue 1-6-4: Intra-band CA

- Proposals

  - Option 1: Activation requirements specified for inter-band SSB-less SCell also apply to intra-band non-contiguous SSB-less SCell activation (Intel, CMCC, SS)

- Recommended WF

  - Discuss above proposals.

Issue 1-6-5: Frequency separation

- Proposals

  - Option 1: RRM requirement discussion of SSB-less SCell operation for FR1 inter-band collocated CA can assume adjacent inter-band scenario as starting point (Apple)

    - Option 1a: RAN4 should study the feasible range of frequency separation between inter-band carriers to enable the SSB-less SCell operation. (CTC)

  - Option 2: As UE reports the SSB-less operation by per band combination, the frequency domain separation is not supposed to be as a side condition of requirements. (Huawei)

  - Option 3: Frequency domain separation or specific band combinations can be defined in RF session (CATT, CTC, ZTE, MTK, LGE)

  - Option 4: For scenario 1, instead of specifying frequency domain separation condition between 2 bands, RAN4 discuss the UE capability implying the same Antenna Set and/or RF chain Set is used in the inter-band CA combination. (Vivo)

  - Option 5: RAN4 should study the feasible range of frequency separation between inter-band carriers to enable the SSB-less SCell operation. (Nokia)

  - Option 6: the definition of different RTD sets could depend partly on the frequency separation range between the bands of SSB-less SCell and the reference cell.

- Recommended WF

  - Discuss above proposals.

Issue 1-6-6: TAE requirements

- Proposals

  - Option 1: The 3us TAE requirement shall be kept for the SSB-less discussion in inter-band co-located scenario.

- Recommended WF

  - Do not discuss TAE requirements in RRM session.

Issue 1-6-7: TAG

- Proposals

  - Option 1: No RRM requirements are specified if the SSB-less SCell is not configured in the same TAG as the reference serving cell (Vivo, CMCC)

- Recommended WF

  - Discuss above proposals.

Issue 1-6-8: PDCCH/PDSCH demodulation

- Proposals

  - Option 1: Do not discuss the PDSCH/PDCCH demodulation performance evaluation on RRM session (SS, ZTE)

- Recommended WF

  - Moderator: Nokia provides simulation results in R4-2311575, which can be considered when defining feasibility conditions.

Topic #2: RRM impacts of other objectives

Main technical topic overview. The structure can be done based on sub-agenda basis.

Sub-topic 2-1 Cell DTX/DRX(Obj#2)

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1-1: RRM impacts of Cell DTX/DRX – general

- Proposals

  - Option 1: If RAN1 concludes that the RS for RRM measurement (e.g RRM/RLM/BFD/BM) is not impacted by cell DTX, there is no RRM impacts for objective#2 (Huawei)

  - Option 2: to indicate RAN1 that: (Apple)

    - Based on network configuration, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX:

    - PRS

    - CSI-RS configured by measObjectNR (for RRM)

    - CSI-RS associated with RLM, BFD, CBD

    - Periodic/Semi-persistent CSI-RS for beam measurement

    - FFS: PDCCH in USS to be decided by RAN1/2

    - FFS: PDCCH in Type-3 CSS to be decided by RAN1/2

  - To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

  - Option 3:  For cell DTX/DRX, the impact to RRM requirements is not necessary. By gNB implementation, adjustment of CSI-RS measurement period by RRC is always possible, and therefore no impact to spec is needed. UE assumes CSI-RS occasions are still available if the CSI-RS periodicity/offset configuration indicates the availability of CSI-RS during cell DTX in-active period (Vivo)

- Recommended WF

  - Discuss above proposals

Issue 2-1-2: RRM impacts of Cell DTX/DRX – SSB based measurement

- Proposals

  - Option 1: RAN4 to reuse the legacy requirement for SSB-based measurement in Cell DTX/DRX (Ericsson)

  - Option 2: RAN4 to discuss if SSB-based measurement due to cell DTX needs to be improved for UE power saving, including SSB-based L3 measurement, SSB-based L1 measurement and RLM/BFD measurements. (Nokia)

- Recommended WF

  - As described in WID: “Note: No change for SSB transmission due to cell DTX/DRX.”

  - Agree on Option 1.

Issue 2-1-3: RRM impacts of Cell DTX/DRX – CSI-RS based measurement

- Proposals

  - Option 1: There is no impact to CSI-RS based measurement for legacy UE. (Ericsson)

  - Option 2: RAN4 to discuss if the UE is expected to perform CSI-RS based measurements during non-active periods of cell DTX, including the CSI-RS based L3/L1/RLM/BFD measurements. (Nokia)

  - Option 3: RAN4 to wait RAN1 and RAN2’s progress on Cell DTX/DRX for NES UE’s CSI-RS measurement

- Recommended WF

  - No Impacts expected on CSI-RS based measurement for legacy UE.

  - FFS for NES UE.

Issue 2-1-4: RRM impacts of Cell DTX/DRX – Others

- Proposals

  - Option 1: RAN4 to study whether the measurement procedures will be impacted due to Cell DTX/DRX activation/deactivation. (Ericsson)

  - Option 2: RAN4 is to discuss the interruption requirements due to changes in Cell DTX/DRX configurations. (Intel)

  - Option 3: RAN4 to discuss how to minimize the impact to SCell activation delay due to cell DTX/DRX. In the best case, the SCell activation delay should not be interrupted or extended by the non-active periods of cell DTX/DRX. (Nokia)

- Recommended WF

  - Discuss above proposals

Sub-topic 2-2 Spatial and power domain techniques(Obj#3)

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-2: RRM impacts of Spatial and power domain techniques

- Proposals

  - Option 1: If spatial and power domain adaptation is not considered for BM enhancement in RAN1, no RAN4 RRM impact is expected for objective#3. (Huawei)

- Recommended WF

  - Agree on option 1.

Sub-topic 2-3 Cell selection/reselection (Obj#4)

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-3: RRM impacts of Cell selection/reselection

Proposals

  - Option 1: Obj#4 (mechanism to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques) has no RRM impact for objective#4. (Huawei)

  - Option 2: RAN4 to wait RAN2’s progress on cell reselection solution to handle legacy UE reselects to NES cell. (Ericsson)

- Recommended WF

  - Agree on option 1.

Sub-topic 2-4 CHO (Obj#5)

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-4: RRM impacts of CHO

- Proposals

  - Option 1: RAN4 shall wait for more progress of RAN2 on CHO to investigate whether there is RRM impacts. (Huawei, Ericsson)

- Recommended WF

  - Discuss above proposal