3GPP TSG-RAN WG4 Meeting #108								               	              R4-2314350

Toulouse, France, August 21 – 25, 2023

Agenda item:			8.30.4

Source:	OPPO

Title:		WF on R18 NR SL RRM requirements (part 2)

Document for:	Approval

Topic #1: RRM requirements for NR SL CA

Issue 1-1: Interruption location of the interruption to WAN at SL carrier addition/release

- Agreements

  - For Mode 2 SL CA operation do not specify the exact interruptions location

    - FFS if constraints on interruptions during WAN paging or SIB reception shall be introduced

Issue 1-2: Interruption length of the interruption to WAN at SL carrier addition/release

- Agreements

  - Interruption length is

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |

Issue 1-3: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation

- Agreements

  - Reuse LTE TS 36.133 clause 13.10 RRM requirements of V2X (re)selection of Synchronization reference source for NR SL CA operation.

  - Note: the agreement can be revisited in case of additional RAN1/2 agreements on NR SL CA operation

Topic #2: RRM requirements for Co-channel coexistence for LTE SL and NR SL

Issue 2-1: Requirements for the dynamic resource sharing co-channel coexistence solution

- Agreements

  - Do not introduce new requirements for dynamic resource sharing co-channel coexistence solution

  - The existing requirements for SL-RSRP would apply for measurements for dynamic resource sharing co-channel coexistence solution

- Issue 2-2: Requirements for solution of FDM based resource pool partitioning

- Agreements

  - No need for RAN4 to consider RRM requirements for FDM based semi-static co-channel coexistence between NR SL and LTE SL.

CR split

- For RRM requirements for SL CA operation，

| Requirement | Clause | Note | volunteer company for draft CR |
| --- | --- | --- | --- |
| Interruptions to WAN due to Sidelink Carrier Aggregation | 12.7.x | - For Mode 2 SL CA operation do not specify the exact interruptions location. - FFS if constraints on interruptions during WAN paging or SIB reception shall be introduced - Interruption length is NR Slot length (ms); Interruption length (slots). 0; 1; 2. 1; 0.5; 3. 2; 0.25; 5. | Xiaomi |
| Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation | 12.y | The delay requirements for SL component carrier addition/release are not needed for Mode 2 operation and up to UE implementation. | Huawei |
| Selection / Reselection of Synchronization Reference Source for Sidelink Carrier Aggregation | 12.z | - Reuse LTE TS 36.133 clause 13.10 RRM requirements of V2X (re)selection of Synchronization reference source for NR SL CA operation. - Note: the agreement can be revisited in case of additional RAN1/2 agreements on NR SL CA operation. | Nokia |

- For RRM requirements for Co-channel coexistence for LTE SL and NR SL，

| Requirement | Clause | Note | volunteer company for draft CR |
| --- | --- | --- | --- |
| Applicability rule for solution of TDM-based semi-static resource pool partitioning and dynamic resource sharing | 12.5.1, 12.7.3, 12.9.1 | The existing RRM requirements can be applicable, at least including - interruptions to WAN in section 12.7.3, - scheduling availability in section 12.9.1,. - L1 SL-RSRP measurements in section 12.5. No need for RAN4 to consider RRM requirements for FDM based semi-static co-channel coexistence between NR SL and LTE SL. | [OPPO] |