3GPP TSG-RAN WG4 Meeting#108	R4-2314380

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR |  | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RLM and Link Recovery requirements for mobile IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce RLM and Link recovery procedures requirements for the mobile IAB-MT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements are added in Clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RLM and link recover requirements will not be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## 12.3A	Signalling Characteristics for mIAB MTs

### 12.3A.1	Radio Link Monitoring

#### 12.3A.1.1	Introduction

The UE requirements in sub-clause 8.1.1 [6] apply for mIAB-MT.

#### 12.3A.1.2	Requirements for SSB based radio link monitoring

##### 12.3.1.2.1	Introduction

The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell or PSCell, provided that the SSB configured for RLM is actually transmitted within mIAB-MT active DL BWP during the entire evaluation period specified in clause 12.3.1.2.2.

Table 12.3A.1.2.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 12.3A.1.2.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

##### 12.3A.1.2.2	Minimum requirement

mIAB-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period.

mIAB-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 12.3A.1.2.2-1 for FR1.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 12.3A.1.2.2-2 for FR2-1-1 with scaling factor N=8.

For FR1,

-	P = 1

For FR2-1-1,

-	P=1, when the RLM-RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when RLM-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

If the mIAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the mIAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the combination of RLM-RS resource and SMTC occasion configurations does not meet previous conditions.

Table 12.3A.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. |  |  |

Table 12.3A.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2-1-1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. |  |  |

##### 12.3A.12.3A	Measurement restrictions for SSB based RLM

The UE requirements in sub-clause 8.1.2.3 [6] apply for mIAB-MT.

#### 12.3A.1.3	Requirements for CSI-RS based radio link monitoring

##### 12.3A.1.3.1	Introduction

The requirements in this clause apply for each CSI-RS based RLM-RS resource configured for PCell or PSCell, provided that the CSI-RS configured for RLM is actually transmitted within mIAB-MT active DL BWP during the entire evaluation period specified in clause 12.3A.1.3.2. mIAB-MT is not expected to perform radio link monitoring measurements on the CSI-RS configured as RLM-RS if the CSI-RS is not in the active TCI state of any CORESET configured in the mIAB-MT active BWP.

Table 12.3A.1.3.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 4dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 12.3A.1.3.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

##### 12.3A.1.3.2	Minimum requirement

mIAB-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS [ms] period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS [ms] evaluation period.

mIAB-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS [ms] period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS [ms] evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 12.3A.1.3.2-1 for FR1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 12.3A.1.3.2-2 for FR2-1-1.

The requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For FR1,

-	P=1 .

For FR2-1-1,

-	P=1, when the RLM-RS resource is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = 3, when the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

If the mIAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the mIAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

NOTE:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

Longer evaluation period would be expected if the combination of RLM-RS resource and SMTC occasionconfigurations does not meet previous conditions.

The values of Mout and Min used in Table 12.3A.1.3.2-1 and Table 12.3A.1.3.2-2 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [8, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

Table 12.3A.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. |  |  |

Table 12.3A.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2-1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. |  |  |

##### 12.3A.1.3.3	Measurement restrictions for CSI-RS based RLM

The UE requirements in sub-clause 8.1.3.3 [6] apply for mIAB-MT.

#### 12.3A.1.4	Minimum requirement for mIAB-MT turning off the transmitter

The UE requirements in sub-clause 8.1.5 [6] apply for mIAB-MT.

#### 12.3A.1.5	Minimum requirement for L1 indication

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, layer 1 of the mIAB-MT shall send an out-of-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [15].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, layer 1 of the mIAB-MT shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [15].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 [10]. Two successive indications from layer 1 shall be separated by at least TIndication_interval.

TIndication_interval is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 12.3A.1.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 12.3A.1.3 if the RLM-RS resource is CSI-RS.

#### 12.3A.1.6	Scheduling availability of mIAB-MT during radio link monitoring

The UE requirements in sub-clause 8.1.7 [6] apply for mIAB-MT.

### 12.3A.2	Link Recovery Procedure

#### 12.3A.2.1	Introduction

The UE requirements in sub-clause 8.5.1 [6] apply for mIAB-MT.

#### 12.3A.2.2	Requirements for SSB based beam failure detection

##### 12.3A.2.2.1	Introduction

The UE requirements in sub-clause 8.5.2.1 [6] apply for mIAB-MT.

##### 12.3A.2.2.2	Minimum requirement

mIAB-MT shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period.

The value of TEvaluate_BFD_SSB is defined in Table 12.3A.2.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 12.3A.2.2.2-2 for FR2-1 with scaling factor N= 8.

For FR1,

-	P=1.

For FR2-1,

-	P=1, when the BFD-RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when the BFD-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

If the mIAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the mIAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the combination of BFD-RS resource and SMTC occasion does not meet pervious conditions.

Table 12.3A.2.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| Note: TSSB is the periodicity of SSB in the set . |  |

Table 12.3A.2.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2-1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N) TSSB) |
| Note: TSSB is the periodicity of SSB in the set . |  |

##### 12.3A.2.2.3	Measurement restriction for SSB based beam failure detection

The UE requirements in sub-clause 8.5.2.3 [6] apply for mIAB-MT.

#### 12.3A.2.3	Requirements for CSI-RS based beam failure detection

##### 12.3A.2.3.1	Introduction

The UE requirements in sub-clause 8.5.3.1 [6] apply for mIAB-MT.

##### 12.3A.2.3.2	Minimum requirement

mIAB-MT shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS ms period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 12.3A.2.3.2-1 for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in Table 12.3A.2.3.2-2 for FR2-1 with N=1.

The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For FR1,

-	P = 1.

For FR2-1,

-	P = 1, when the BFD-RS resource is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	Psharing factor = 3.

If the mIAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the mIAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

NOTE:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the BFD-RS resource and SMTC occasion configurations does not meet pervious conditions.

The values of MBFD used in Table 12.3A.2.3.2-1 and Table 12.3A.2.3.2-2 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3.

Table 12.3A.2.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P] TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

Table 12.3A.2.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2-1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P N] TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

##### 12.3A.2.3.3	Measurement restrictions for CSI-RS based beam failure detection

The UE requirements in sub-clause 8.5.3.3 [6] apply for mIAB-MT.

#### 12.3A.2.4	Minimum requirement for L1 indication

When the radio link quality on all the RS resources in set  is worse than Qout_LR, layer 1 of the mIAB-MT shall send a beam failure instance indication to the higher layers. A layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [15].

The beam failure instance evaluation for the RS resources in set  shall be performed as specified in clause 6 in TS 38.213 [10]. Two successive indications from layer 1 shall be separated by at least TIndication_interval_BFD.

TIndication_interval_BFD is max(2ms, TSSB-RS,M) ) or max(2ms, TCSI-RS,M), where TSSB-RS,M and TCSI-RS,M is the shortest periodicity of all RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB  in the set  or CSI-RS resource in the set .

#### 12.3A.2.5	Requirements for SSB based candidate beam detection

##### 12.3A.2.5.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within mIAB-MT active DL BWP during the entire evaluation period specified in clause 12.3A.2.5.2.

##### 12.3A.2.5.2	Minimum requirement

Upon request the mIAB-MT shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB ms period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table in B.2.4.1 [6] for a corresponding band.

The mIAB-MT shall monitor the configured SSB resources using the evaluation period in table 12.3A.2.5.2-1 and 12.3A.2.5.2-2 which is applicable to the non-DRX mode only.

The value of TEvaluate_CBD_SSB is defined in Table 12.3A.2.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in Table 12.3A.2.5.2-2 for FR2-1 with scaling factor N=8.

Where,

For FR1,,

-	P = 1.

For FR2-1,

-	P=1, when the candidate beam detection RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when candidate beam detection RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

If the mIAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the mIAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the combination of CBD-RS resource and SMTC occasion configurations does not meet pervious conditions.

Table 12.3A.2.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | Ceil(3 P) TSSB |
| Note: TSSB is the periodicity of SSB in the set . |  |

Table 12.3A.2.5.2-2: Evaluation period TEvaluate_CBD_SSB for FR2-1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | Ceil(3 P N) TSSB |
| Note: TSSB is the periodicity of SSB in the set . |  |

##### 12.3A.2.5.3	Measurement restriction for SSB based candidate beam detection

The UE requirements in sub-clause 8.5.5.3 [6] apply for mIAB-MT.

#### 12.3A.2.6	Requirements for CSI-RS based candidate beam detection

##### 12.3A.2.6.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set  configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within IAB MT active DL BWP during the entire evaluation period specified in clause 12.3A.2.6.2.

##### 12.3A.2.6.2	Minimum requirement

Upon request the mIAB-MT shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table in B.2.4.2 [6] for a corresponding band.

The mIAB-MT shall monitor the configured CSI-RS resources using the evaluation period in table 12.3A.2.6.2-1 and 12.3A.2.6.2-2 which is applicable to the non-DRX mode only.

The value of TEvaluate_CBD_CSI-RS is defined in Table 12.3A.2.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in Table 12.3A.2.6.2-2 for FR2-1 with scaling factor N=8.

For FR1,

-	P = 1.

For FR2-1,

-	P = 1, when candidate beam detection RS is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = 3, when candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

If the mIAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the mIAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 12.3A.2.6.3.

The values of MCBD used in Table 12.3A.2.6.2-1 and Table 12.3A.2.6.2-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Table 12.3A.2.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | Max(25, Ceil(MCBD P) TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

Table 12.3A.2.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2-1

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | Max(25, Ceil(MCBD P N) TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

##### 12.3A.2.6.3	Measurement restriction for CSI-RS based candidate beam detection

The UE requirements in sub-clause 8.5.6.3 [6] apply for mIAB-MT.

#### 12.3A.2.7	Scheduling availability of mIAB-MT during beam failure detection

The UE requirements in sub-clause 8.5.7 [6] apply for mIAB-MT.

#### 12.3A.2.8	Scheduling availability of mIAB-MT during candidate beam detection

The UE requirements in sub-clause 8.5.8 [6] apply for mIAB-MT.

----------------------END OF CHANGE----------------------------