3GPP TSG-RAN WG4 Meeting #108 								R4-2314429

Toulouse, FR, 21st – 25th Aug., 2023

Title:	WF on R17 NR NTN RRM maintenance

Source:	Xiaomi

Agenda Item:	5.4

Document for:	Approval

- Core part maintenance for NTN RRM

Issue 1-1: Ambiguity issue on UL scheduling restriction

- Agreement

  - Update specification to clarify that UL slots that are overlapping with gap or scheduling restriction (considering TA) are interrupted.

Issue 1-2: Dynamic SMTC shifting

- Agreement

  - In Rel-17, when multiple satellites are associated to the SMTC in IDLE/Inactive mode, it is up to UE implementation and no requirement shall be applied to link satellite with the SMTC adjustment.

- Performance part for NTN RRM

Issue 2-1: Location margin in the test cases where AT command approach is used to set the GNSS location.

- Agreement

  - 50m location margin is also added in the test cases where “Update UE Location Information” procedure is used to set the GNSS location.

Issue 2-2: Tmargin in the transmit timing test case

- Agreement

  - For NTN UE timing test cases, the additional timing error (Tmargin) due to TE is defined as 2Ts.

Issue 2-5: UE location aquisition in RRM test cases.

Companies are encouraged to provide contribution for the following proposals in RAN4#108bis meeting:

- Proposal 1: Revisit the UE location information and its format of definition. Move and specify the UE location information need, details and provision method in a common section. Detailed exceptions can be handled in individual test cases.

- Proposal 2: Revisit UE location information need with GNSS signals. Conclude and state clearly whether GNSS performance is required as part of the test scope.

- Proposal 3: For the introduction release of NTN (both closed WIs for NR (Rel17) and IoT (Rel-18)), define RRM requirements with provision of UE location and movement relevant information, only through AT commands, similarly as RF.

- Reference

[1]	R4-2314148,	Topic Summary for [108][203] NR_NTN_solutions