3GPP TSG-RAN WG4 Meeting # 108 	    R4-2314460

Toulouse, France, 21 – 25 August 2023

Agenda Item:	8.22.3.4

Source: 	Ericsson

Title:	Simulation assumptions for RedCap positioning with FH

Document for:	Approval

- Introduction

RAN4#108 agreed on simulation assumptions for RedCap UE positioning with FH. Agreed parameters for link level simulations are captured in Table 1 and Table 2. Companies are encouraged to provide simulation results based on the agreed performance metric in section 3 for RSTD and UE Rx-Tx measurements.

# Simulation assumptions

Table 1. General parameters.

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TDD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz, 60 kHz - FDD, TDD | - 40 GHz - 60 kHz, 120 kHz - TDD |
| Propagation conditions | AWGN, TDL-A (30 ns delay spread, 5Hz). |  |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | For 2Rx: Rel. 17 side conditions for Nsample = 4 and Nsample = 1. For 1Rx: side conditions agreed for no FH case. |  |
| Number of samples | 4 and 1 |  |
| Number of UE receive antennas | 1 (only FR1), 2 (FR1 and FR2) |  |
| UE measurement bandwidth in each hop | 15kHz = 10MHz 30kHz = 20MHz 60kHz (FR1) = 20MHz 60kHz (FR2) = 50MHz 120kHz = 100MHz |  |
| UE total measurement bandwidth after all hops | 15kHz = 50MHz (6 hops) 30kHz = 100MHz (6 hops) 60kHz (FR1) = 100MHz (6 hops) 60kHz (FR2) = 200MHz (5 hops) 120kHz = 400MHz (5 hops) |  |
| Number of overlapping PRBs between hops | 1 (other values are not precluded) |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb 2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 2 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: Companies are encouraged to check suitability of the parameters and the values. NOTE 3: Number of hops in UE total measurement bandwidth after all hops is calculated assuming number of overlapping PRBs between hops is 1. Other values for number of hops are not precluded. |  |  |

Table 2. PRS transmission configuration parameters.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sampling rate (Tc) |
|  | 15 | 272 | 6 | 32 |
|  | 30 | 272 | 6 | 16 |
|  | 60, FR1 | 132 | 6 | 16 |
|  | 60, FR2 | 272 | 6 | 8 |
|  | 120 | 272 | 6 | 4 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms, 320ms |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |
| NOTE 1: Repetition 6 is achieved by PRS in 6 consecutive slots with PRS-ResourceTimeGap=1. |  |  |  |  |

- Performance metric

At least the following performance characteristics are to be provided for PRS RSTD:

- RSTD error CDFs for the 2 neighbour cells

- 5%-ile and 95%-ile of the RSTD errors

At least the following performance characteristics are to be provided for UE Rx-Tx:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge).

- UE Rx-Tx error = abs (estimated TUE-RX – ideal TUE-RX) (based on perfect channel and UE location knowledge).