3GPP TSG-RAN WG4 Meeting #108		R4-2314483

Toulouse, France, 21 – 25 August 2023

Title:	LS on PRS/RRM measurement when eDRX cycle > 10.24s

Response to:	N/A

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	N/A

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments:	N/A

1. Overall Description:

RAN4#108 discussed issues related to PRS and RRM measurement for positioning needs when UE is configured with eDRX cycle > 10.24s and made the following agreements:

Agreements:

- Use the following assumptions on the start of PRS measurement to derive the requirements

      - When eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity, UE starts positioning measurement within PTW.

      - When eDRX cycle is longer than configured PRS measurement reporting periodicity, positioning measurement start is not limited to PTW.

- For the case when UE is configured to perform PRS measurements or to perform SRS transmission for positioning

- When eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity (if PRS measurement is configured) and configured SRS transmission periodicity (if SRS transmission for positioning is configured), RRM measurements for positioning needs are performed within PTW.

- Otherwise, RRM measurements for positioning needs are not limited to PTW.

- RRM measurements for positioning needs include at least RRM measurement for cell reselection.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN2 to take above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #108bis		09 – 13 October 2023				Xiamen, CN

TSG RAN WG4 Meeting #109			13 – 17 November 2023			Chicago, US