3GPP TSG-RAN WG4 Meeting #108                                R4-2314629

Toulouse, France, 21-25 Aug, 2023

Source:	Samsung, KDDI

Title:	TP for TR 38.898 HPUE DC_41A_n28A-n77A

Agenda item:			7.17.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_41_n28-n77 according to the request in [1].

All PC2 fallbacks have been specified.

2. Reference

[1] RP-231032 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.x	DC_41_n28-n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_41A_n28A-n77A14 | DC_41A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Since the maximum output power requirement for PC2 UL DC_41_n77 is already specified in the specification, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- IMD2/3/5 of dual UL may fall into Rx frequencies of band n28.

The MSD values for IMD2 reused the value for PC2 CA_n28-n41-n77 in the specifications.

New PC2 MSDs are defined in the following tables.

Table 5.x.3-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_41A_n28A-n77A | n28 | 743 | 5 | 25 | 798 | 36.8 | IMD21,X |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE X: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

<End of Text Proposal>