3GPP TSG-RAN WG4 Meeting #108	R4-2314634

Toulouse, France, 21st August – 25th August 2023

Source:	Ericsson, Bell Mobility

Title:	TP for TR 38.898 adding PC2 DC_2-66_n78

Agenda item:			7.17.2

Document for:	Approval

# Introduction

This paper is TP for PC2 DC_2-66_n78 for TR 38.898 [1].

# Text proposal

### ---Start of changes---

### 5.x	DC_2-66_n78

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-66A_n78A5,14 DC_2A-66A_n78(2A) 5,14 DC_2A-66A-66A_n78A5,14 DC_2A-66A-66A_n78(2A) 5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_2_n78 and PC2 DC_66_n78, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Coexistence studies show that UL 2_n78 give 4th order IMD into band 66 DL.

Coexistence studies show that UL 66_n78 give 2nd, 4th and 5th order IMD into band 2 DL.

MSD value for IMD5 band 2 is derived from CA_n25-n66-n77, and all other MSD values are derived from DC_2A-66A_n77A.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-66A_n78A | 2 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 21.1 | IMD4 |
|  | n78 | 3500 | 5 | 25 | 3500 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 37.6 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3720 | 5 | 25 | 3720 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | 19.8 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n78 | 3385 | 5 | 25 | 3385 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 13.2 | IMD5 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |

#### 5.x.4	∆TIB and ∆RIB values

There is no change to the values for PC3, so this section is omitted.

---End of changes---

# References

[1]	3GPP TR 38.898: “High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL) (Release 18)”