3GPP TSG-RAN WG4 Meeting #xxx	R4-2314658

<Meeting Location>, <Date> ‒ <Date>, 2023

Title: 	WF on other band combinations for NR_700800900_combo_enh

Agenda Item:	7.30	Enhancement for 700/800/900MHz band combinations

Source: 	Skyworks Solution, Inc. , Huawei, Apple, Qualcomm, Mediatek

Document for:	Approval

# CA configuration of CA_n5-n105

Inputs for this band combinations were provided by 4 companies in [1, 2, 3, 4]

## ΔTIB,c

Three contributing companies have proposed 0.5dB in both bands

<Way forward/Agreement>: following table is captured in 38.101-1

Table 1: ΔTIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n105 | 0.5 | 0.5 |

## ΔRIB,c

Three contributing companies have proposed 0dB in both bands while a third one proposed 0.3dB for both bands, as a compromise averaging is used.

<Way forward/Agreement>: following table is captured in 38.101-1

Table 2: ΔRIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n105 | 0.1 | 0.1 |

Cross band MSD for n5 due to n105 UL

Four companies provided values and test point with the same test point and just some different MSD values with one company values is based on 3 antenna architecture and proposes 0dB MSD. Average amongst 2 antennas is 1.7dB, with 0dB value added average is 1.3dB. given the small difference it is suggested to use the worst case based on two antenna approach.

<Way forward/Agreement>: following table is captured in 38.101-1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 1.7 | >ACLR2 |

## Cross band MSD for n105 due to n5 UL

Four companies provided values and test point with the same test point and just some different MSD values with one company values is based on 3 antenna architecture and proposes 0dB MSD. Average amongst 2 antennas is 3.3dB, with 0dB value added average is 2.5dB. given the small difference it is suggested to use the worst case based on two antenna approach.

<Way forward/Agreement>: following table is captured in 38.101-1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.3 | >ACLR2 |

# CA configuration of CA_n28-n105

Inputs for this band combinations were provided by 4 companies in [1, 5, 6, 7]

## ΔTIB,c

Three contributing companies have proposed same value for both bands at 1dB, 1.1dB or 0.7dB the lower one based on 3 antennas. Since delta T should enable 2 antennas in the future, 1dB can be adopted.

<Way forward/Agreement>: following table is captured in 38.101-1

Table 1: ΔTIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n28-n105 | 1.0 | 1.0 |

## ΔRIB,c

Three contributing companies have proposed same value for both bands at 0.8dB, 0.7dB or 0.2dB with the lower one based on 3 antennas. Since delta R should enable 2 antennas in the future, 0.7dB can be adopted.

<Way forward/Agreement>: following table is captured in 38.101-1

Table 2: ΔRIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n28-n105 | 0.7 | 0.7 |

Cross band MSD for n28 due to n105 UL

Four companies provided values and test point with the same test point and just some different MSD values with one company values is based on 3 antenna architecture. Average amongst 2 antennas is 6.9dB, with 2dB value added average is 5.7dB. given the small difference it is suggested to use the worst case based on two antenna approach.

<Way forward/Agreement>: following table is captured in 38.101-1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 6.9 | >ACLR2 |

## Cross band MSD for n105 due to n28 UL

Four companies provided values and test point with the same test point and just some different MSD values and given the small difference it is suggested to use averaging approach.

<Way forward/Agreement>: following table is captured in 38.101-1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 12.1 | ACLR2 |

# CA configuration of CA_n26-n28

Inputs for this band combinations were provided by 4 companies in [1, 8, 9, 10]

## ΔTIB,c

Three contributing companies have proposed same values.

<Way forward/Agreement>: following table is captured in 38.101-1

Table 1: ΔTIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n26-n28 | 0.7 | 0.7 |

## ΔRIB,c

Three contributing companies have proposed same values.

<Way forward/Agreement>: following table is captured in 38.101-1

Table 2: ΔRIB,c due to NR CA (two bands).

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n26-n28 | 0.2 | 0.2 |

Cross band MSD for n28 due to n26 UL for full band n28 and lower 30MHz n28

three companies provided values with the same test point for ACLR2 and four companies for ACLR1 cases with different MSD values and with one company providing input on filter performance.

For ACLR 1 the average amongst 4 companies the range is 31dB to 47.4dB and the average is 36.9dB and can be used. For ACLR 2 the average amongst 3 companies is 13.7dB and can be used.

Since it is not convenient to capture two test points only one can be captured in the TS. However, we can capture both test points and values in the TR to provide useful information for operators and regions that only uses the lower 30MHz of band n28

<Way forward/Agreement>: the inputs from companies on both test points is captured in 38.872

following table is captured in 38.101-1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | 36.9 | ACLR1 |

## Cross band MSD for n26 due to n28 UL

One company also proposed cross band MSD for band n26 from n28 UL at 1.7dB due to transmitter noise floor since other companies have found this issue to be negligible and the small value proposed, this MSD may be ignored

<Way forward/Agreement>: no MSD is captured for n26 due to n28 UL

Cross band MSD for n28 due to n26+n28 UL

One company provided values for dual UL cross band MSD for 20MHz n26 UL + n28 30MHz UL for the two n28 cases:

- Full band n28 at 13.5dB MSD

- Lower 30MHz of band n28 at 1.9 dB MSD

Since it is not convenient to capture two points only the worst one can be captured in the TS and second test point can be captured in the TR. And since it is a single company input, it is put in [] to allow further checks

<Way forward/Agreement>: following table is captured in 38.101

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 788 | 30 | [13.5] | ACLR1 from n26 and n28 |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

# References

[1] R4-2311753 MSD analysis for CA_n5-n105, CA_n28-n105, and CA_n26-n28 combinations, Apple

[2] R4-2311209 CA_n5-n105 requirements compared to CA_n5-n71, Skyworks Solutions Inc.

[3] R4-2312969 Discussion on RF requirements for CA_n5-n105, Huawei, HiSilicon

[4] R4-2313373 Considerations on CA_n5-n105, Qualcomm France

[5] R4-2311748 CA_n28-n105 cross band MSD based on measurements, Skyworks Solutions Inc.

[6] R4-2312970 Discussion on RF requirements for CA_n28-n105, Huawei, HiSilicon

[7] R4-2313375 Considerations on CA_n28-n105, Qualcomm France

[8] R4-2311116 CA_n26-n28 cross band MSD based on measurements, Skyworks Solutions Inc.

[9] R4-2312968 Discussion on RF requirements for CA_n26-n28, Huawei, HiSilicon

[10] R4-2313371 Considerations on CA_n26-n28, Qualcomm France

[11] R4-2312174 CR for TR 38.872 to introduce some potential RF impacts for for CA_n26-n28, Mediatek