3GPP TSG-RAN WG4 Meeting #108	R4-2314661

Toulouse, France, 21st Aug 2023 - 25th Aug 2023

Title: 	WF on FS_NR_sub1GHz_combo_enh

Agenda Item:	8.3.3

Source: 	Huawei

Document for:	Approval

CA_n5A-n28A-n105A

Issue 1-1-1: Two-antenna RF architecture

- Proposal: RAN4 to make an early decision on whether 2-antenna implementation shall be considered in the study item for CA_n5A-n28A-n105A.

  - Option 1: 2-antenna implementation shall be considered in the study item for CA_n5A-n28A-n105A.

  - Option 2: 2-antenna implementation shall NOT be considered in the study item for CA_n5A-n28A-n105A.

  - Option 3: Others.

Agreement:

- Further discuss the 2-antenna or 3-antena RF architectures only for specifying delta_Tib and delta_Rib requirements only.

Issue 1-1-3: Four-antenna RF architecture

- Proposal:

  - Option 1: it’s proposed not to use Four-antenna RF architecture specify the requirements.

  - Option 2: Others.

Agreement:

- Do not use Four-antenna RF architecture to specify the requirements

Issue 1-2-1: Delta Tib and Rib

Agreement:

- Capture both options in the TR and further discuss down-selection of options in WI phase.

Issue 1-3-1: MSD due to cross band isolation

- Proposals

  - Option 1: Cross-band analysis done for 2-band cases are sufficient, and no new cross-band analysis are needed for 3-band case

  - Option 2: the REFSENS impact (MSD) caused by cross-band interference between n5 UL and n28 DL may need to be revisited as the 3-band filter implementation could be different from that of the fallback combination CA_n5A-n28A.

Agreement:

- Agree on Option 1.

Issue 1-3-2: MSD in DL band n105 due to IMD3 from UL CA_n5-n28

  - Option 1:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n28 | 738.0 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | 25 | 649.5 | 30.3 | FDD | IMD3 |

  - Option 2:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 22.9 | FDD | IMD3 |

In addition, generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly.

Agreement:

- Capture the values of option 1 and option 2 in the TR.