3GPP TSG-RAN WG4 Meeting #108	R4-2314679

Toulouse, FR, 21 – 25 August, 2023

Title: 	WF on NR MIMO evo DL UL UERF

Agenda Item:	8.29.4

Source: 	Samsung

Document for:	Approval

<Topic 1: STxMP>

<Agreement> PCMAXf,c,k

-	LS is sent to RAN1 to inform that RAN4 will introduce PCMAXf,c,k for STxMP (See R4-2314698)

-	How to incorporate the  PCMAXf,c,k in to the spec will be discussed in RAN4#108-bis

<Way forward> MPRf,c,k/A-MPRf,c,k

-	MPRf,c,k/A-MPRf,c,k will be further discussed and determined in RAN4#108-bis from the following options

- Option 1: MAX[(MPRk , A-MPRk, MPRp, A-MPRp) ] +3dB in lower bound for beam k and p

- Option 2: MAX(X, MPRf,c,k, A- MPRf,c,k), X = 10*log10(number of UL TCI-states indicated for [STxMP]) dB in lower bound

- Option 3: Define ‘per-panel’ requirements of MPRf,c,k = MPRf,c + 3dB, and A-MPRf,c,k = A-MPRf,c + 3dB

- Option 4: Reuse MPRf,c and A-MPRf,c requirements, and add 3dB relaxation to lower bound

- Option 5: Do not extend the current MPR concept at least in this release.

- Option 6: Other proposals based on legacy MPR/A-MPR requirements are not precluded for RAN4#108-bis

<Agreement> P-MPRf,c,k-

-	RAN4 recognizes that the overlapping issue may have an impact on P-MPRf,c,k

-    In RAN4#108-bis, it will be discussed and determined how to handle the MPE

- Option 1: Consider UE reporting of P-MPR value for pairs of TCI states

- Option 2: Extend existing PHR mechanism to ‘per-panel’ P-MPRf,c,k

<Way forward> PUMAXf,c,k

-	Whether to introduce PUMAXf,c,k will be determined based on the discussion results of MPRf,c,k/A-MPRf,c,k

- If new approach is agreed to derive MPRf,c,k/A-MPRf,c,k, PUMAXf,c,k will be introduced based on legacy requirements

<Way forward> New signalling

-	Whether to introduce new signalling for overlapped beams indication depends on the discussion results of MPRf,c,k/A-MPRf,c,k in RAN4#108-bis

<Way forward> Testability

-	RAN4 will check the testability issue before PUMAXf,c,k is introduced, e.g., sending LS to RAN5 and/or other means