3GPP TSG-RAN4 Meeting #108	R4-2314681

# Toulouse, France, 21-25 August 2023

Title: 	WF on channel raster enhancements

Agenda Item:	8.5.4

Source: 	Ericsson

Document for:	Approval

### Approaches / Alternatives

One of the following approaches/alternatives to be chosen:

  - Approach 1: Specify a new channel raster

The new channel raster step size: 10 kHz

  - Approach 2: Do not specify new channel raster entries

    - Alternative 1

    - Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

    - The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth subject to UE capability; a subset of requirements applies for the UE-specific CHBW within a wider carrier

    - Alternative 3:

- For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

  - consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

  - is a maximum transmission BW configuration

  - but need not be centered on the channel raster.

- For UEs with the capability to support a UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

### Way forward

For the next meeting, companies are encouraged to comment on open issues and/or to detail the expected specification updates of their preferred approach.

### 4-	UE capability

Agreement:

A new UE capability will be specified to support the WI objectives

Open issues:

The new UE capability should be per band.

FFS from which release should the UE capability be applicable.

Whether the capability should (at least for some bands) be mandatory from Rel-18 onwards.

6- 		Reminder – previous agreements

#### 6.1: 	Agreement from RAN4#107 (R4-2310269)

For approach 1: Specify a new channel raster

The new channel raster should be specified:

- For both UE and gNB.

- For all FR1 bands below 3GHz that currently have 100 kHz channel raster.

#### 6.2: 	Agreement from RAN4#106-bis (R4-2306598)

The channel raster enhancement is also applicable to NTN. Following this, the changes made to TN will also be made to NTN.

## Annex (for information only): Some highlights from offline emails discussion.

- The RAN4#107 agreement is not verified by the conformance test specifications, it even does not claim that 3GPP's UE RF performance requirements are met.

- If the SIB1 carrierBandwidth is off the channel raster, legacy UEs need a UE specific channel BW on the channel raster.

Open questions:

Whether, and if so which, UE RF performance requirements should apply to carrier locations off the 100 kHz channel raster.

Whether the 3GPP specifications should continue to require that the carrierBandwidth in SIB1 is centered (for all UEs) on the 100 kHz channel raster or
whether 3GPP RF performance requirements apply also for a SIB1 carrierBandwidth off the 100 kHz channel raster to UEs which support the channel raster enhancement.

Whether in a configuration with multiple numerologies, legacy UEs only need that any one of the numerologies is centered on the 100 kHz channel raster.