3GPP TSG-RAN WG4 Meeting #108	 R4-2314686

Toulouse, France, 21 – 25 August 2023

Source:	Skyworks Solutions, Inc.

Title:	TP for TR 38.846 guidelines on simplification for 3DL2UL MSD

Agenda item:			8.1.3 Simplification of specification and reduction of test burden

Document for:		Approval

# 1. Introduction

This text proposal captures guidelines to ensure triple uplink configurations are no longer specified in the inter-band NR-CA or EN-DC 3DL/2UL MSD tables due to dual-uplink IMD interference. These guidelines are implemented in companion CRs [1,2,3,4] at this meeting.

# 2. Discussion

Since early Rel-15, the MSD test points due to dual uplink IMD interference for 3DL/2UL combinations have been specified with three simultaneous uplink transmissions. These MSD test points should be configured with a maximum total of two uplink transmissions, one in each of the UL band. These guidelines are implemented at this meeting for Rel-17 and Rel-18 in CRs [1,2,3,4].

# 3. References

[1]	R4-2311136 CR to TS 38.101-3 Rel-18 Removing triple uplink configurations, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[2]	R4-2311137 CR to TS 38.101-3 Rel-17 Removing triple uplink configurations, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[3]	R4-2311138 CR to TS 38.101-1 Rel-18 Removing triple uplink and corrections to 2UL CA exceptions, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[4]	R4-2311139 CR to TS 38.101-1 Rel-17 Removing triple uplink and corrections to 2UL CA exceptions, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

# 4. Text proposal

******************************* Start of TP to TR 38.486 *******************************

6.6 Guidelines on simplification for 3DL/2UL MSD due to 2UL IMD interference

For inter-band NR-CA or inter-band EN-DC 3DL/2UL MSD requirements due to dual UL IMD interference, a maximum of two uplink configurations shall be specified: one uplink configuration in each band of the 2UL inter-band CA or DC configuration.

The UL carrier frequency "UL Fc" and the UL RB allocation "UL LCRB" of the 3rd DL band which is affected by 2UL IMD interference shall be specified as "N/A".

These guidelines are illustrated in the examples of Table 6.6-1 and Table 6.6-2, including the special case of 3DL/2UL EN-DC combination with only 2 frequency bands where one of the constituent bands is configured for intra-band CA operation, for example DC_66A_(n)5AA captured in Table 6.6-2.

Table 6.6-1: Example of removing 3UL configurations for NR-CA 3DL/2UL MSD test points due to 2UL IMD interference.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1818.5 | 4.0 | FDD | IMD5 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2139 | 11.0 | FDD | IMD4 |
| CA_n1-n3-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2507.5 | 5.0 | TDD | IMD5 |

Table 6.6-2: Example of removing 3UL configurations for EN-DC 3DL/2UL MSD test points due to 2UL IMD interference.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1818.5 | 4.0 | IMD5 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2139 | 11.0 | IMD4 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_66A-(n)5AA | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
|  | 5 | N/A | 5 | N/A | 878 | 25 | IMD2 |
|  | n5 | 838 | 5 | 25 | 883 | 30 | IMD2 |

******************************* END of TP to TR 38.486 *******************************