3GPP TSG-RAN WG4 Meeting #108	R4-2314688

Toulouse, France, 21st - 25th Aug 2023

Title: 	WF on Delta_TIB and Delta_RIB

Agenda Item:	8.1.3

Source: 	ZTE, CHTTL

Document for:	Approval

- Introduction

In this meeting, a discussion paper [1] was proposed for clarification on delta TIB and RIB values for band combinations. It is observed that for some band combinations with the component bands having special bands such as SDL bands, SUL bands and immediately close bands, the delta TIB and RIB tables are currently also denoted as “-” which are not reasonable. In this paper, we’d like to propose the WF for the delta TIB and RIB values for some special bands.

2. WF on delta TIB and RIB values for some special bands

Issue 4-1A: delta TIB and RIB values for band combination

- Proposals

  - Option 1: It is suggested to differentiate the value of “-” and “N/A” for delta TIB and RIB in NR CA and EN-DC configurations.

    - For the band which has the value of zero, for the sake of simplicity the value in the delta TIB/RIB table will be denoted as “-”, as what it is now in the specification.

    - For the component bands which are SDL bands, SUL bands, immediately close bands, band combination with overlapping bands, or EN-DC combination with LTE LAA band, etc

        - If uplink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta T requirements for that constituted band of the band combination.

        - If downlink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta R requirements for that constituted band of the band combination.

  - Option 2: Others.

<Way forward >:

- In the current spec, it is observed that for the values of delta TIB and RIB,

  - Unless otherwise stated, the ΔTIB,c / ΔRIB,c is set to zero which is denoted as “-”.

  - For some band combinations which have special constituent band that uplink/downlink is not supported, the values of ΔTIB,c / ΔRIB,c are also denoted as “-”.

- How to differentiate the delta TIB and RIB values for the band combinations which have component bands of SDL bands, SUL bands, immediately close bands, band combination with overlapping bands, or EN-DC combination with LTE LAA band is FFS.

3. Reference

[1] R4-2312611, Clarification on delta TIB and RIB values for band combination, ZTE, CHTTL