3GPP TSG-RAN WG4 Meeting #108	R4-2314696

Toulouse, France, 21th – 25th August 2023

Title:	LS on signaling support for intra-band non-collocated NR-CA,EN-DC

Response to:

Release:	Rel-18 (NR-CA), Rel-18 (EN-DC)

Work Item:	NonCol_intraB_ENDC_NR_CA-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:

Name:	Yasuki Suzuki

Email Address:	ui-suzuki@kddi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

The UE capability “interBandMRDC-WithOverlapDL-Bands-r16” was introduced since Rel-16 for UE to indicate the support of non-collocated deployment for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping bands, while the UE capability to support non-collocated deployment for TDD-TDD intra-band NR-CA is being discussed in Rel-18.

1) [New UE capability]

During RAN4#107 meeting, RAN4 concluded a separate new capability signalling is needed, which is to indicate the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA in 7.10A in 38.101-1.

If the new UE capability is absent, the UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1.

This new UE optional capability is expected to be per BC, and applicable to only band n77 and n78.

2) [New BS signaling]

In addition, during RAN4#108 meeting, RAN4 conclude the necessity of a new IE from BS to configure UE to support different capability according to different BS scenarios (collocated or non-collocated scenario). And this new IE is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands and TDD-TDD intra-band NR-CA provided the UE has indicated [New UE capability].

- If [New BS signaling] is provided by BS, UE shall satisfy the requirements as described in interBandMRDC-WithOverlapDL-Bands-r16 if indicated for TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands or the requirements described in [New UE capability](as above 1) if indicated for TDD-TDD intra-band NR-CA.

- If [New BS signaling] is not provided by BS, UE shall satisfy the requirements as described in interBandMRDC-WithOverlapDL-Bands-r16 if absent for TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands or the requirements described in [New UE capability](as above 1) if absent for TDD-TDD intra-band NR-CA.

2	Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN2 to specify the above UE capability reporting for intra-band non-collocated NR-CA in Rel-18, and new BS signaling for intra-band non-collocated NR-CA in Rel-18 and inter-band non-collocated EN-DC with overlapping or partially overlapping bands in Rel-18.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #108bis              09th – 13th Oct 2023             	 Xiamen, China

TSG-RAN WG4 Meeting #109                  13th – 17th Nov 2023             	 Chicago, US