3GPP TSG-RAN WG4 Meeting #108	R4-2314705
Toulouse, FR, 21 – 25 Aug, 2023

Agenda item:			7.29.4

Source:	OPPO

Title:	WF on 3Tx and 4Rx requirements

Document for:	Approval

- Topic #2: WID revison for 3T/4R WI

- Sub-topic 2-1 New example band combination

Issue 2-1-1: 3T4R WI is a non-basket WI

WF: RAN4 clarify that the 3T4R WI is a non-basket WI

Issue 2-1-2: Add new example band combinations for PC1.5 3Tx

Agreement online:

- Agree on adding two new PC1.5 band combinations to the 3T4R WID to cover all the PC1.5 3Tx MSD types within RAN4 group.

- To add follow two urgent PC1.5 items in to Rel-18

| CA_n25A-n77A | PC3@n25 1Tx, PC1.5@n77 2Tx | Bell Mobility, TELUS |
| --- | --- | --- |
| CA_n5A-n77A | PC3@n5A 1Tx, PC1.5@n77 2Tx | AT&T, Verizon |

- Topic #3: 4Rx at low frequency band (<1GHz)

- Sub-topic 3-1 ΔRIB,4R

Issue 3-1-1: Value of ΔRIB,4R

Agreement online: Value of ΔRIB,4R is -2.4dB.

- Topic #4: Tx requirements for 3Tx inter-band UL CA/EN-DC

- Sub-topic 4-1 SAR

Issue 4-1-1: Which duty cycle IE (0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1) is applied to PC1.5 band

- WF

  - Both 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 can be applied to PC1.5 UE.

  - When both IEs are reported, the smaller one will be applied, i.e. min{0.5*maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1}

  - When only IE maxUplinkDutyCycle-PC2-FR1 is reported, 0.5*maxUplinkDutyCycle-PC2-FR1 is applied

  - When only IE maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported, maxUplinkDutyCycle-PC1dot5-MPE-FR1 is applied

  - When both IE are absent, 25% will be applied.

- Sub-topic 4-2 UL CA+TxD

Issue 4-2-1: Where to capture 3Tx with UL CA+TxD

- WF: new sections for inter-band UL CA + TxD are introduced

- Sub-topic 4-3 Configuration table update

Issue 4-3-1: How to indicate the 3Tx band combinations in the CA configuration tables

- WF: FFS whether below updates to the NOTEs of CA configuration tables is agreeable in next meeting.

- Sub-topic 4-4 new per FS TxD capability

Issue 4-4-1: Introduce new per FS TxD capability

- Proposals (R4-2312249)

  - Proposal 1: A new per FS TxD capability should be introduced for inter-band combination CA/DC with more than 2Tx.

    - The TxD capability introduced from Rel-16 is per band indicated, which is not applicable for the case for the 3Tx inter-band scenario.

  - Proposal 2: It is proposed to introduce the new per FS TxD capability from Rel-16.

  - Proposal 3: A LS should be sent to RAN2 to introduce a per FS TxD capability.

- WF: FFS whether new per FS TxD capability is needed for a band under a band combination that support TxD.

- Topic #5: Rx requirements for 3Tx inter-band UL CA/EN-DC

- Sub-topic 5-1 General principle for MSD

Issue 5-1-1: PC1.5 Tx power configuration for IMD

WF: 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

Issue 5-1-3: Reuse PC1.5 2Tx harmonic mixing and cross band leakage MSD for 3Tx

Agreement online:

- Confirm that the conclusion of “use same MSD for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class for harmonic mixing and cross-band leakage” is also applicable to 3Tx with PC1.5 band combination.

Issue 5-1-4: How to introduce MSD requirements for 3Tx

Agreement online:

For harmonic, harmonic mixing and cross band isolation

- Only add a text in subclause “7.3A.2.3 Reference sensitivity power level for Inter-band CA” to clarify that “the same requirements for REFSENS exceptions apply for UL aggressor configured with either 1Tx or 2Tx for UL MIMO or Tx diversity operation

- Sub-topic 5-2 Band combination status

Issue 5-2-1: Completed band combinations

- WF: Companies further check the status of below band combinations.

  - CA_n28A-n41A, CA_n8A-n78A, CA_n71A-n41A, CA_n41A-n77A, and DC_3A_n78A are complete for 3Tx WI

  - CA_n28A-n78A can be completed by reusing the DC_28A-n78A with 2Tx PC2 IMD5 MSD.

Issue 5-2-2: PC2 Band combinations needs MSD studies

- Proposals

  - The PC2 band combinations in the WID which needs MSD studies are as below (OPPO)

| Band comb | 3Tx MSD |
| --- | --- |
| CA_n26A-n78A | - FFS: 2Tx PC2 IMD4 MSD was not defined in spec - Keep as not defined: 2Tx PC2 harmonic mixing 4th order MSD |
| DC_40A_n78A | - FFS: 2Tx PC2 harmonic mixing 3rd order MSD was not defined in spec - FFS: 2Tx PC2 cross band leakage MSD was not defined in spec |

- WF: Encourage companies to check status.

Issue 5-2-3: PC1.5 Band combinations needs MSD studies

- Proposals

  - The PC1.5 band combinations in the WID which needs MSD studies are as below (OPPO)

| Band combination | MSD staus |
| --- | --- |
| CA_n71A-n41A | - Reuse: 2Tx PC3 MSD for n71 harmonic 4th order interfere n41 is defined in the spec, and can be reused to 3Tx - Keep as not defined: PC2 2Tx harmonic mixing 4th order MSD - FFS: 3T PC1.5 IMD4 |

- WF: Encourage companies to check status.

Issue 5-2-4: New PC1.5 Band combinations needs MSD studies

- Proposals

  - If below new band combinations are introduced for PC1.5, the below MSDs needs to be studied (OPPO)

| Band combination | MSD staus |
| --- | --- |
| CA_n25A -n77A (CA_n2A -n77A) | - n25 UL harmonic 2 with PC3 - FFS: 3T PC1.5 harmonic mixing 2nd order - FFS: 3T PC1.5 IMD2/4 - FFS: 3T PC1.5 cross band isolation |
| CA_n5A -n77A | - n5 UL harmonic 4/5 with PC3 - Keep as not defined: 3T PC1.5 harmonic mixing 4th order - FFS: 3T PC1.5 IMD4 |

- WF: Encourage companies to check status.

- Sub-topic 5-3 PC2 CA_n26A-n78A

Issue 5-3-1: CA_n26A-n78A PC2 IMD4 with 3Tx and 2Tx

- Proposals

  - Option 1: as below (OPPO)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 25 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |

- WF: FFS on the MSD

- Sub-topic 5-4 PC2 DC_40A_n78A

Issue 5-4-1: DC_40A_n78A PC2 harmonic mixing 3Tx and 2Tx

- Proposals

  - Option 1: as below (OPPO)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 40 | 16.2 | 13.2 | 11.4 | 10.2 |  |  |  |  |  |  |  |

- WF: FFS on the MSD

Issue 5-4-2: DC_40A_n78A PC2 cross-band leakage from n78 to b40 with 3Tx and 2Tx

- Proposals

  - Option 1: as below (OPPO)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 7 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 6.6 | >ACLR2 |

- WF: FFS on the MSD

- Sub-topic 5-5 PC1.5 CA_n41-n71

Issue 5-5-1: CA_n41-n71 PC1.5 IMD4

- Proposals

  - Option 1: as below (Huawei)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 28.7 | FDD | IMD4 |

  - Option 2: as below (OPPO)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71x | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 20.4 | FDD | IMD4 |
| NOTE x: The transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) of the band with 1Tx, and set at min (+27.8 dBm, PCMAX_L,f,c) of the band with 2Tx as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |

- WF: FFS on the MSD based on 23+27.8 power configuration in next meeting.

- Sub-topic 5-6 PC1.5 CA_n25A-n77A (CA_n2A-n77A)

Issue 5-6-1: CA_n25A-n77A (CA_n2A-n77A) PC1.5 IMD2/4

- Proposals

  - Option 1: The MSD of IMD2 is 35.4dB and IMD4 is 19.4dB for CA_n25A-n77A (CA_n2A-n77A) with 3Tx PC1.5. (OPPO)

- WF: FFS on the MSD based on 23+27.8 power configuration in next meeting.

- Sub-topic 5-7 PC1.5 CA_n5A-n77A

Issue 5-7-1: CA_n5A-n77A PC1.5 IMD4

- Proposals

  - Option 1: The MSD of IMD4 is 21.3dB for CA_n5A -n77A with 3Tx PC1.5. (OPPO)

- WF: FFS on the MSD based on 23+27.8 power configuration in next meeting.