# 3GPP TSG-RAN WG4 Meeting #108	R4-2314706

# Toulouse, France, August 21 – August 25 , 2023

Title: 	WF on UE RF requirements for NR NTN

Agenda Item:	8.28.6

Source: 	ZTE Corporation, Samsung

Document for:	Approval

Sub-topic 1  UE types and antenna assumption for NTN UE

Issue 1-1: Differentiate UE types from mobility perspective

Agreement:

- Define two set of requirement for fixed and mobile VSAT

  - Differentiate the electronic and mechanical steering, but the requirements of electronic and mechanical steering can be applicable to both fixed and mobile VSAT

  - Strive to minimize the number of different requirements.

Issue 1-2: Phase antenna array

FFS

Sub-topic 2  Noise figure for NTN UE

Issue 2-1:  Noise figure

Agreement:

- Define two sets of requirements

  - Two sets of noise figures

    - Set #1: 2.5dB for LEO and GEO

    - Set #2: 6dB only for LEO

  - [Apply to test based on UE declaration for G/T which is used to derive EIS requirements].

Sub-topic 3  RF requirements for NTN UE in Ka-band

Issue 3-1: Beam pointing/accuracy related requirements

Agreement:

For mobile VSAT

  - RAN4 to specify NTN UE VSAT pointing accuracy based on manufacturer declaration. RAN4 to reuse the explanation from EN 303 978:

“The applicant shall declare the peak pointing accuracy () and the associated statistical basis.

The antenna shall maintain the declared peak pointing accuracy (), such that the off-axis EIRP emission density pattern projected onto the geostationary arc remains within the mask specified in clause [Total EIRP density specification] when shifted by an angle of ±(°), taking into account the following factors:

the worst case operational environmental conditions;

maximum ESOMP dynamics; and

the range of latitude, longitude and altitude relative to the satellite orbital position.”

For fixed VSAT

  - Proposal 3: RAN4 should specify antenna accuracy requirement for fixed VSAT, based on ETSI EN 301 360.

Issue 3-2: on-axis and off-axis EIRP requirements

Agreement:

For mobile VSAT:

  - RAN4 to use ITU recommendation S.524 and/or EN 303 978 for radiated off-axis requirement.

  - RAN4 to consider the following (ITU recommendation S.524-9) as requirement:

“4	that earth stations operating in GSO networks in the FSS transmitting in the 27.5-30 GHz frequency band be designed in such a manner that at any angle, , which is 2 or more off the main lobe axis of the earth station antenna, the e.i.r.p. density in any direction within 3 of the GSO should not exceed the following values:

Angle off-axis	Maximum e.i.r.p. per 40 kHz

2	    	7	(19 – 25 log ) dB(W/40 kHz)

0	 7	    	9.2	–2 dB(W/40 kHz)

0	 9.2	    	48	(22 – 25 log ) dB(W/40 kHz)

48	    	180	–10 dB(W/40 kHz).

For any direction in the region outside 3° of the GSO, the above levels may be exceeded by no more than 3 dB.

”

Together with other relevant notes for instance:

“NOTE 10 – FSS earth stations operating in the 27.5-30 GHz band, which have lower elevation angles to the GSO will require higher e.i.r.p. levels relative to the same terminals at higher elevation angles to achieve the same power flux-densities (pfds) at the GSO due to the combined effect of increased distance and atmospheric absorption. Earth stations with low elevation angles may exceed the levels of recommends 4 by the following amount (see Annex 2):

Elevation angle to GSO ()	Increase in e.i.r.p. density (dB)

5	2.5

5°				30	3 – 0.1

For fixed VSAT:	FFS

For fixed and mobile VSAT: FCC regulation should be further studied.

It’s encouraged for the companies to contribute the regulations and requirements from other interested regions/countries.

Issue 3-3: on-axis and Off-axis spurious requirements

Agreement:

For mobile VSAT

  - RAN4 shall specify on-axis and off-axis spurious requirements

  - Reuse the Off-axis and On-axis spurious emission requirement in EN 303 978

For fixed VSAT:

  - FFS

Issue 3-4: Cessation of emissions and related self-monitoring

FFS

Issue 3-5: beam switching related requirement

FFS

Issue 3-6: Others

FFS