3GPP TSG-RAN WG4 Meeting #108															R4-2314710

Toulouse, France, 21 – 25th August 2023

Agenda item:			7.31

Source:	Apple Inc.

Title:	WF on NTN L-/S-band

Document for:	Information

# Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band.

This document presents a way forward and agreements made during the RAN#108 meeting.

# Topic #1: System parameters

Agreements for the system parameters:

-	15MHz channel bandwidths is mandatory

-	Introduce 5+10MHz, 5+15MHz, and 10+15MHz UL/DL asymmetric channel combinations for the new NTN band under the existing RAN4 principles

-	Variable Tx-RX separation distance 862 – 885 MHz is defined

-	For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster of SS Block pattern Case C also is defined

# Topic #2: UE RF requirements

Agreements for the UE RF requirements:

-	ETSI out-of-band emission requirements, as presented in the table below, are applied to band n254.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

-	ETSI in-band emission requirements, as presented in the table below, are applied to band n254.

Additional in-band emission requirements for 1610-1618.25MHz.

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE: Spectrum emissions from -2 to -26dBm are linearly interpolated in dBm versus frequency offset. |  |  |

Additional in-band emission requirements for 1618.25-1626.5MHz.

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE: Spectrum emissions from -2 to -26dBm are linearly interpolated in dBm versus frequency offset. |  |  |

-	NS_03N value is introduced for the FCC regulations.

-	NS_04N and NS_05N for ETSI regulations are FFS.

-	For REFSENS use band n53 REFSENS with 0.5dB insertion loss (i.e. same as the NTN band n256 REFSENS).

-	Adopt existing NTN in-band blocking requirements

# Topic #3: RRM requirements

No new agreements.

# Topic #4: SAN

Agreements from the UE RF system parameters are also applied to SAN RF.

- Agreements from RAN4#107 meeting

### Topic #2: System parameters

Agreements for the system parameters:

-	15MHz channel bandwidths is specified for the band n254

FFS: On whether it should be optional or mandatory

### Topic #2: UE RF requirements

-	FCC out-of-band emission requirements, as presented in the table below, shall be applied for band n254.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

- Agreements from RAN4#106bis meeting

### Topic #1: System parameters

Agreements for the system parameters:

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band

NOTE: The 15MHz channel bandwidth may need further technical discussions.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.

### Topic #2: UE RF requirements

Agreements for the UE RF requirements:

-	Existing MPR (same as for other NTN bands) is re-used for this band.

-	UE co-existence requirements:

-	Use L-band UE co-existence requirements as a baseline considering additional bands as follows:

-	NR band n54, n77, n78, n105; E-UTRA band 31, 54, 72, 73, 87, 88, 103;

-	Other bands can be considered during next meetings.

-	Reference sensitivity:

-	n53 REFSENS + "insertion loss" dB; where potential options for the insertion loss are as follows

-	Option 1: 0dB (resulting in same REFSENS as band n53);

-	Option 2: 0.5dB (resulting in same REFSENS as band n256);

-	Other options are not excluded.

### Topic #3: RRM requirements

No new agreements.

### Topic #4: SAN

Agreements from the UE RF system parameters are applied to SAN:

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.