3GPP TSG-RAN WG4 Meeting #108	R4-2314716

Toulouse, France, 21st ‒ 25th August, 2023

Title: 	WF on IoT_NTN_FDD_LS_band

Agenda Item:	9.5.6

Source: 	MediaTek

Document for:	Approval

# Agreements

Issue 1-1-2: For B254 category M1, Is it agreeable to introduce the following three NS flags in total for FCC or ETSI?

| NS value | Regulatory domain | Applicable range (MHz) | In-band emissions | Out-of-band emissions |
| --- | --- | --- | --- | --- |
| NS_03N | ETSI | 1610-1618.25 | ETSI (1610-1618.25) | ETSI (1559-1610MHz) ETSI (>1628.5MHz) |
| NS_04N |  | 1618.25-1626.5 | ETSI (1618.25-1626.5) |  |
| NS_05N | FCC | 1610-1626.5 | FCC (1610-1626.5) | FCC (1559-1610MHz) |

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1?

Agreement:

| NS value | Regulatory domain | Applicable range (MHz) | In-band emissions | Out-of-band emissions |
| --- | --- | --- | --- | --- |
| NS_03N | FCC | 1610-1626.5 | FCC (1610-1626.5) | FCC (1559-1610MHz) |

Issue 1-2-1: For configured Tx power, should the current TS 36.102 requirements be reused for band B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1?

Agreement:

Option 1.

Issue 1-2-2: For power control requirements, should the current TS 36.102 requirements be reused for band B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1?

Agreement:

Option 1.

Issue 1-2-3: For frequency error requirements, should the current TS 36.102 requirements be reused for band B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1?

Agreement:

Option 1.

Issue 1-2-4: For transmit modulation quality requirements, should the current TS 36.102 requirements be reused for band B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1?

Agreement:

Option 1.

Issue 2-1-1: For band B254 category M1, REFSENS should be specified as:

- Proposals

  - Option 1: -103.7dBm (no insertion loss, i.e., -106.2 + 2.5)

  - Option 2: -102.2dBm (Similar to B255 with 0.5dB insertion loss)

  - Option 3: -103.1dBm for HD-FDD (0.6dBm insertion loss), -102.3dBm for FDD (additional 0.8dBm Tx desense)

- Recommended WF

  - TBA

Agreement:

- -102.2dBm for FDD

- -103.1dBm for HD-FDD

Issue 2-1-2: For band B254 category NB1/NB2, REFSENS should be specified as:

- Proposals

  - Option 1: -108.2dBm

  - Option 2:  Others, please elaborate

- Recommended WF

  - Option 1?

Agreement:

- Agree on Option 1

Issue 2-2-1: For band B254 category M1, should the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, i.e., also for band B255, be re-used as Out-of-Band Blocking (OOBB)?

- Proposals

  - Option 1: Yes

  - Option 2: No, please propose a new value.

- Recommended WF

  - Option 1?

Agreement:

Agree on Option 1.

Issue 2-2-2: For band B254 category NB1/NB2, should the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, i.e., also for band B255, be re-used as Out-of-Band Blocking (OOBB)?

- Proposals

  - Option 1: Yes

  - Option 2: No, please propose a new value.

- Recommended WF

  - Option 1?

Agreement:

Agree on Option 1.

# Reference

[1] R4-2311222 Power back-off for IoT in the NTN L-/S-band	Apple

[2] R4-2312045 Discussion on UE RF requirements for FDD band (L+S band) for IoT NTN operation	ZTE Corporation

[3] R4-2312046 CR to TS36.102 Introduction of a new FDD band (L+S band) for IoT NTN operation	ZTE Corporation

[4] R4-2312047 CR to TS36.108 Introduction of a new FDD band (L+S band) for IoT NTN operation	ZTE Corporation

[5] R4-2312048 CR to TS36.133 Introduction of a new FDD band (L+S band) for IoT NTN operation	ZTE Corporation

[6] R4-2313001 on IOT NTN LS band	OPPO

[7] R4-2313624 Running draftCR to TS 36.102 on introducing L+S FDD band for IoT NTN operation	MediaTek Inc.

[8] R4-2313625 DraftCR to TS 36.102 on REFSENS for B254 for IoT NTN operation	MediaTek Inc.

[9] R4-2313626 Further discussion on UE RF requirements for B254 for IoT NTN operation	MediaTek Inc.

[10] R4-2313627 Draft TR for IoT NTN operating bands	MediaTek Inc.