3GPP TSG-RAN WG4 Meeting #108												R4-2314727

Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.20.4

Source:	vivo

Title:	WF on FS_NR_LPWUS

Document for:	Approval

- Topic #1: LP-WUR architectures

- Sub-topic 1-1 UE ACS vs Guard RB evaluation

Issue 1-1-1: Required number of guard RBs for LP-WUS ACS

Agreement:

- Inform RAN1 the guard RB numbers for LP-WUS ACS proposed by companies in this RAN4 meeting.

  - For 5th order filter, the guard RB number is in the range of 1RB ~ 3RBs for 30KHz SCS, or 2RBs ~6RBs for 15KHz SCS.

  - Include the assumption information in the LS to RAN1.

Issue 1-1-3: Link-level simulation based guard RB analysis

Agreement:

- For link-level simulation based guard RB analysis, use 1% BLER as metric for guard RB evaluation.

  - BER and missing detection rate can also be used

    - FFS on the percentage values for BER and missing detection rate

- Sub-topic 1-2 UE ASCS evaluation

Issue 1-2-1: required Guard RBs for LP-WUS ASCS

Agreement:

- Inform RAN1 the guard RB numbers for LP-WUS ASCS proposed by companies in this RAN4 meeting.

  - For 5th order filter, the guard RB number is in the range of 0.5RB ~ 2RBs for 30KHz SCS, or 1RBs ~4RBs for 15KHz SCS.

  - Include the assumption information in the LS to RAN1.

    - Including how to handle ACS and ASCS simultaneous

Issue 1-2-2: WUS location within the carrier

Agreements:

  - LP-WUS can be flexible located within NR carrier as long as the required guard RBs are configured.

Issue 1-2-3: Order of filter for consideration

Agreements

- The filter assumption for guard band size evaluation shall be reasonable for low power WUR.

- Sub-topic 1-3 UE Noise figure

Issue 1-3-1: Required Noise Figure

Agreements

- RAN4 further discuss the Noise figure in Q4 based on the outcome of SNR and coverage in RAN1.

Sub-topic 1-4 WUS power boosting

Issue 1-4-1: LP-WUS power boosting without NR impacted

Agreements

- For OFDM-based WUS waveform, reuse existing NR RE power control dynamic range of BS in TS 38.104 for LP-WUS as starting point. WUS power boosting should minimize any impacts on legacy UEs.

- RAN4 further check the feasibility of 6dB power boosting for LP-WUS assumed by RAN1

- Sub-topic 1-5 Dedicated LP-WUS operation band

Issue 1-5-1: Separated band for LP-WUS operation

- Proposals

  - Option 1: RAN4 shall continue to study the scenario of WUS in a separate band from the main radio.

  - Option 2: No further discussion of dedicated band for LP-WUS in RAN4.

- Agreements

  - FFS