3GPP TSG-RAN WG4 Meeting # 108															R4-2314737

Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.30.4

Source:	Moderator (OPPO)

Title:	WF on NR_SL_enh2_UERF_part1

Document for:	Approval

Introduction

This WF captures the agreement for the online session and also try to have some further agreements on issues such as image rejection, simulation assumptions for PSFCH and S-SSB and remaining RX requirements.

Agreements

Issue 2-1-1: How to capture the regulation note

Agreement:

- Capture the note in both TR 38.786 and TS 38.101-1

  - FFS how to capture notes in the TR and TS.

Issue 2-2-1: Sync raster

- Proposals

  - Option 1: Follow the same rule in Rel-16 and do not define the sync raster for SL-U operating bands. (VIVO)

- Recommended WF

  - Agree on option 1.

Agreement:

- Agree on Option 1

Issue 3-2: A-MPR NS values

Agreement:

- Finalize the following NS values for each band in Rel-18:

    - n46 :  NS_31

    - n96 :  NS_53, NS_60, NS_61

    - n102 : NS_58

Issue 3-1-1: maximum output power

Agreement:

- Agree on only Power Class 5 is considered in Rel-18

  - PC3 can be considered in future releases

Wayforward for MPR/A-MPR simulation

- Issue 3-1-1: Simulation assumption and cases for PSSCH/PSCCH

- WF

- Waveforms: CP-OFDM for SL-U

- Supported CBW: 20/40/60/80/100MHz.

- Modulation: QPSK/16-QAM/64-QAM/256-QAM

- SCS: 15/30/60kHz

- C-IMD: 60dBc

Moderator note: For the sake of simplicity of simulation, 60kHz is removed at this meeting.

- Issue 3-1-2: Image rejection for simulation

- Proposals

  - Option 1: For image rejection, it is proposed to use 30dBc for QPSK/16QAM/64QAM and 34dBc for 256QAM. (OPPO)

  - Option 2: In NR a 28dBc carrier and image is specified but in NR-U, to meet the wideband operation mask of 28dBr outside the allocated sub-band, there are case which would be marginal even without added spectral regrowth. (Skyworks)

Moderator note: Based on the online discussion, it seems 34dBc can be agreed. Furthermore, based on moderator understanding, even the in-band image should not be ignored since the SEM for in-band still apply and need to take the image into account.

- WF

  - To use 34dBc image rejection for QPSK/16QAM/64QAM/256QAM

- General for Issue 3-1-2 and Issue 3-1-3 For the non-contiguous RB sets PSFCH and S-SSB transmission

- The latest RAN1 agreement for contiguous and non-contiguous RB sets PSFCH and S-SSB transmission is  captured as below:

- In such case, we provide the updated simulation assumption of PSFCH and S-SSB to include the multiple RB sets contiguous and non-contiguous cases.

- It is observed that currently for non-contiguous RB sets, there is no RAN4 SEM requirement yet. Hence the mask captured in the agreed WF R4-1915979 has been attached at the end of the SL-U WF for information. This requirement can be used for MPR/A-MPR simulation while for sure the requirement will be further checked in the next RAN4 meeting.

- As currently there is only 2 meetings left to finish the WID, we propose to treat this non-contiguous work for PSFCH and S-SSB as best effort that it is encouraged companies to do our best to finish the work while if it cannot be finished, then in the end of the WID, this feature can be put in future release.

- Issue 3-1-3: Simulation assumption for PSFCH

- Proposals

  - Option 1: For PSFCH simulation assumption, it is proposed to simulate one interlace for each PSFCH. (OPPO)

  - Option 2: Update the simulation assumption for PSFCH MPR/A-MPR. (LGE)

- WF

- The PSFCH simulation assumption is updated as below:

| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set [and multiple RB-sets] will be considered based on RAN1 decision. For single RB-set, RAN4 consider interlacing RBs for PSFCH For multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of interlaced transmission should meet at least 80% of channel bandwidth in a single RB-set i.e. [, Ngap = RBend – RBstart ]. |
| --- | --- |

- Issue 3-1-4: Simulation assumption for S-SSB

- Proposals

  - Option 1: Update the simulation assumption for S-SSB MPR/A-MPR.  (LGE)

- WF

- It has been agreed in RAN1#114 meeting that for PSFCH and S-SSB, both contiguous and non-contiguous multiple RB sets will be supported hence the simulation assumption is updated as below. The non-contiguous RB set requirement for NR-U DL is put in Annex A for information. MPR/A-MPR simulation can be based on the requirement in Annex A for NR-U DL non-contiguous RB sets. Further discussion on the requirement is needed in the next RAN4 meeting.

- The S-SSB simulation assumption is updated as below:

| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of repeated SSB transmission should meet at least 80% of channel bandwidth.in a single RB-set. ( Ngap = RBend – RBstart ) |
| --- | --- |

- Issue 3-3-1: MPR requirement structure

Further discuss next meeting.

- Issue 3-3-2: MPR simulatrion results:

Further discuss next meeting.

Wayforward for RX requirements

- Issue 4-1-1: RB configuration for REFSENS

- Proposals

  - Option 1: To adopt 135 RB for 100MHz with 60kHz SCS for REFSENS calculation for SL-U bands n46, n96 and n102. (vivo)

  - Option 2: Proposal 1: To agree on 135 RBs for SL-U 60kHz 100MHz configuration and remove the “[]” of current version of TR 38.786. (OPPO)

  - Option 3: propose the RB configuration for REFSENS requirements as follow (Meta)

- Table 3-1: Proposed RB configuration for REFSENS requirements for SL-U UE

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |

Moderator note: Since the RB numbers can be supported, we see no reason why we decrease the numbers.

  - WF:

    - Agree on option 1.

- Issue 4-1-2: REFSENS requirement

- Proposals

  - Option 1: Based on RB configuration of issue 4-1-1 option 1 and 2. (OPPO, vivo)

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

Moderator note: This is to remove the square bracket. The issue 4-1-1 discussion does not influence this issue.

  - WF:

    - Agree on option 1.

Sub-topic 4-2 ΔRIB,4R

- Proposals

  - Option 1: It is proposed to re-use the same ΔRIB,4R as NR-U to SL-U. (OPPO)

Table 7.3F.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n46, n96, n102 | -2.2 |

Moderator note: This is to reuse the NR-U allowance.

  - WF:

    - Agree on option 1.

Sub-topic 4-3 FRC

- Issue 4-3-1: FRC for QPSK

- Proposals

- Option 1:  (OPPO)

  - To reuse current NR SL FRC for 15kHz SCS for QPSK, 64QAM and 256QA.

  - For 30kHz SCS, up to 100MHz is supported hence the table A.7.2-2 needs to be updated with 60/80 and 100MHz added..

  - For 60kHz SCS, up to 100MHz is supported hence the table A.7.2-2 needs to be updated with 60/80 and 100MHz added..

- Table A.7.2-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 1608 | 3624 | 7936 | 12296 | 16896 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 2 | 3 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 26556 | 41076 | 55860 |
| Max. Throughput averaged over 100ms | Mbps | 0.1608 | 0.3624 | 0.7936 | 1.2296 | 1.6896 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

- Table A.7.2-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | [30] | [30] | [30] |
| Subchannel size |  | 12 | 10 | 15 | 15 | [10] | [12] | [15] |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | [160] | [216] | [270] |
| MCS Index |  | 4 | 4 | 4 | 4 | [4] | [4] | [4] |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | [QPSK] | [QPSK] | [QPSK] |
| Transport Block Size |  | 1608 | 3624 | 5632 | 7936 | [12296] | [16392] | [21000] |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | [24] | [24] | [24] |
| LDPC base graph |  | 2 | 2 | 1 | 1 | [1] | [1] | [1] |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 | [2] | [2] | [3] |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | [2.25] | [2.25] | [2.25] |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | [1] | [1] | [1] |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 18636 | 26556 | [41076] | [55860] | [70116] |
| Max. Throughput averaged over 100ms | Mbps | 0.3216 | 0.7248 | 1.1264 | 1.5872 | [2.4592] | [3.2784] | [4.2] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

- Table A.7.2-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | [60] | [60] | [60] |
| Subchannel size |  | 10 | 12 | 12 | 10 | [15] | [15] | [15] |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | [75] | [105] | [135] |
| MCS Index |  | 4 | 4 | 4 | 4 | [4] | [4] | [4] |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | [QPSK] | [QPSK] | [QPSK] |
| Transport Block Size |  | 456 | 1608 | 2536 | 3624 | [5504] | [7936] | [10248] |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | [24] | [24] | [24] |
| LDPC base graph |  | 2 | 2 | 2 | 2 | [1] | [1] | [1] |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 | [1] | [1] | [2] |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | [2.25] | [2.25] | [2.25] |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 | [1] | [1] | [1] |
| Binary Channel Bits per Slot |  | 1464 | 5160 | 8328 | 12036 | [18636] | [26556] | [34476] |
| Max. Throughput averaged over 100ms | Mbps | 0.1824 | 0.6432 | 1.0144 | 1.4496 | [2.2016] | [3.1744] | [4.0992] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

  - WF:

    - Agree on option 1.

- Issue 4-3-2: FRC for 16QAM

- Proposals

  - Option 1: (OPPO)

- Table A.7.3-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 11528 | 27144 | 60456 | 92200 | 127080 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 4 | 8 | 11 | 16 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 79524 | 123084 | 167436 |
| Max. Throughput averaged over 100ms | Mbps | 1.1528 | 2.7144 | 6.0456 | 9.22 | 12.708 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

- Table A.7.3-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | [30] | [30] | [30] |
| Subchannel size |  | 12 | 10 | 15 | 15 | [10] | [12] | [15] |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | [160] | [216] | [270] |
| MCS Index |  | 24 | 24 | 24 | 24 | [24] | [24] | [24] |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | [64QAM] | [64QAM] | [64QAM] |
| Transport Block Size |  | 11528 | 27144 | 42016 | 60456 | [92200] | [125016] | [155776] |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | [24] | [24] | [24] |
| LDPC base graph |  | 1 | 1 | 1 | 1 | [1] | [1] | [1] |
| Number of Code Blocks per Slot |  | 2 | 4 | 5 | 8 | [11] | [15] | [19] |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | [6.25] | [6.25] | [6.25] |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | [1] | [1] | [1] |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 55764 | 79524 | [123084] | [167436] | [210204] |
| Max. Throughput averaged over 100ms | Mbps | 2.3056 | 5.4288 | 8.4032 | 12.091 | [18.44] | [25.0032] | [31.1552] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

- TableA.7.3-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | [60] | [60] | [60] |
| Subchannel size |  | 10 | 12 | 12 | 10 | [15] | [15] | [15] |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | [75] | [105] | [135] |
| MCS Index |  | 24 | 24 | 24 | 24 | [24] | [24] | [24] |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | [64QAM] | [64QAM] | [64QAM] |
| Transport Block Size |  | 3240 | 11528 | 18960 | 27144 | [42016] | [59432] | [77896] |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 | [24] | [24] | [24] |
| LDPC base graph |  | 2 | 1 | 1 | 1 | [1] | [1] | [1] |
| Number of Code Blocks per Slot |  | 1 | 2 | 3 | 4 | [5] | [8] | [10] |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | [6.25] | [6.25] | [6.25] |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 |  |  |  |
| Binary Channel Bits per Slot |  | 4248 | 15336 | 24840 | 35964 | [55764] | [79524] | [103284] |
| Max. Throughput averaged over 100ms | Mbps | 1.296 | 4.6112 | 7.584 | 10.858 | [16.8064] | [23.7728] | [31.1584] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

  - WF:

    - Agree on option 1.

- Issue 4-3-3: FRC for 256QAM

- Proposals

  - Option 1:

- Table A.7.4-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 15880 | 36896 | 81976 | 127080 | 172176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 5 | 10 | 16 | 21 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 | 3 |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 106080 | 164160 | 223296 |
| Max. Throughput averaged over 100ms | Mbps | 1.588 | 3.6896 | 8.1976 | 12.708 | 17.218 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

- Table A.7.4-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | [30] | [30] | [30] |
| Subchannel size |  | 12 | 10 | 15 | 15 | [10] | [12] | [15] |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | [160] | [216] | [270] |
| MCS Index |  | 23 | 23 | 23 | 23 | [24] | [24] | [24] |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | [256QAM] | [256QAM] | [256QAM] |
| Transport Block Size |  | 15880 | 36896 | 58384 | 81976 | [127080] | [172176] | [217128] |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | [24] | [24] | [24] |
| LDPC base graph |  | 1 | 1 | 1 | 1 | [1] | [1] | [1] |
| Number of Code Blocks per Slot |  | 2 | 5 | 7 | 10 | [16] | [21] | [26] |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | [6.25] | [6.25] | [6.25] |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 | [3] | [3] | [3] |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 74400 | 106080 | [164160] | [223296] | [280320] |
| Max. Throughput averaged over 100ms | Mbps | 3.176 | 7.3792 | 11.677 | 16.395 | [25.416] | [34.4352] | [43.4256] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

- Table A.7.4-3: Fixed reference channel for V2X receiver requirements (SCS 60kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | [60] | [60] | [60] |
| Subchannel size |  | 10 | 12 | 12 | 10 | [15] | [15] | [15] |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | [75] | [105] | [135] |
| MCS Index |  | 23 | 23 | 23 | 23 | [23] | [23] | [23] |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | [256QAM] | [256QAM] | [256QAM] |
| Transport Block Size |  | 4480 | 15880 | 25608 | 36896 | [57376] | [81976] | [106576] |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | [24] | [24] | [24] |
| LDPC base graph |  | 1 | 1 | 1 | 1 | [1] | [1] | [1] |
| Number of Code Blocks per Slot |  | 1 | 2 | 4 | 5 | [7] | [10] | [13] |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | [6.25] | [6.25] | [6.25] |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 | [3] | [3] | [3] |
| Binary Channel Bits per Slot |  | 5760 | 20544 | 33216 | 48000 | [74400] | [106080] | [137760] |
| Max. Throughput averaged over 100ms | Mbps | 1.792 | 6.352 | 10.243 | 14.758 | [22.9504] | [32.7904] | [42.6304] |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

  - WF:

    - Agree on option 1.

Annex A NR-U requirement for non-contiguous RB set DL

The SEM isbased on the agreed WF R4-1915979:

Small modification is marked as yellow that for point D, it should be 0.5N-1 instead of 0.5N+1 which is quite straight forward.