3GPP TSG-RAN WG4 Meeting #108	R4-2314738

Toulouse, France, 21 – 25 August 2023

Title: 	WF on NR_SL_enh2_UERF_part2

Agenda Item:	8.30.4

Source: 	LG Electronics

Document for:	Approval

1-1: Tx requirements of con-current operation

1-1-1: MOP for Uu@Licensed and SL@Un-licensed

Proposals:

  - Option 1: Defines MOP per UE as the total transmitted power from each operating band

  - Option 2: Define MOP per each operating band in Rel-18

WF:

  - Further discuss this issue with Issue 1-1-2 as a whole package in the next meeting.

1-1-2: Configured transmitted power for Uu@Licensed and SL@Un-licensed

Proposals:

  - Option 1: Apply the principle of the configured transmitted power of NR inter-band CA for the total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR sidelink

  - PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p)+ pCMAX_L,c,SL (q)], PPowerClass_CA, PEMAX,CA}

  - PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL (q)], PPowerClass_CA, PEMAX,CA}

  - Option 2: Consider the maximum total transmit power (e.g., PEMAX,con-current)

  - Option 3: Follow the same approach as Rel-17 inter-band concurrent operation

WF:

  - Further discuss this issue in the next meeting.

1-2: Rx requirements of con-current operation

1-2-1: MSD for Uu@Licensed and SL@Un-licensed

Agreement:

  - Reuse MSD requirements of inter band CA_n46-n78 for con-current operation with Uu@78 + SL@n46

2-1: LTE SL and NR SL co-channel coexistence scenarios

2-1-2: RF requirement impact due to NR 2nd slot power limitation of RAN1 agreement for LTE SL and NR SL co-channel coexistence scenario

Proposals

  - Option 1: (R4-2310314 (WF in RAN4#107))

  - RAN4 should analyze if additional functionality or requirements are needed to ensure that NR SL UEs follow the behavior as RAN1 agreement.

  - RAN4 should analyze if this beahvior that ensures the co-channel coexistence can impact some other minimum performance requirements. One example here can be the PUMAX,f,c.

  - In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured using the 1st slot only

  - Option 2

  - Wait until RAN1 finish the complete mechanism first and after that we can further check if additional UE RF output power requirement needs to be defined (Oppo)

  - Option 3

  - Requirements on PUMAX.f.c will not be impacted by RAN1 constraint on not increasing transmit power in two consecutive slots overlapping the same LTE SL subframe for sidelink co-channel coexistence between NR and LTE V2X, when NR SL is configured for 30kHz SCS (Nokia)

  - Discuss whether and how to capture the RAN1 constraint on not increasing transmit power difference in two consecutive slots overlapping the same LTE SL subframe for sidelink co-channel coexistence between NR and LTE V2X, when NR SL is configured for 30kHz SCS (Nokia)

  - Option 4

  - RAN4 can solve the RAN1 agreement as UE implementation (Meta)

WF:

  - Discuss whether and how to capture the RAN1 constraint on not increasing transmit power difference in two consecutive slots overlapping the same LTE SL subframe for sidelink co-channel coexistence between NR and LTE V2X, when NR SL is configured for 30kHz SCS

  - Option 1: In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured using the 1st slot only

  - Option 2 : Others