# 3GPP TSG-RAN WG4 Meeting # 108                                                   	R4-2314752

Toulouse, France, 21 - 25 Aug 2023

Source:	DISH Network

Title:	TP for TR 38.896 to add PC2 for n70

Agenda item:			7.22.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.896 to add PC2 for n70 including RSD requirements based on the WF in [1].

# 2. Reference

[1] R4-2314646	WF on high power UE for FDD single bands, China Unicom

# 3. Text Proposal

<Start of changes>

## 5.x	NR band n70

### 5.x.1	UE maximum output power

Table 5.1.1-1: UE output power for PC2

| NR Band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n70 | 26 | +2/-3 |

### 5.x.2	A-MPR requirements

On network signalling NS_03, for the NR waveforms DFT-s-OFDM and CP-OFDM there is no need for additional power back-off and the PC3 table could be re-used.

### 5.x.3	Reference sensitivity requirements

For PC2 single band n70, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n70 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.x.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | Skyworks(R4-2311152) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Apple(R4-2311251) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | N/A | N/A | N/A | N/A | N/A |
|  | Mediatek(R4-2312178) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Qualcomm (R4-2313553) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |

For PC2 single band n70 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.x.3-2:

Table 5.x.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | Skyworks(R4-2311152) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Apple(R4-2311251) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | N/A | N/A | N/A | N/A | N/A |
|  | Mediatek(R4-2312178) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Qualcomm (R4-2313553) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |

<End of changes>