# 3GPP TSG-RAN WG4 Meeting # 108	      R4-2314755

# Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.13.6

Source:	CMCC

Title:	WF on ATG co-existence evaluation

Document for:	Approval

- Background

RAN#96 meeting approved RP-221369 Revised WID on Air-to-ground network for NR in Rel-18. The agreements and open issues are captured in this way forward.

Previous agreed WFs are R4-2310401, R4-2217736, R4-2220542, R4-2303573 and R4-2306607.

- Open issues

  - Topic #1: Simulation assumption

Issue 1-2: wrap around in TN for unsynchronized scenarios

- Agreement:

  - In the simulation of unsynchronized scenarios, no wrap around in TN and ATG

Issue 1-3: clarification of metric for TN gNB as victim

- Agreement:

  - The TN cell with worst throughput loss is the cell with maximum value of LossACI

    - LossACI=1-ThputACIThputSINGLE

Issue 1-5: ISD assumption

- Proposals

  - Option 1: 900m ISD is proposed for 4GHz non-synchronized scenario 5 and 7. (Huawei)

    - For scenario 5, if 3500m ISD is assumed for 4GHz, it seems that the throughput loss is smaller than 5% without any isolation distance. However, if the ISD 3500m for 4GHz is decreased, the throughput loss can’t be ignored.

- Agreement:

  - Keep 3500m ISD

  - Topic #2: Simulation results of synchronized scenario

Issue 2-1-1: ACLR/ACS value based on simulation results

- Agreement:

Agreed ATG ACLR and ACS values based on synchronized scenarios

| ATG |  | Values |
| --- | --- | --- |
| ATG BS | ACLR | 45 |
|  | ACS | 46 |
| ATG CPE | ACLR | 30 |
|  | ACS | 33 |

  - Topic #2: Simulation results of non-synchronized scenario

Note: Summary of simulation results in RAN4#108 meeting are captured in section 2.3.1~2.3.3 for information

Agreement:

Perform calibration for non-synchronized  before next meeting, the metrics to be calibrated would be:

- Cross coupling loss between ATG BS and TN BS with 0 azimuth angle (TN and ATG BS on the same horizontal line)

- 5 km isolation distance

    - Preliminary simulation results for scenario 5: ATG DL -> TN UL (4GHz)

Summary of simulation results:

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CMCC 7km, ZTE 17km, Ericsson 32km)

  - whereas other companies show larger than 100km is required(Qualcomm 120km) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (Ericsson 3-11km, CMCC 2-5km, ZTE 3-17km)

    - Preliminary simulation results for scenario 7: TN rural DL -> ATG UL (4GHz)

Summary of simulation results:

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CATT 11km, CMCC 9km, ZTE 20km)

  - whereas other companies show 100-300km is required(Qualcomm larger than 200km, Ericsson 150km) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (CATT 11km, CMCC 5km, ZTE 9km, Ericsson 16km)

    - Preliminary simulation results for scenario 14: TN rural DL -> ATG UL (2GHz)

Summary of simulation results:

- When boresight is zero,

  - some companies show tens of km is enough even for the case when boresight is zero (CATT 20km, CMCC 12km, ZTE 20km)

  - whereas other companies show 100-300km or even larger is required(Qualcomm larger than 200km, Ericsson 220km, Huawei larger distance) is required.

- When boresight is 30 degree or 60 degree,

  - companies show tens of km is enough even for the case when boresight is zero (CMCC 3-5km, ZTE 10-17km, Ericsson 6-26km)