3GPP TSG-RAN4 Meeting #108	R4-2314757

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0649 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Feature CR for RF requirements of FR2 HST multi-panel operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FR2 HST multi-panel operation is new feature of PC6 UE. RF requierments for this feature need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | introduce 2AoA spherical coverage requirement for PC6 UE in new clause 7.3K - The concerned two AoA directions are selected from different coverage areas, i.e., Area-1 and Area-2 respectively - DL power level requirements of each AoA are the same as legacy EIS spherical coverage requierment - all possible angular separations are applicable for core requirements as long as the two test points are from Area-1 and Area-2 respectively; 150° angular separation in theta of reference coordination is used for test verification - no need to consider AoA+ offset and AoA- offset and different UE orientations - other agreements from Multi-Rx DL WI which are deemed applicable for PC6 UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 HST multi-panel operation feature could not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3K.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

7.3D	Void

7.3K. 6	2AoA spherical coverage for power class 6

For enhanced FR2-1 power class 6 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier supporting the following set of capabilities, 2AoA spherical coverage requirements apply:

- simultaneousReceptionDiffTypeD-r16

- 4L per CC in DL

- either of:

  - singleDCI-SDM-scheme-r16 or

  - multiDCI-MultiTRP-r16 and either of:

    - overlapPDSCHsFullyFreqTime-r16.

    - overlapPDSCHsInTimePartiallyFreq-r16

The requirement applies with simultaneous reception of rank 2 PDSCH, where each layer uses overlapping RBs in both time and frequency and is associated with a TCI state and AoA. The scheduled TCI states for the two layer PDSCH shall be configured with different QCL type-D reference signals.

The requirements are applicable with network signalling highSpeedDeploymentTypeFR2-r17 configured as bidirectional. UE spherical coverage evaluation areas are found in Table 6.2.1.6-3a in clause 6.2.1.6, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex K.1. If one AoA is within Area-1 and another AoA is within Area-2, the 2AoA spherical coverage requirements apply with DL power specified in Table 7.3K..6-1 for the PDSCH of each AoA. For any AoA pair selected from Area-1 and Area-2, respectively, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels which is specified in A.3.x and A.3.y for UEs supporting singleDCI-SDM-scheme-r16 and for UEs supporting multiDCI-MultiTRP-r16 respectively. The requirement is verified with a 150° angular separation between 2AoAs. The requirement is verified with the test metric of Throughput (Link=2AoA Spherical coverage grid, Meas=Link angle).

Table 7.3K. 6-1: DL power for 2AoA spherical coverage requirement for power class 6

| Operating band | PDSCH DL power over UE spherical coverage evaluation areas (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -82.6 | -79.6 | -76.6 | -73.6 |
| n258 | -82.8 | -79.8 | -76.8 | -73.8 |
| n261 | -82.6 | -79.6 | -76.6 | -73.6 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The 2AoA spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. NOTE 3: The requirements in this table are applicable with the network signalling highSpeedMeasFlagFR2-r17 configured as set2. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for 2AoA spherical coverage shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1) configured.

<< End of change>>