3GPP TSG-RAN WG4 Meeting #108	R4-2314761

Toulouse, France, 21st Aug 2023 - 25th Aug 2023

Title: 	WF on NR_ATG_UERF_part2

Agenda Item:	8.13.6

Source: 	Huawei

Document for:	Approval

Way forward

Issue 2-1-1: Minimum distance assumption between ATG BS and ATG UE

- Proposal:

  - Option 1: Considering the possibility of ATG serving the aircraft directly above in the future still exists, 3km could be used as the minimum distance between ATG UE from BS.

  - Option 2: 20km referring to coexistence study.

  - Option 3: Others

Issue 2-1-2: Minimum output power

Agreement:

Use the following table as baseline.

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Frequency | 2GHz | 4GHz |
| Distance | TBD | TBD |
| PL | tbd | TBD |
| ATG BS antenna gain | TBD | TBD |
| ATG UE antenna gain | 0dB | TBD |
| Thermal noise power density | -114dBm/MHz | -114dBm/MHz |
| ATG BS Noise figure | 5dB | 5dB |
| Minimum output power | -13dBm/MHz TBD | -17dBm/MHz TBD |

Issue 2-3-1: 256QAM transmit modulation quality requirements

- Proposal:

  - Option 1: The existing 256QAM transmit modulation quality requirements for TN UE in TS 38.101-1 should be reused for ATG UE.

  - Option 2: Others

Agreement:

Agree on Option 1.

Issue 2-3-1: SEM requirements

- Proposal:

  - Option 1: to reuse the FR1 UE SEM requirement for ATG CPE.

  - Option 2: Others

<Way forward>: It’s encouraged to provide more analysis on how to derive SEM requirements.

Issue 3-1-2: Maximum input level

Agreement:

Use the following table as template to capture the assumption

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Carrier frequency (GHz) | 2 | 4 |
| Distance (km) | TBD | TBD |
| Path Loss (dB) | TBD | TBD |
| BS output power (dBm) | 46 | 53 |
| BS antenna gain (dBi) | TBD | TBD |
| ATG UE antenna gain (dBi) | 0 | TBD |
| Maximum input level (dBm) | TBD | TBD |

Issue 3-2-1: ACS requirements

- Proposal:

  - Option 1: to confirm 33dBc ACS requirement for ATG UE. (CMCC, ZTE)

  - Option 2: Others

Agreement:

Agree on Option 1.

Issue 3-2-1: In-band blocking

- Proposal:

  - Option 1: The in-band blocking requirements in TS 38.101-1 could be reused.

  - Option 2: Propose to use -56 dBm interference power level for in-band blocking.

    - It is not necessary to consider blocking from other ATG networks when assessing the in-band blocking requirement.

Agreement:

- The in-band blocking requirements in TS 38.101-1 could be reused.

Issue 3-2-1: Out-of-band blocking/ Spurious response

<Way forward>: FFS.

Issue 3-2-1: Intermodulation characteristics

- Proposal:

  - Option 1: to reuse Intermodulation characteristics requirements specified in TS 38.101-1 for ATG UE.

  - Option 2: The RX intermodulation requirement should be considered after the maximum input level and blocking requirements are determined.

  - Option 3: Others

Agreement:

Agree on Option 2.