3GPP TSG-RAN WG4 Meeting #108	R4-2314769

Toulouse, France, 21st August – 25th August 2023

Source:	Ericsson, Bell Mobility

Title:	TP for 38718-02-01 adding UL CA_n78(2A) to DL CA_n25A-n78(2A)

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 adding UL CA_n78(2A) to DL CA_n25A-n78(2A).

# 2. Text Proposal

# ---Start of changes---

## 5.x		CA_n25-n78

### 5.x.1		Specific for 1 bands UL CA

#### 5.x.1.1 Operating bands for CA

CA_n25-n78 has already been specified and this section does not need to be revisited.

#### 5.x.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) is to be added to DL CA_n25A-n78(2A) in the existing configuration table of 38.101-1. See Table 5.x.1-1 below.

Table 5.x.1.2-2: Supported bandwidths per CA band combination of band n25+n78

| CA_n25A-n78(2A) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | CA_n78(2A) | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.1.3-1 gives a non-contiguous uplink IMD interference analysis.

Table 5.x.1.3-1: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.x.2.2-3, there are IMD7 products falling inside n25 DL, which may occur in simultaneous RX/TX configurations.

#### 5.x.1.4	∆TIB and ∆RIB values

CA_n25-n78 has already been specified and this section does not need to be revisited.

#### 5.x.1.5	REFSENs requirements

Based on the co-existence studies there is a need to define MSD values. MSD value are reused from CA_n2A-n77A with CA_n77(2A) in the UL.

Table 5.x.1.5-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n78 | n25 | N/A | 5 | N/A | 1980 | 2.7 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 RBSTART=7 | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 RBSTART=0 | 3760 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-231115, Revised WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE

[2]		R4-2308181, TR 38.718-02-01 v0.6.0: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE Corporation