3GPP TSG-RAN WG4 Meeting #108                                                    	R4-2314818

Toulous, France, 21– 25 Aug., 2023

Source:	Huawei, Hisilicon, Ericsson, Rogers

Title:	TP to TR 37.718-11-21 Addition of DC_12A_n2A-n77A

Agenda item:			7.6.2

Document for:	Approval

# 1. Introduction

The new basket WID for DC_R18_xBLTE_2BNR_yDL2UL was approved in RAN #99 meeting [1]. This contribution provides a TP for TR 37.718-11-21 to finish the requirements for DC_12A_n2A-n77A.

2 Discussion

We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1.

Figure 1 Main/diversity antenna topology

References

[1] RP-230130, Revised WID on Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)), LG Electronics, RAN#99，Mar.  2023

Text Proposal

<TP for TR 37.718-11-21>

<Start of Text Proposal>

6.x	DC_12_n2-n77

6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n77 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.x.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A_n2A-n77A | DC_12A_n2A DC_12A_n77A |

6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 35 MHz |  | 40 MHz |  | 45 MHz |  | 50 MHz |  | 60 MHz |  | 70 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz |  | Maximum aggregated bandwidth [MHz] |  |
| DC_12A_n2A-n77A |  | 12 |  | 15 |  | 51 |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150 |  |
|  |  | n2 |  | 15 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  | 40 |  |  |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  | 40 |  |  |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  | 40 |  |  |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |

6.x.3	Co-existence studies

The harmonic and IMD products caused by UL DC_12_n2 are shown below.

Table 6.x.3-1: The harmonic and IMD products caused by UL DC_12_n2

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 699 | 716 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1398 | 1432 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2097 | 2148 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 2796 | 2864 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 3495 | 3580 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1211 | 1134 | 2549 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2984 | 3121 | 512 | 418 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4399 | 4536 | 3248 | 3342 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4834 | 5031 | 187 | 298 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6249 | 6446 | 3947 | 4058 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2268 | 2422 | 5098 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1014 | 886 | 6941 | 6684 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1552 | 1723 | 4332 | 4118 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4646 | 4774 | 8099 | 8356 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5797 | 5968 | 6948 | 7162 |

Based on the above table,  3rd, 4th and 5th order IMD products generated by DC_12_n2 uplink may fall into own Rx of band n77.

Table 6.x.3-2: The harmonic and IMD products caused by UL DC_12_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

The IMD3 and IMD4 interference of UL DC_12_n77 may have an impact on the Rx of band n2.

The IMD5 interference of UL DC_12_n77 may have an impact on the Rx of band n12.

6.x.4	∆TIB and ∆RIB values

For DC_12_n2-n77, the TIB,c and RIB,c values are reused from CA_n2-n12-n77 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n2-n77 | 0.3 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n2-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.x.5	MSD requirements

Based on co-existence studies MSD need to be defined. MSD values are reused from CA_n2-n12-n77 for IMD3. Based on the guideline in TR 38.846, that is, for type 2 UL configurations, if the DL band may be affected by a mix of even and odd order IMD products, then the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications, the IMD4 is added.

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n2A-n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.5 | IMD3 |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n2 | 1890 | 5 | 25 | 1970 | 12 | IMD4 |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | IMD34, |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n77 | 4000 | 10 | 50 | 4000 | 12 | IMD4 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

<End of Text Proposal>