3GPP TSG-RAN WG4 Meeting #108	R4-2314848

Toulouse, France, 21 – 25 August 2023

Source:	Ericsson, Rogers

Title:	TP for TR 37.718-21-11: to include band combinations DC_66A-71A_n12A

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_66A-71A_n12A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.x	DC_66-71_n12

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A-71A_n12A | DC_66A_n12A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_66_n12, there is no own Rx impact of the 3rd band for this combination.

### 5.x.3	∆TIB and ∆RIB values

For DC_66-71_n2, the TIB,c and RIB,c values are reused from DC_5-66_n71 are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66-71_n12 | 0.3 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.X.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66-71_n12 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	Reference sensitivity exceptions

Based in the co-existence studies in 5.x.2, there is no need to define MSD values.

---End of changes---

# Reference

[1]		RP-231132, Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei