3GPP TSG-RAN WG4 Meeting # 108										          R4-2314855

Toulouse, France, August 21 – August 25, 2023

Title: 	WF on CA_n5B DL/UL BCS1 MSD test point

Agenda Item:	7.1.1.2

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Background

In RAN#100, UL CA_n5B with a maximum 25MHz aggregated UL BW was recently requested. This BCS request was initially analysed in RAN4 #100-e [1], and at this meeting in [2] and [3]. This WF captures the MSD test point based on guidelines [4].

# Way forward

<Agreement>:

At RAN4#108 multiple proposals for MSD were presented for UL CA_n5B as listed in the references [2,3]. During the ad-hoc discussions, an agreement was reached to add only 1 test point specific to BCS1 as captured in Table 1 below based on guidelines [4]. It is agreed that the legacy BCS0 test point is kept and that a new footnote is added to clarify the test point applicability to BCS1.

Table 1 Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [FFS] | [FFS] | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

# References

- R4-2114579 CA_n5B MSD, 3GPP TSG-RAN WG4 Meeting # 100e Electronic meeting, Skyworks Solutions Inc.

- R4-2313380 MSD for UL CA_n5B, 3GPP TSG-RAN WG4 #108 Toulouse, France, Qualcomm France.

- R4-2311260 CA_n5B ULCA MSD for BCS 1 and 2, 3GPP TSG-RAN WG4 #108 Toulouse, France, Skyworks Solutions Inc.

- R4-2314864 WF on intra-band contiguous UL-CA MSD, 3GPP TSG-RAN WG4 #108 Toulouse, France, Skyworks Solutions Inc.