3GPP TSG-RAN WG4 Meeting #108	R4-23xxxxx

Toulouse, France, 21st - 25th Aug 2023

Title: 	WF on CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29 MSDs with CA_n5B UL

Agenda Item:	7.1

Source: 	Murata, Skyworks, AT&T, Qualcomm, [Apple] …

Document for:	Approval

# Introduction

In this meeting, a discussion paper [1] was proposed for CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29 MSDs with CA_n5B UL In this paper, we’d like to propose the WF with aligned test points and UL configuration to finalize MSD values.

# WF on CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29 MSDs with CA_n5B UL

<Background >:

Issue 1-2: CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29 MSDs with CA_n5B UL

Proposal:

- MSD due to IMDs of intra band CA_n5B UL should be specified for CA_n5B-n12, CA_n5B-n14 and CA_n5B-n29

  - Using total RB allocation equal to the REFSENS UL configuration (10RB+10RB in this case) as an exception for LBLB combinations

  - Alternatively, wait for another meeting to evaluate the “regular” 1RB+1RB allocation

- Cross-band MSD of n5 UL for CA_n5B-n12, CA_n5B-n14 and CA_n5B-n29 can be ignored

Proposal for MSD based on 10RB+10RB UL allocation:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 10 (RBSTART=42) | 884 | N/A |  |  |
|  | n12 | N/A | 5 | N/A | 743.5 | 7.3 | FDD | IMD9 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 10 (RBSTART=42) | 884 | N/A |  |  |
|  | n14 | N/A | 5 | N/A | 765.5 | 11.5 | FDD | IMD7 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 10 (RBSTART=42) | 884 | N/A |  |  |
|  | n29 | N/A | 5 | N/A | 725.5 | 6.2 | FDD | IMD11 |

<Way forward >:

- The following option is presented as a WF.

  - Option 1: Use UL configuration 1RB+ 1RB (Equal PSD) to align with existing specification in 38.101-1

    - Finalize IMD order and DL frequency test point if necessary. Adjust RB start position if necessary to center IMD noise. Determine MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n12 | N/A | 5 | N/A | [743.5] | TBD | FDD | [IMD9] |
| CA_n5-n14 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n14 | N/A | 5 | N/A | [765.5] | TBD | FDD | [IMD7] |
| CA_n5-n29 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n29 | N/A | 5 | N/A | [725.5] | TBD | FDD | [IMD11] |

  - Option 2: Other options not precluded for further information.

# 3. Reference

[1] R4-2311269, CA_n5B ULCA MSD due to IMD for CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29, Skyworks Solutions Inc.

[2] R4-231115 Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE Corporation, RAN#100