3GPP TSG-RAN WG4 Meeting # 108												R4-2314585

Toulouse, France, August 21 – August 25, 2023

Title: 	WF on MSD for n71-n85

Agenda Item:	7.1.1.2

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., Qualcomm

Document for:	Approval

# 1	Background

To finalise CA_n71-n85 BCS4/5 request, the main aspect left open is the cross-band MSD of band n85 due to the new largest Band n71 UL CBW. The latest RAN approved maximum symmetrical UL/DL CBW of 30MHz has been used so far in deriving the cross-band MSD. However, the band combination proponent has now requested 35MHz symmetrical UL/DL CBW for n71.

Separately the Release 18 specification will have to handle two types of UEs: Legacy UEs that only support up to 20MHz UL CBW and UEs that will support all or part of the new symmetrical UL CBW.

This way forward provides input on MSD test point and values depending on UE UL CBW.

# Way forward

<Agreement>:

At RAN4#108 multiple proposals for MSD were presented as listed in the references. During discussion an agreement were reached as captured below.

Table 6.x.1.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
|  |  | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 23Y | ACLR1 |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=0) | 649.5 | 5 | 3.8 | >ACLR2 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE y: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

The MSD vales above is captured in the TP to 38.718-02-01 - R4-2314850.

# References

- R4-2220560, WF on 1UL CA_n71-n85 architecture and requirement, T-Mobile USA, R4#105-e

- R4-2306479, WF on CA_n71-n85 and related band 12 and 71 combinations, T-Mobile USA, R4#106bis-e

- R4-2310318, “WF on MSD test point for n71-n85 depending on max BW and legacy/new BW support”, T-Mobile USA, Skyworks Solutions, Inc., R4#107, Incheon, South Korea

- R4-2304957, “Discussion on addition of CA_n71-n85”, Nokia,T-Mobile, R4#106bis-e

- R4-2313045, TP to TR 38.717-02-01: Addition of CA_n71-n85”, Nokia / T-Mobile

- R4-2311304, CA_n71-n85_30M_35M, Murata Manufacturing Co Ltd.

- R4-2313352, MSD for CA_n71A-n85A, Qualcomm France

- R4-2313258, MSD analysis for CA_n71A-n85A with wider n71 UL CBWs, Apple

- R4-2311118, CA_n71-n85 cross band MSD for 35MHz n71 UL CBW, Skyworks Solutions Inc.