3GPP TSG-RAN WG4 Meeting #108                                     R4-2314875

Toulouse, France, August 21 – August 25, 2023

Source: 	Google, CHTTL

Title: 	WF on IE supportedBandwidthCombinationSetIntraENDC

Agenda Item:	5.1.3

Document for:	Approval

# Background

Backgrounds for the issue of IE supportedBandwidthCombinationSetIntraENDC.

  - For the higher order inter-band EN-DC/NE-DC combinations with a common band on the LTE and NR side, RAN2 had introduced a signaling IE, supportedBandwidthCombinationSetIntraENDC, to indicate the BCS for the intra-band EN-DC/NE-DC component based on the RAN4 LS [1][2].

  - For example, for the band combination DC_3A_n3A-n77A, this IE can be used to indicate BCS for the intra-band components DC_3A_n3A within inter-band EN-DC band combination.

  - If this IE is not included, the network assumes the UE supports BCS0 for the intra-band EN-DC/NE-DC as defined in the TS 38.101-3 [3][4][5][6][7].

  - Note that this IE can indicate only one intra-band EN-DC/NE-DC component of an inter-band EN-DC/NE-DC band combination.

  - The description of the signaling IE in TS 38.306 is provided below.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportedBandwidthCombinationSetIntraENDC Defines the supported bandwidth combination set for a band combination that allows configuration of at least one EUTRA serving cell and at least one NR serving cell in the same band, as defined in the TS 38.101-3 [4], table 5.3B.1.2-1 and table 5.3B.1.3-1. - For intra-band (NG)EN-DC with additional inter-band CA component(s) of LTE and/or NR, the field defines the bandwidth combinations for the intra-band (NG)EN-DC component. - For intra-band NE-DC with additional inter-band CA component(s) of LTE and/or NR, the field defines the bandwidth combinations for the intra-band NE-DC component. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in the TS 38.101-3 [4]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. - It is mandatory if the band combination is an intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component. - It is optional if the band combination is an intra-band (NG)EN-DC/NE- DC combination without supporting UL in both the bands of the intra- band (NG)EN-DC/NE-DC UL part. If not included, the network assumes the UE supports BCS0 as defined in TS 38.101-3 [4], table 5.3B.1.2-1 and table 5.3B.1.3-1 for the intra-band (NG)EN-DC/NE-DC. | BC | CY | N/A | N/A |

  - However, it was identified that this signaling IE might encounter an ambiguous issue if two intra-band EN-DC/NE-DC components is within an inter-band EN-DC/NE-DC band combination [8].

  - In Rel-17, some inter-band EN-DC band combinations with two intra-band EN-DC components are introduced, which are captured in the following table.

  - For example, DC_3A-41A_n3A-n41A is an inter-band EN-DC band combination with two intra-band EN-DC components DC_3A_n3A and DC_41A_n41A.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

  - As the signaling IE supportedBandwidthCombinationSetIntraENDC can only indicate one intra-band EN-DC component, there will be an ambiguous issue for the above example combinations that the network does not know which BCS of the intra-band EN-DC component the UE is referring to.

  - In addition, the UE is not able to indicate the BCSs of the two intra-band EN-DC components due to the current signalling restriction.

# Way Forward

- Companies are encouraged to check whether the current RAN2 signaling exists an issue on reporting the BCS(s) of the intra-band EN-DC components for a higher order inter-band EN-DC band combination with two intra-band EN-DC components.

# References

- R4-2102149, “Reply LS to RP-202935 on BCS reporting and support for intra-band EN-DC band combinations”, RAN4#98-e, T-Mobile USA

- R4-2103401, “Second Reply LS to RP-202935 = R4-2100025 on BCS reporting and support for intra-band EN-DC band combinations”, RAN4#98-e, T-Mobile USA

- R4-2103402, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R4-2103403, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R4-2103404, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R2-2102216, “Clarification on the supportedBandwidthCombinationSetIntraENDC capability”, RAN2#113-e, Nokia, Nokia Shanghai Bell

- R2-2102217, “Clarification on the supportedBandwidthCombinationSetIntraENDC capability”, RAN2#113-e, Nokia, Nokia Shanghai Bell

- R4-2313499, “Discussion on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#108, Google Inc