# 3GPP TSG-RAN4 Meeting #108	R4-2314881

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1646 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 Rel-18 Introduction of TDD uplink RMC for shorter transients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_R18_BWs-Core |  |  |  |  |  |  | Date: |  | 2023-08-24 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed at RAN4 meeting #106 that the transients EVM for shorter transient period capability within FR1 are: - verified using the Figure 6.3.3.7-3 ON-to-ON time mask, - not triggered by TPC commands, they are triggered by an RB allocation change. For TDD bands, a new UL RMC channel is needed to meet the Figure 6.3.3.7-3 ON-to-ON time mask. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For shorter transient EVM: - TDD active uplink slot configuration Table A.2.1-4, - additional TDD pattern Table A.2.1-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EVM for shorter transient period capability within FR1 may not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

## << Start of change >>

# A.2	UL reference measurement channels

## A.2.1	General

The measurement channels in the following clauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

The measurement channels in the following clauses are applicable to both FDD and TDD.

The active uplink slots for TDD configurations are specified in table A.2.1-1. TDD slot patterns defined for reference sensitivity tests will be used for TDD UL RMCs, unless otherwise stated. The active uplink slots for additional TDD configuration specified in Table A.2.1-2, with the TDD pattern defined in Table A.2.1-3, is used for the phase continuity tests for DMRS bundling with 15 kHz SCS. The active uplink slots configuration specified in Table A.2.1-4 and the additional TDD pattern in Table A.2.1-5 are used for shorter transient period capability EVM tests at 15 kHz SCS.

Table A.2.1-1: TDD active uplink slots

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 4, 9 |
| 30 kHz | 8, 9, 18, 19 |
| 60 kHz | 16, 17, 18, 19, 36, 37, 38, 39 |

Table A.2.1-2: TDD active uplink slots for additional TDD configuration

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 8, 9 |

Table A.2.1-3: TDD pattern for additional TDD configuration

| Parameter |  | Value |
| --- | --- | --- |
|  |  | SCS 15 kHz (µ0) |
| TDD Slot Configuration pattern (Note 1) |  | 7DS2U |
| Special Slot Configuration (Note 2) |  | 6D+4G+4U |
| referenceSubcarrierSpacing |  | 15 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 10 ms |
|  | nrofDownlinkSlots | 7 |
|  | nrofDownlinkSymbols | 6 |
|  | nrofUplinkSlot | 2 |
|  | nrofUplinkSymbols | 4 |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |

Table A.2.1-4: TDD active uplink slots for shorter transient period capability

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 3,4 |

Table A.2.1-5: Additional TDD pattern for shorter transient period capability

| Parameter |  | Value |
| --- | --- | --- |
|  |  | SCS 15 kHz (µ0) |
| TDD Slot Configuration pattern (Note 1) |  | 2DS2U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U |
| referenceSubcarrierSpacing |  | 15 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms |
|  | nrofDownlinkSlots | 2 |
|  | nrofDownlinkSymbols | 10 |
|  | nrofUplinkSlot | 2 |
|  | nrofUplinkSymbols | 2 |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |

## << End of change >>