3GPP TSG-RAN4 Meeting #108	R4-2314890

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1660 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_unlic_enh-Core] CR for TS 38.101-1 Rel-18: Corrections to NS_53 A-MPR for 100MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh-Core. |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The A-MPR for 100MHz of NS_53 is still in brackets and requires finalisation. The minimum required back-off for channel of 100MHz is 4dB when considering interlaced allocations and the PSD limit of -1dBm/MHz. Additional simulations on NS_53, CP-OFDM, QPSK have shown an additional need of 0.5dB for 100MHz channel. The A-MPR values is increased from 3.5dB to 4.0dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To enable compliance with PSD limits the DFT-s-OFDM Partial A-MPR value is set to a minimum of 4dB. Updated the value for NS_53, CP-OFDM, QPSK and 100MHz to 4.0dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compliance with PSD limits for NS_53 might not be achievable and use of NS_53 might not be possible in the field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version covered the correction for 100MHz DFT-s-OFDM A-MPR based on PSD considerations. The revision includes correction for 100MHz CP-OFDM QPSK based on additional simulation analysis. |  |  |  |  |  |  |  |  |

<<< Start of changed section>>>

#### 6.2F.3.6	A-MPR for NS_53

When “NS_53” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.6-1 for power class 5 and in Table 6.2F.3.6-2 for power class 3.

Table 6.2F.3.6-1: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 4.0 |
|  | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 2.0 | ≤ 4.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2F.3.6-2: A-MPR for NS_53 power class 3 with single Tx

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 8.5 | ≤ 6.5 | ≤ 7.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

<<< End of changed section>>>