3GPP TSG-RAN WG4 Meeting # 108	R4-2314927

Toulouse, France, August 21st – August 25th, 2023

Title: 	WF on the requirements for n109

Agenda Item:	[108][118] NR_FDD_ULn28_DLn75_n76

Source: 	Vodafone

Document for:	Approval

# Band definition and system parameters:

## Sub-topic 1-1: NR band definition

Issue 1-1-1: Assigning NR operating band number

Agreement: Assigning NR operating band number n109 as follows:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n109 | 703 MHz – 733 MHz | 1432 MHz – 1517 MHz | FDD |

Issue 1-1-2: CA_n28-n75

Agreement: no need for a note that mandates the support of CA_n28-n75

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n109 | 703 MHz – 733 MHz | 1432 MHz – 1517 MHz | FDD |

## Sub-topic 1-2: Channel BW

Issue 1-2-1: CBW UL&DL

Agreement: FDD_ULn28_DLn75 can support channel bandwidth as given in the table below:

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 |
| n109 | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 403 |  | 503 |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 403 |  | 503 |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |

Issue 1-2-2: Asymmetric channel BW

Agreement: asymmetric channel bandwidth support:

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n109 | 5 | 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | 10 | 15, 20, 25, 30, 40, 50 |  |
|  | 15 | 20, 25, 30, 40, 50 |  |
|  | 20 | 25, 30, 40, 50 |  |
|  | 25 | 30, 40, 50 |  |
|  | 30 | 40, 50 |  |

# UE RF requirements	:

## Sub-topic 2-1: Second harmonic of band n28 overlaps with n75

Issue 2-1-1: REFSENS

Agreement: The new operating band n109 REFSENS requirements are specified as in the table below.

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n109 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) X | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) X |  |
| NOTE X: Applies for DL channels for which channels edges are > A MHz away from 2xFUL at 15 kHz SCS and > B MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |

Way forward:

Further check to be done regarding the values A and B in NOTE X on the above table whether we need more offset than 10 MHz for 15k SCS and 20 MHz for 30k SCS.

Issue 2-1-2: REFSENS UL configuration:

Agreement: UL configuration is always 5MHz 25RB for 15kHz SCS and 10MHz 24RB for 30kHz SCS:

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | Duplex Mode |
| n109 | 15 | 25 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | FDD |
|  | 30 |  | 24 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 |  |
| Note 7: For this DL channel bandwidth, the UL configuration of 5MHz for 15kHz SCS and 10MHz for 30kHz shall be used. |  |  |  |  |  |  |  |  |  |  |

# Adding band n109 to the TS/TR

Way forward:

Draft TP documents to add n109 band on the impacted TS/TR that are listed on the WID “Addition of FDD bands using the uplink from n28 and the downlink of n75 and n76”