3GPP TSG RAN WG4 Meeting #108  	                                                                   R4-2314931

Toulouse, France, August 21 – August 25, 2023

Title:	Reply LS to RAN1 on low-power wake-up receiver architectures

Release:	Rel-18

Work Item:	FS_NR_LPWUS

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Ruixin Wang

ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R4-2314726

1	Overall description

For evaluation purpose, RAN4 define a new term as guard RB of LP-WUS (granularity of RB), to distinguish from the traditional guardband for NR channel bandwidth, defined in Clause 5.3 of TS 38.101-1.

For ACS/ASCS impacts, RAN4 agreed to focus on the evaluation of required guard RB of LP-WUS rather than ACS/ASCS requirements with co-existence simulation. Based on RAN4 analysis until RAN4#108, RAN4 suggest the following guard RBs for ACS/ASCS:

- For ideal 5th order filter, the guard RB number for LP-WUS ACS is in the range of 1RB ~ 3RBs for 30KHz SCS, or 2RBs ~6RBs for 15KHz SCS.

  - It is noted that the analysis does not consider all impairments together, e.g., different combinations of phase noise, non-linearity, frequency error, implementation margin

- For 5th order filter, the guard RB number for LP-WUS ASCS is in the range of 0.5RB ~ 2RBs for 30KHz SCS, or 1RBs ~4RBs for 15KHz SCS.

- Guard RB for lower filter orders is under discussion

The detailed analysis outcome has been endorsed in TP to TR 38.869, as attached.

2	Actions

To RAN1:

ACTION: 	3GPP RAN4 respectfully asks RAN1 to take the above feedback into account.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #108bis                          	9-13 October, 2023    		Xiamen, China

TSG-RAN WG4 Meeting #109               						13th– 17th November 2023    Chicago, US