3GPP TSG-RAN WG4 Meeting #108	R4-2311010

Toulouse, France, 21 – 25 August 2023

3GPP TSG-RAN5 Meeting #99			R5-233669

Incheon, South Korea, 22nd May 2023 – 26th May 2023

Title:	LS on additional UE Gain parameters

Response to:	NA

Release:	Rel-15

Work Item:	TEI15_Test, 5GS_NR_LTE-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:	-

Contact Person:

#### Name:	Jakub Kolodziej

Tel. Number:

E-mail Address:	jakub.kolodziej@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

Discussion paper R5-233642 has been presented at RAN5#99. RAN5 investigated the alignment of the rough and fine beams as well as the difference in UE gain for different frequencies and the additional UE gain related parameters added to TS 38.133 in clauses B.2.1.5.2 and B.2.1.5.3 as well as in the measurement accuracy test cases in Table A.5.7.1.2.3-2 and Table A.7.7.1.2.3-2.

RAN5 came to the below conclusion and seeks further guidance from RAN4.

- The discussion in RAN4 that lead to additional UE gain parameters being introduced targeted FR2 inter-frequency measurement accuracy test cases A.5.7.1.2 and A.7.7.1.2.

- It is inconclusive if the parameters introduced in TS 38.133 in clauses B.2.1.5.2 and B.2.1.5.3, namely the UE gain difference between inter-frequencies Ginter  and additional gain reduction D, is applicable only to measurement accuracy test cases A.5.7.1.2 and A.7.7.1.2, or should it affect other test cases as well.

In RAN5 understanding the fact that the parameters are specified in the Annex B makes them applicable to all relevant test cases. For UE gain difference between inter-frequencies Ginter it is FR2 inter-frequency relative measurement test cases, and in case of additional gain reduction D it is all FR2 cases where UE is using rough beams in beam peak direction.

On the other hand, gain reduction D has been specified in Table A.7.7.1.2.3-2 (A.5.7.1.2.3-2) for relative accuracy requirements and is missing in Table A.7.7.1.2.3-1 (A.5.7.1.2.3-1) for the absolute accuracy, which may suggest that RAN4 intention was to introduce the UE Gain reduction D only for SS-RSRP relative accuracy test requirement for this particular case where UE is comparing the signal coming from a beam peak direction and signal coming from spherical coverage direction.

RAN5 has the following questions:

Q1: Is the D parameter applicable only to SS-RSRP relative accuracy test requirement in test cases A.5.7.1.2 and A.7.7.1.2 or it should be also applied to SS-RSRP absolute accuracy test requirement in A.5.7.1.2 and A.7.7.1.2, and other test cases as well?

Q2: Is the Ginter parameter applicable only to SS-RSRP relative accuracy test requirement in FR2 inter-frequency measurement accuracy test cases A.5.7.1.2 and A.7.7.1.2 or it should be also applied to other FR2 inter-frequency test cases as well?

2. Actions:

To RAN4 group.

ACTION:

RAN5 respectfully asks RAN4 to consider the observations shared in this LS as well as to provide feedback on the questions above.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#100	 21st – 25th August 2023	Toulouse, France

TSG-RAN5 Meeting#101	 13rd – 17th November 2023	Chicago, USA