3GPP TSG-RAN WG4 Meeting #108	R4-2311029

Toulouse, France, 21 – 25 August 2023

3GPP TSG-RAN WG1 Meeting #113	R1-2306211

Incheon, Korea, May 22nd – May 26th, 2023

# Title:	Reply LS on Rel-18 Multi-carrier enhancement for NR

# Response to:	R1-2304313/R4-2306623

# Release:	Rel-18

# Work Item:	NR_MC_enh-Core

Source:	TSG RAN WG1

To:	TSG RAN WG4

Cc:

Contact Person:

Name:	Jianchi Zhu

E-mail Address:	zhujc@chinatelecom.cn

1. Overall Description:

RAN1 thanks RAN4 for the LS on Rel-18 Multi-carrier enhancement for NR. RAN1 has discussed the following question.

In sub-clause 6.1.6 of TS 38.214, it is specified that: If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. On the definition and understanding of T0: - When gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, in RAN4 understanding, the T0 is the starting time for an uplink transmission on the switched-to carrier from network scheduling perspective and the starting time of actual uplink transmission from UE perspective. - When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. In this case, is T0 the starting time of uplink transmission from network scheduling perspective, or the starting time of actual uplink transmission from UE perspective?

RAN1 provides the following answer:

When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. From RAN1 perspective, T0 is the starting time of uplink transmission from network scheduling perspective.

2. Actions:

To RAN WG4:

ACTION:

RAN1 respectfully asks RAN4 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #114            21 - 25 August, 2023              Toulouse, France

TSG RAN WG1 Meeting #114bis        9 - 13 October, 2023              Xiamen, China