3GPP TSG-RAN WG4 Meeting #108	R4-2311036

Toulouse, France, 21 – 25 August 2023

3GPP TSG RAN WG1 #113 		                                    		       R1-2306248

Incheon, Korea, May 22nd – May 26th, 2023

Title:	LS on determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE states

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

Name:	Jingwen Zhang

E-mail Address:	zhangjingwen@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1. Overall Description:

In the Rel-18 WI Expanded and Improved NR Positioning, with regard to LPHAP (Low Power High Accuracy Positioning), RAN1 discussed the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity consists of multiple cells, and made the following agreement:

Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: - By default, UE maintains the TA from the last serving cell. - UE can adjust its UL timing according to the change in DL reference timing. - If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens. - Send LS to RAN4 asking the feasibility and necessary conditions (e.g., whether the above behaviour applies when the DL reference timing difference between the last camping cell and current camping cell exceeds a threshold and how UE adjusts it, or additional RRM procedure to obtain the timing difference needs to be defined.) - The DL reference timing follows the DL timing of current camping cell.

# 2. Actions:

To RAN4:

ACTION: RAN1 respectfully asks RAN4 on the feasibility and necessary conditions on the UE autonomous TA adjustment when cell-reselection happens, e.g., whether the autonomous TA adjustment applies when the DL reference timing difference between the last camping cell and current camping cell exceeds a threshold and how UE adjusts it, or additional RRM procedure to obtain the timing difference needs to be defined.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #114              21 Aug. – 25 Aug. 2023        Toulouse, France

TSG RAN WG1 Meeting #114b            9 Oct. – 13 Oct. 2023        Xiamen, China