3GPP TSG-RAN WG4 Meeting # 108bis  											R4-2315160

Xiamen, China, October 09 - October 13, 2023

Agenda Item:	5.13.5.1

Source: 	CMCC

Title: 			 Discussion on specification and UE feature for NR ATG

Document for:	Discussion

- Introduction

A reply LS for UE feature was sent by RAN1 [1], we copy the content for information:

- Overall Description: RAN1 would like to thank RAN4 for the LS(R4-2309797) on UE features used for ATG deployment scenario. Regarding whether the UE features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ can be used for Rel-18 NR ATG, RAN1 has the discussion and provides the following information. From RAN1’s perspective, the functionality corresponding to UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ can be used for Rel-18 NR ATG. New ATG UE features of ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ should be introduced in Rel-18. RAN1 asks RAN4 to take the corresponding information into consideration. Besides that, RAN1 has not discussed other UE features from Rel-17/18 NTN, e.g. Koffset (FG26-1), TA reporting (FG 26-4) and UE-specific koffset (26-9), which can be considered in the future discussions of new ATG UE features. # 2. Actions: To RAN4 ACTION: RAN1 respectfully asks RAN4 to take the above response into account in the future work.

In this reply LS, RAN1 mentioned that there are other UE features whose functionality should be also considered in the discussions of new ATG UE features. Therefore, we give an overall discussion about ATG UE features in this contribution.

In addition, we give our suggestion on ATG specification optimization, in order to diminish the spec size.

- ATG UE feature

In the reply LS, RAN1 mentioned that other UE features from R17/18 NTN should be taken into considered in the future discussion of new ATG features. Hence, we review all R17 NTN features and give our perspective about which feature should be introduced in ATG. The related NTN features are attached in the Annex.

L1 ATG UE feature 1: Uplink Time and Frequency pre-compensation and timing relationship enhancements

As we known, uplink time and frequency pre-compensation and timing relationship enhancements were agreed to introduced to ATG UE, therefore, we propose to introduce the related UE feature, and the detailed components should be updated.

Proposal 1: Introduce the ATG UE feature of uplink time and frequency pre-compensation and timing relationship enhancements as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink Time and Frequency pre-compensation and timing relationship enhancements | Support of UE specific TA calculation based on its GNSS-acquired position and the indicated BS location. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation) and closed (i.e., received TA commands) control loops Support of pre-compensation of the calculated TA in its uplink transmissions Support of frequency pre-compensation to counter shift the Doppler experienced on the service link Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated Support of UE receiving cell-specific K_offset in system information |  | TBD | BandNR | No | No | An ATG UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the indicated BS location Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling For UE supports NR communication via ATG, UE must indicate this FG is supported. |

L1 ATG UE feature 2/3: UE reporting of information related to TA pre-compensation/UE-specific Koffset

TA report was introduced to address GP issue in last meeting, therefore, the related UE feature should be introduced. Besides, since TAR can be supported in ATG, we think the UE-specific koffset functionality could also be supported by both network and UE, which could help to optimize the timing relationship.

Proposal 2: Introduce the ATG UE feature of UE reporting of TA information and UE-specific K_offset as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE reporting of TA information | Support UE reporting of TA information | TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements] | TBD | BandNR | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |
| UE-specific K_offset | Support of reception of UE-specific K_offset via MAC-CE Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements And UE reporting of information related to TA pre-compensation] | TBD | BandNR | No | No | Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |

L1 ATG UE feature 4/5: Increasing the number of HARQ processes/K1 range extension

As RAN1 reply LS stated, the functionality corresponding to UE features‘Increasing the number of HARQ processes’ and‘K1 range extension’ can be used for Rel-18 NR ATG. UE who support both features could support HARQ feedback when network configure some specific TDD patterns to address the GP issue in ATG deployment, e.g., 30D4S6U. Therefore, we propose to introduce the UE features.

Proposal 3: Introduce the ATG UE feature of Increasing the number of HARQ processes and K1 range extension as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | TBD | BandNR | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |
| K1 range extension | Support of extended K1 value range of (0..31) for unpaired spectrum |  | TBD | BandNR | No | No | Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |

L2/L3 ATG UE feature 1/2/3: Location based CHO/Event A4 based CHO/Location-based measurement report trigger

Location based CHO mechanism as R17 NR NTN has agreed for R18 ATG in last meeting. Therefore, the related UE feature should be introduced as well.

Proposal 4: Introduce the ATG UE feature of Location based CHO, Event A4 based CHO and Location-based measurement report trigger as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location based CHO | Indicates whether the UE supports location based CHO | condHandover-r16 is set for ATG bands. | TBD | BandNR | No | No | UE shall set the capability value consistently for all ATG operating bands. | Optional with capability signalling |
| Event A4 based CHO | Indicates whether the UE supports Event A4 based CHO | condHandover-r16 is set for ATG bands. | TBD | BandNR | No | No | UE shall set the capability value consistently for all ATG operating bands. | Optional with capability signalling |
| Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) | TBD [Location based CHO] | TBD | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |

L2/L3 ATG UE feature 4/5: SR triggered by a TA report and TA reporting during initial access

TA report was introduced to address GP issue in last meeting, therefore, the related UE feature should be introduced.

Proposal 5: Introduce the ATG UE feature of SR triggered by a TA report and TA reporting during initial access as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. |  | TBD | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | n/a | n/a | n/a | n/a |  | Conditional mandatory without capability signalling |

L2/L3 ATG UE feature 6: RRC_INACTIVE in ATG

The UE requirements in RRC_INACTIVE has been defined in RAN4, which means UE should report whether it support RRC_INACTIVE in ATG. We propose to introduce the related UE feature.

Proposal 6: Introduce the ATG UE feature of RRC_INACTIVE in ATG as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC_INACTIVE in ATG | Indicates whether the UE supports RRC_INACTIVE in ATG |  | TBD | TBD | No | No |  | Conditional mandatory with capability signalling |

RRM ATG UE features 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

In last meeting, we achieved the agreement that UE supporting set2 requirements for ATG is an optional feature without capability signaling. We propose to introduce the related UE feature.

Proposal 7: Introduce the ATG UE feature of Enhanced RRM requirements for measurements in IDLE and INACTIVE modes as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | If UE does not support the capability, other ATG measurement requirements (as specified in TS 38.133, clause 4.2X.2) are applied. |  | n/a | n/a | No | No |  | Optional without capability signaling |

- ATG spec optimization

After adding the ATG spec to TS 38.133, the size of spec is much expanded. Meanwhile, we observed that there are some ATG clauses which could directly refer to legacy clauses with additional note (if needed) to clarify that the legacy requirements shall apply only for FR1 and NR SA operation mode without considering CA/DC and inter RAT measurement. By taking such method, the spec size could be diminished a lot.

Proposal 8: For the ATG requirements, revise the current ATG clauses which could directly refer to legacy clauses with additional note (if needed) to clarify that the legacy requirements shall apply only for FR1 and NR SA operation mode without considering CA/DC and inter RAT measurement. Such kind of ATG Clauses including:

- 4.2X.2.5 	Maximum interruption in paging reception

- 6.1X.2.2.3	Preparation time

- 6.1X.2.2.4	Interruption time

- 6.2X.2.2	Requirements for 4-step RA type

- 6.2X.2.3	Requirements for 2-step RA type

- 8.1X.4	Minimum requirement at transitions

- 8.1X.5	Minimum requirement for UE turning off the transmitter

- 8.1X.6	Minimum requirement for L1 indication

- 8.5X.4	Minimum requirement for L1 indication

- 8.5X.9	Minimum requirement at transitions for beam failure detection

- 8.13X.2	UE-specific CBW change delay

- 8.14X.3	MAC-CE based pathloss reference signal switch delay

- 8.19X.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC

- 8.19X.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration

- 9.1X.2	Measurement gap

- 9.1X.2.1a	SA: Measurement Gap Sharing

- 9.1X.4 Capabilities for Support of Event Triggering and Reporting Criteria

- 9.1X.6	Minimum requirement at transitions

- 9.1X.7.2	Requirements applicability

- 9.1X.7.3	Requirements

- 9.1X.7.3.1	Requirements for autonomous activation/deactivation mechanism

- 9.1X.7.3.2	Requirements for network-controlled activation/deactivation mechanism

- 9.1X.7.3.3	Requirements for reception/transmission during activation/deactivation

- 9.2X.3.1	Requirements for FR1

- 9.2X.5.3	Scheduling availability of UE during intra-frequency measurements

- 9.2X.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

- 9.3X.3.1	Requirements for FR1

- 9.3X.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Introduce the ATG UE feature of uplink time and frequency pre-compensation and timing relationship enhancements as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink Time and Frequency pre-compensation and timing relationship enhancements | Support of UE specific TA calculation based on its GNSS-acquired position and the indicated BS location. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation) and closed (i.e., received TA commands) control loops Support of pre-compensation of the calculated TA in its uplink transmissions Support of frequency pre-compensation to counter shift the Doppler experienced on the service link Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated Support of UE receiving cell-specific K_offset in system information |  | TBD | BandNR | No | No | An ATG UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the indicated BS location Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling For UE supports NR communication via ATG, UE must indicate this FG is supported. |

Proposal 2: Introduce the ATG UE feature of UE reporting of TA information and UE-specific K_offset as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE reporting of TA information | Support UE reporting of TA information | TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements] | TBD | BandNR | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |
| UE-specific K_offset | Support of reception of UE-specific K_offset via MAC-CE Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | TBD [Uplink Time and Frequency pre-compensation and timing relationship enhancements And UE reporting of information related to TA pre-compensation] | TBD | BandNR | No | No | Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |

Proposal 3: Introduce the ATG UE feature of Increasing the number of HARQ processes and K1 range extension as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | TBD | BandNR | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |
| K1 range extension | Support of extended K1 value range of (0..31) for unpaired spectrum |  | TBD | BandNR | No | No | Note: This UE feature group is applicable only for ATG operating bands | Optional with capability signalling |

Proposal 4: Introduce the ATG UE feature of Location based CHO, Event A4 based CHO and Location-based measurement report trigger as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location based CHO | Indicates whether the UE supports location based CHO | condHandover-r16 is set for ATG bands. | TBD | BandNR | No | No | UE shall set the capability value consistently for all ATG operating bands. | Optional with capability signalling |
| Event A4 based CHO | Indicates whether the UE supports Event A4 based CHO | condHandover-r16 is set for ATG bands. | TBD | BandNR | No | No | UE shall set the capability value consistently for all ATG operating bands. | Optional with capability signalling |
| Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) | TBD [Location based CHO] | TBD | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |

Proposal 5: Introduce the ATG UE feature of SR triggered by a TA report and TA reporting during initial access as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. |  | TBD | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | n/a | n/a | n/a | n/a |  | Conditional mandatory without capability signalling |

Proposal 6: Introduce the ATG UE feature of RRC_INACTIVE in ATG as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC_INACTIVE in ATG | Indicates whether the UE supports RRC_INACTIVE in ATG |  | TBD | TBD | No | No |  | Conditional mandatory with capability signalling |

Proposal 7: Introduce the ATG UE feature of Enhanced RRM requirements for measurements in IDLE and INACTIVE modes as follows:

| Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | If UE does not support the capability, other ATG measurement requirements (as specified in TS 38.133, clause 4.2X.2) are applied. |  | n/a | n/a | No | No |  | Optional without capability signaling |

Proposal 8: For the ATG requirements, revise the current ATG clauses which could directly refer to legacy clauses with additional note (if needed) to clarify that the legacy requirements shall apply only for FR1 and NR SA operation mode without considering CA/DC and inter RAT measurement. Such kind of ATG Clauses including:

- 4.2X.2.5 	Maximum interruption in paging reception

- 6.1X.2.2.3	Preparation time

- 6.1X.2.2.4	Interruption time

- 6.2X.2.2	Requirements for 4-step RA type

- 6.2X.2.3	Requirements for 2-step RA type

- 8.1X.4	Minimum requirement at transitions

- 8.1X.5	Minimum requirement for UE turning off the transmitter

- 8.1X.6	Minimum requirement for L1 indication

- 8.5X.4	Minimum requirement for L1 indication

- 8.5X.9	Minimum requirement at transitions for beam failure detection

- 8.13X.2	UE-specific CBW change delay

- 8.14X.3	MAC-CE based pathloss reference signal switch delay

- 8.19X.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC

- 8.19X.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration

- 9.1X.2	Measurement gap

- 9.1X.2.1a	SA: Measurement Gap Sharing

- 9.1X.4 Capabilities for Support of Event Triggering and Reporting Criteria

- 9.1X.6	Minimum requirement at transitions

- 9.1X.7.2	Requirements applicability

- 9.1X.7.3	Requirements

- 9.1X.7.3.1	Requirements for autonomous activation/deactivation mechanism

- 9.1X.7.3.2	Requirements for network-controlled activation/deactivation mechanism

- 9.1X.7.3.3	Requirements for reception/transmission during activation/deactivation

- 9.2X.3.1	Requirements for FR1

- 9.2X.5.3	Scheduling availability of UE during intra-frequency measurements

- 9.2X.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

- 9.3X.3.1	Requirements for FR1

- 9.3X.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

- Reference

- R1-2308531, Reply LS on UE features for NR ATG, RAN1

- R4-2314307, WF on NR ATG RRM requirements, CMCC

- Annex

Related R17 NTN UE features:

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | 1. Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. 2. Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) 3. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 4. Support of pre-compensation of the calculated TA in its uplink transmissions 5. Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT 6. Support of frequency pre-compensation to counter shift the Doppler experienced on the service link 7. Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated 8. Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure 9. Support of UE receiving cell-specific K_offset/K_mac in system information |  | uplinkPreCompensation-r17 | BandNR | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | uplink-TA-Reporting-r17 | BandNR | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optionalwith capability signalling |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | max-HARQ-ProcessNumber-r17 | BandNR | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | 1. Support of reception of UE-specific K_offset via MAC-CE Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | ue-specific-K-Offset-r17 | BandNR | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum |  | k1-RangeExtension-r17 | BandNR | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_NTN_solutions-Core | 34-4 | Location based CHO | Indicates whether the UE supports location based CHO | 34-1, and condHandover-r16 is set for NTN bands. | locationBasedCondHandover-r17 | BandNR | No | No | UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
|  | 34-6 | Event A4 based CHO | Indicates whether the UE supports Event A4 based CHO | 34-1, and condHandover-r16 is set for NTN bands. | eventA4BasedCondHandover-r17 | BandNR | No | No | UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
|  | 34-7 | SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. | 34-1 | sr-TriggeredBy-TA-Report-r17 | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-13 | Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) | 34-4 | eventD1-MeasReportTrigger-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-14 | RRC_INACTIVE in NTN | Indicates whether the UE supports RRC_INACTIVE in NTN | 34-1 | inactiveStateNTN-r17 | NTN-Parameters-r17 | No | No |  | Conditional mandatory with capability signalling |
|  | 34-17 | TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | n/a | n/a | n/a | n/a |  | Conditional mandatory without capability signalling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-4 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | If UE does not support the capability, other NTN measurement requirements (as specified in TS 38.133, clause 4.2C.2) are applied for both LEO and GEO. |  | n/a | n/a | FDD only | FR1 only |  | Optional without capability signaling |